3GPP TSG-RAN WG2 Meeting #128	R2-2410211

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.7.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	DSR Enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#127bis:

We do not change the definition of delay-critical data For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. There is no need to include PSI in the enhanced DSR MAC CE. A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds.

# 2	Discussion

## 2.1	Remaining Time reporting thresholds

DSR is currently triggered when there is data with remaining time below the triggering threshold that has not been transmitted and has not been reported before.

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel.

Allowing at least one reporting threshold to be higher than the triggering threshold should not impact the trigger as the data that has been reported before (even though not as delay critical data when reported) would not need to trigger DSR again since the NW already gets the information and can deduce based on data reported/scheduled and their reported remaining time to know they would become delay critical data now without extra reporting. The early reporting gives NW more scheduling flexibility as well as reduce the overhead of further DSR and potential SR transmission.

For cancellation, DSR is cancelled when the DSR is transmitted, or when all the delay critical data are transmitted or discarded. Same could still apply even if some of the reporting thresholds are higher than the triggering threshold. When there is no data below the triggering threshold, the triggered DSR can be cancelled as currently defined.

The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

Observation 1: No impact on DSR trigger and cancellation with multiple reporting thresholds even if there is a reporting threshold higher than the triggering threshold.

Thus, in our view, the DSR triggering threshold and the reporting threshold(s) can be separately configured. The purpose of multiple reporting thresholds is to provide more granular information to assist the gNB in radio resource management. It does not exclude that a reporting threshold can have the same value of the triggering threshold. However, we see no strong need to limit the triggering threshold to be the highest or lowest reporting threshold, as it could be beneficial to report the data that will soon become delay critical when a DRS is triggered by a lower threshold. The reporting threshold(s) could even be configured relatively to the triggering threshold if needed. It is more flexible to allow triggering threshold and reporting threshold(s) to be separately configured.

Proposal 1: The DSR triggering threshold and the reporting threshold(s) are separately configured.

## 2.2	DSR Format

Flexible size DSR based on data status was agreed in the previous meeting.  Therefore, the Rel-19 DSR format would be able to report single entry BS and RT for the LCGs configured with multiple reporting thresholds as well. With extension bit, there is no extra overhead to always use the new DSR format even if there is only one entry for each LCG or only the LCGs configured with legacy single threshold are to be reported. It could also simplify the DSR selection to always use the new DSR format as long as there is any LCG configured with multiple reporting thresholds without extra overhead.

Proposal 2: for each reported BS/RT entry, an extension bit is used to indicate whether there is further BS/RT pair following for the LCG (the R bit in RT octet can be used).

Proposal 3: Rel-19 DSR format is used when there is any LCG configured with multiple reporting thresholds since there is no overhead difference from legacy DSR even if all the LCGs are with single entry.

Regarding non-delay critical data ahead of the queue, it is clear that with the current DSR, the NW is unaware of it. Therefore, since the amount of non-delay critical data has not been reported in the DSR (and the gNB is not able to extract it from the BSR), the gNB schedules resources based on what reported in the buffer size field of the DSR, and the allocated resources may not be sufficient for the transmission of delay-critical data. Actually, in case of DSR with multiple reporting thresholds, this does not only apply to delay-critical data, but to any data being reported in the DSR. To illustrate this problem, let’s consider an example where data belonging to different PDU sets (PDU set#0, #1, and #2 in Figure2.2-1) arrive in the UE buffer interlaced. We assume that PDU set based discard (pdu-SetDiscard-r18) is configured.

Figure 2.2-1: DSR with low-importance and high-importance data

In the example, the UE is configured with three reporting thresholds: RTT0, RTT1 and RTT2. The triggering threshold is assumed to be equal to RTT0. When the DSR is triggered, there is data of PDU set #1 (i.e. PDU #4), whose associated remaining time RT2 is in the range [RTT0, RTT1), which is ahead of data of PDU set#0, whose associated remaining time 
RT0 is in the range [0, RTT0). If the data ahead of PDU #4 with remaining time larger than the remaining time of PDU #4 (i.e. PDUs #2 and #3) is not included in the BS calculation of the range [RTT0, RTT1), then the DSR with multiple reporting thresholds would look like the one depicted in Table 2.2-1.

Table 2.2-1: Buffer size in multiple reporting thresholds

| Remaining Time range | Reported Remaining Time | Reported Buffer Size |
| --- | --- | --- |
| [0,RTT0) | RT0 | S0+S1+S4 |
| [RTT0,RTT1) | RT2 | S2+S3+S5+S6 |

Then, the gNB may think that allocating a transport block size of S0+S1+S4 would be enough for the UE to transmit data with associated remaining time RT0. However, this is not the case. Therefore, a better option for the DSR with multiple reporting thresholds would be the one shown in Table 2.2-2.

Table 2.2-2: Buffer size in multiple reporting thresholds

| Remaining Time range | Reported Remaining Time | Reported Buffer Size |
| --- | --- | --- |
| [0,RTT0) | RT0 | S0+S1+S2+S3+S4 |
| [RTT0,RTT1) | RT2 | S5+S6 |

Proposal 4: Include in the BS calculation for DSR corresponding to a remaining time range all PDUs until the last PDU belonging to any PDU set whose associated remaining time is within the remaining time range.

# 3	Conclusion

Details on DSR enhancements are discussed in this contribution with the following observation and proposals proposed:

Observation 1: No impact on DSR trigger and cancellation with multiple reporting thresholds even if there is a reporting threshold higher than the triggering threshold.

Proposal 1: The DSR triggering threshold and the reporting threshold(s) are separately configured.

Proposal 2: for each reported BS/RT entry, an extension bit is used to indicate whether there is further BS/RT pair following for the LCG (the R bit in RT octet can be used).

Proposal 3: Rel-19 DSR format is used when there is any LCG configured with multiple reporting thresholds since there is no overhead difference from legacy DSR even if all the LCGs are with single entry.

Proposal 4: Include in the BS calculation for DSR corresponding to a remaining time range all PDUs until the last PDU belonging to any PDU set whose associated remaining time is within the remaining time range.