3GPP TSG-RAN WG2 Meeting #128		         R2-2410230

Orlando, US, 18th – 22nd Nov 2024

Agenda Item:	8.6.4

Source: 	ITRI

Title:  	Discussion on early TA acquisition for conditional intra-CU LTM

Document for:	Discussion

Introduction

In last RAN2 meeting, it agreed the following agreements for conditional intra-CU LTM:

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

It was agreed that the PDCCH-ordered early TA acquisition is supported for conditional LTM. In this contribution, we present our preliminary thoughts on PDCCH-ordered early TA acquisition procedure for C-LTM.

Discussion

According to the agreement made in the previous meeting, the PDCCH-ordered early TA acquisition is supported for conditional LTM. In Rel-18 LTM, the PDCCH-ordered early TA acquisition procedure can be generally divided into the following steps:

Step 0: The source cell determines a potential target cell from the candidate cells.

Step 1: The source cell sends a PDCCH-ordered indicating the candidate cell, and initiating the early TA acquisition procedure.

Step 2: Upon receiving the PDCCH-ordered, the UE transmits a preamble to the indicated candidate cell. The UE does not need receive a random access response from the candidate cell in order to minimize data disruption at the source cell.

Step 3: After the source cell receives the TA value from the candidate cell, it includes this TA value in the cell switch command.

Step 4: After receiving the cell switch command with the TA value, the UE performs the cell switch procedure to the indicated candidate cell and applies the corresponding TA value.

Based on the above PDCCH-ordered early TA acquisition procedure applied to Rel-18 LTM, several issues may need to be addressed when applying this procedure in the conditional LTM.

Issue 1: Selection of Target Cell(s) for Early TA Acquisition Procedure

In the last meeting, some companies proposed that the UE could provide assistance information to help the source cell choose the possible target cell(s). Other companies suggested that the existing L1 or L3 measurements are sufficient for the source cell to select the target cell(s). From our perspective, the first method would require defining a new UE procedure (e.g., specifying when and how the assistance information should be provided). Reusing the current L1 or L3 measurement reports would have less impact on the specifications. Therefore, we propose that the selection of the target cell(s) for the PDCCH-ordered early TA acquisition procedure be left to network implementation, based on existing L1 or L3 measurement results.

Proposal 1: The selection of possible target cell(s) for the PDCCH-ordered early TA acquisition procedure should be left to network implementation, such as utilizing L1 or L3 measurement results.

Issue 2: Support for CBRA in the PDCCH-Ordered Early TA Acquisition Procedure

Currently, the PDCCH-ordered early TA acquisition procedure in Rel-18 uses CFRA. This approach minimizes data interruptions at the source cell, as the UE does not need to handle any contention issues. The time required for handling contention is unpredictable, which could lead to longer data interruptions. Therefore, we propose that CFRA be considered the baseline solution for the PDCCH-ordered early TA acquisition procedure.

Proposal 2: CFRA should be the baseline solution for the PDCCH-ordered early TA acquisition procedure. FFS for CBRA-based PDCCH-ordered early TA acquisition procedure.

Issue 3: TA Value acquisition from the Indicated Candidate Cell

In conditional LTM, whether the UE triggers a cell switch depends on the configured execution conditions. It is possible for the UE to perform the PDCCH-ordered early TA acquisition procedures several times before actually triggering the cell switch. As a result, data interruptions at the source cell may occur multiple times before the cell switch takes place. Therefore, each interruption caused by the PDCCH-ordered early TA acquisition should be minimized to reduce data disruption and prevent a decrease in UE throughput.

In Rel-18 PDCCH-ordered early TA acquisition, to minimize data interruption at the source cell, the UE does not receive a random access response after transmitting a preamble. We believe this principle should also apply to conditional LTM. Since receiving the TA value from the indicated candidate cell could introduce unpredictable delays, we propose that the UE instead acquire the TA value from the source cell.

Proposal 3: The UE should acquire the TA value from the source cell for the PDCCH-ordered early TA acquisition procedure as a baseline solution.

Issue 4: Validity of Acquired TA for Early TA Acquisition

In conditional LTM, the decision to trigger a cell switch may depend on whether the relevant execution conditions are met. Therefore, it is possible that the UE acquires the TA value through the early TA acquisition procedure before triggering the cell switch. In such cases, the UE must maintain the validity of the acquired TA value.

In legacy systems, the validity of the TA value is typically managed using a timer (e.g., TAT) and/or a signaling difference threshold. A similar approach is reused the legacy methods to maintain the validity of the acquired TA in Conditional LTM.

Proposal 4: The UE should maintain the validity of the acquired TA value prior to executing the Conditional LTM procedure.

Option 1: The UE maintains a timer similar to TAT to track the validity of the TA value.

Option 2: The UE considers the TA value valid if the L1 measurement result does not change beyond a specified threshold.

Conclusion

Proposal 1: The selection of possible target cell(s) for the PDCCH-ordered early TA acquisition procedure should be left to network implementation, such as utilizing L1 or L3 measurement results.

Proposal 2: CFRA should be the baseline solution for the PDCCH-ordered early TA acquisition procedure. FFS for CBRA-based PDCCH-ordered early TA acquisition procedure.

Proposal 3: The UE should acquire the TA value from the source cell for the PDCCH-ordered early TA acquisition procedure as a baseline solution.

Proposal 4: The UE should maintain the validity of the acquired TA value prior to executing the Conditional LTM procedure.

Option 1: The UE maintains a timer similar to TAT to track the validity of the TA value.

Option 2: The UE considers the TA value valid if the L1 measurement result does not change beyond a specified threshold.