3GPP TSG-RAN WG2 #128	R2-2410242

Orlando, USA, Nov 18th-22nd, 2024

Agenda item:		8.6.2	Inter-CU LTM

Title:		Discussion on inter-CU LTM

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

During the last meeting, RAN2 discussed several aspects to support inter-CU LTM in non-DC and DC scenario, including configuration and signalling procedures at different LTM stages, some agreements for inter-CU LTM have been reached:

Agreements on inter-CU LTM

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

- The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

- Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

- The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

- The MN can request an SCG reference configuration from any of the involved SNs.

5.	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g., a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7.	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

9.	Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

10.	Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

11.	RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

In this contribution, we further discuss the remaining open issues or details for inter-CU LTM and provide our consideration.

# 2. Discussion

2.1 inter-CU MCG LTM

Inter-CU MCG LTM without DC

During the RAN2#127 meeting, RAN2 agreed that the inter-CU LTM re-uses the reference configuration from Rel-18 LTM, no additional reference configurations are supported. Regarding the issue of which node to decide a reference configuration, we think that the source gNB can decide it. If the source gNB decides to prepare the reference configuration by itself, then the source CU can request the source DU to provide it. If the source gNB decides that the candidate gNB should provide the reference configuration, the source CU may include the reference configuration request indication in handover request message, then the candidate CU can obtain the reference configuration from the candidate DU and provide it to source CU in handover request acknowledge message.

Proposal 1: For non-DC inter-CU LTM, either the source gNB or the candidate gNB can prepare the reference configuration, and the source gNB can decide which node should prepare it.

In Rel-18 intra-CU LTM, both CBRA-based and CFRA-based procedures can be supported. And for CFRA-based intra-CU LTM, the CFRA resource may be provided in RRC configuration, or CFRA resource related information may be included in LTM MAC CE, which similar to those in the legacy PDCCH order triggered RACH. We think that the intra-CU LTM mechanism can be reused for inter-CU LTM.

Proposal 2: CFRA-based LTM can be supported for inter-CU LTM.

For security solution for inter-CU MCG LTM without DC, the reply LS from SA3 indicates that RRC signalling based solution is preferred by them, and if RAN2 is aiming to avoid RRC sigalling between cell switches, SA3 will work on further solutions like MAC CE based solution. We think that we’d better not go with the RRC based solution easily. Because additional RRC signalling increases singalling overhead and introduces additional delay, and should be avoided if there is other alternative solution.

Proposal 3: RAN2 sends reply LS to SA3 to further work on solutions other than RRC signalling based security solutions for inter-CU LTM non-DC DC case.

Inter-CU MCG LTM with DC

RAN2 agreed that the SCG configuration can be changed (e.g., release or reconfiguration) when the inter-CU MCG LTM is executed, so there is the case that the network configures the candidates for inter-CU MCG LTM with SCG configuration and MCG LTM with SCG release simultaneously. Therefore, it is natural to support the inter-CU MCG LTM with SCG addition to ensure that UE can perform subsequent LTM cell switch between those candidates.

Proposal 4: RAN2 confirm that the inter-CU MCG LTM cell switch with SCG addition is supported.

To support inter-CU MCG LTM with SCG, how to derive/update SCG should be specified. Since the master key is updated during the LTM cell switch procedure, the secondary key should also be derived/updated regardless the SCG is kept, changed or added. As UE needs a sk-counter value to derive secondary key, how UE gets this sk-counter value needs to be specified. And how and when the target SN gets the secondary key also needs to be specified by RAN3.Since mechanism of secondary key derivation/update should be up to SA3 decision, RAN2 can send LS to SA3 to ask about it for both initial LTM cell switch and subsequent LTM cell switch.

Proposal 5: If inter-CU MCG LTM with SCG addition is supported, RAN2 sends LS to SA3 to ask how the UE gets the sk-counter value for secondary key derivation and how the target SN gets the secondary key, for both initial LTM cell switch and subsequent LTM cell switch.

2.2 inter-CU SCG LTM

For inter-CU SCG LTM, RAN2 agreed that the reference configuration for inter-CU SCG LTM at least include SCG part and whether it includes the MCG part is still FFS. Due to RAN2 support coexistence of inter-SN LTM and intra-MN LTM, there is the case that the UE performs the intra-MN LTM first and then triggers the inter-SN LTM. In this case, the MCG configuration is changed due to first intra-MN LTM. And then for inter-SN LTM procedure, if the MCG part is not included in the reference configuration, the UE generates the MCG part configuration based on the changed MCG configuration (i.e., MCG configuration after the first intra-MN LTM) instead of the original MCG configuration (i.e., MCG configuration before the first intra-MN LTM procedure), which may misalignment with network. Alternatively, the UE can store the MCG configuration before the first LTM execution and generate the MCG configuration based on the stored configuration, which is complicated. In order to avoid the misalignment configuration or complicated mechanism, we think the simplest way is included the MCG part in reference configuration.

Proposal 6: For inter-CU SCG LTM, the reference configuration should include the MCG part.

In CPAC, upon execution of CPAC procedure, the UE transmits a MN RRCReconfigurationComplete including SN RRCReconfigurationComplete message and the identity of the applied conditional reconfiguration. The motivation behind is to assist the MN to identify the target SN of the CPAC procedure. During last meeting, it was agreed that upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. Similar as CPAC, to assisted the MN to identify the target SN, the UE should include the identity of the applied LTM candidate configuration in the MN RRCReconfigurationComplete message.

Additionally, in Rel-18 intra-CU LTM, it was agreed to include the identity of the applied LTM candidate configuration in the RRCReconfigurationComplete message, with the motivation behind is to indicate to the gNB-CU that the RRCReconfigurationComplete is for the LTM cell switch procedure, in case that LTM and CHO are configured for the same candidate cell, or multiple LTM configuration are associated with the same candidate cell. The corresponding chairman’s note can be seen in the below table. For SN/SCG, it is also possible that LTM and CPAC are configured for the same candidate PSCell, or multiple LTM configuration are associated with the same candidate PSCell. Therefore, for inter-CU SCG LTM, to assist the target SN to identify the applied LTM candidate configuration, the UE should also include the identity of the applied LTM candidate configuration in the SN RRCReconfigurationComplete message.

R2-2403177 [E068][E231][E074][E240][S792]Resolution of remaining RILs for LTM Ericsson discussion Rel-18 NR_Mob_enh2-Core Late E74: “Transaction ID” - Apple agrees with Ericsson proposal, and think this is helpful for R19 as well. - CATT think it is not a valid case that multiple configurations are applicable for one target cell. - MTK think now we have some cases where this may apply and think this is helpful for R19 - HW think the only scenario is LTM and CHO config for the same cell, think only 1 bit is needed. - vivo prefer to not do any optimization, can configure different C-RNTI for different config. - LGE think that with extended transaction ID space this is resolved. - Xiaomi think that complete message already contains the conditional configuration ID. - Ericsson think there can be multiple LTM configs per target cell, and think that CRNTI check does not work. - Intel support this proposal, it seems to be a clean solution. Lenovo as well. - Nokia wonders if this is really a real problem .. Upon an LTM cell switch, the UE includes the applied LTM candidate configuration identifier within the RRCReconfigurationComplete message.

Proposal 7a. For inter-CU SCG LTM, to assist the MN to identify the target SN, UE includes the identity of applied LTM candidate configuration in the MN RRCReconfigurationComplete message.

Proposal 7b. For inter-CU SCG LTM, to assist the SN to identify the applied LTM candidate configuration, UE includes the identity of applied LTM candidate configuration in the embedded SN RRCReconfigurationComplete message.

# 3. Conclusion

In this contribution we discussed some issues related to inter-CU LTM and made the following proposals.

inter-CU MCG LTM

Proposal 1: For non-DC inter-CU LTM, either the source gNB or the candidate gNB can prepare the reference configuration, and the source gNB can decide which node should prepare it.

Proposal 2: CFRA-based LTM can be supported for inter-CU LTM.

Proposal 3: RAN2 sends reply LS to SA3 to further work on solutions other than RRC signalling based security solutions for inter-CU LTM non-DC DC case.

Proposal 4: RAN2 confirm that the inter-CU MCG LTM cell switch with SCG addition is supported.

Proposal 5: If inter-CU MCG LTM with SCG addition is supported, RAN2 sends LS to SA3 to ask how the UE gets the sk-counter value for secondary key derivation and how the target SN gets the secondary key, for both initial LTM cell switch and subsequent LTM cell switch.

inter-CU SCG LTM

Proposal 6: For inter-CU SCG LTM, the reference configuration should include the MCG part.

Proposal 7a. For inter-CU SCG LTM, to assist the MN to identify the target SN, UE includes the identity of applied LTM candidate configuration in the MN RRCReconfigurationComplete message.

Proposal 7b. For inter-CU SCG LTM, to assist the SN to identify the applied LTM candidate configuration, UE includes the identity of applied LTM candidate configuration in the embedded SN RRCReconfigurationComplete message.

# 4. References

[1] R2-2409534, Reply LS on security handling for inter-CU LTM in non-DC cases