3GPP TSG-RAN WG2 Meeting #128	R2-2410268

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:		8.8.4

Source:		Lenovo

Title:		NTN broadcast service area indication and service continuity

Document for:		Discussion

- Introduction

The latest RAN2#127 meeting made the following agreements regarding broadcast services in NTN:

- For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

In this contribution we discuss some further issues of introducing broadcast service area indication and service continuity in NTN.

- Discussion

For an NTN cell that could be much larger than an MBS broadcast service area, a UE may not be automatically authorized to receive an MBS broadcast service by the serving NTN cell it is in, i.e., not all UE within the same NTN cell is authorized to receive the same MBS broadcast service. For this reason, RAN2 have agreed to introduce indication of geographical broadcast service area in NTN, and further impacts and issues are expected to be resolved as well, including how to indicate the area information, MCCH monitoring enhancements based on the area information and broadcast service continuity.

## Broadcast service area indication and MCCH monitoring enhancements

In the latest RAN2#1267bis meeting there was a discussion on where and how to include the broadcast service area information. It was observed that due to the size limitation of SI message (2976 bits), it seems hard to include the corresponding area information in an existing SIB (i.e., SIB20). A compromise was made to at least include service area ID in MCCH and further discussion for the area information indication is expected. In our understanding if using existing SIB20 is precluded due to the SIB size limitation, introducing a new SIB may have the same issue as there is no clear justification on how many areas shall be indicated in an NTN cell. For reference, the size of warningAreaCoordinates used for CMAS as specified in ATIS-0700041 can be up to 1038 bits.

Observation 1: Introducing a new SIB to include the broadcast area information needs to consider the number of areas to be indicated.

Thanks to the segmentation supported for the MCCH logical channel, the size of MBSBroadcastConfiguration message is not restricted and thus can include the intended area information, and the MBS session ID in MBSBroadcastConfiguration can be easily associated to the area information.

Observation 2: MCCH has the advantages in size and MBS session ID association to include the broadcast area information.

Therefore, it seems more reasonable to have the MCCH option.

Proposal 1: The intended service area(s) is included in MCCH in association with MBS session ID.

For the initial MCCH monitoring, in legacy TN a UE starts to monitor upon entering the cell providing SIB20, and it is up to UE implementation to use the cell/tracking area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service. In an NTN cell which could be much larger than an MBS broadcast service area, such implementation relaying on the cell-level area information in USD may need to be further specified, with the geographical area information introduced. The previous RAN2#127 meeting agreed that UE may not need to (re)acquire up-to-date MCCH when the UE is not in any intended service area of its interested broadcast services. If MCCH is adopted to include the area information, it means that the latest area information can only by obtained by MCCH monitoring, which causes a dilemma between power saving and service reception. In our understanding the key is whether and how often the broadcast service area could change.

Observation 3: Whether and how often the broadcast service area could change in an NTN cell will impact on how UE can skip MCCH re-acquisition.

From specification perspective, we think there could at least an additional description that UE in an NTN cell can also use the indicated broadcast area information when determining this. And how to use (e.g., whether to be specified) can be further discussed.

Proposal 2: UE can use the indicated broadcast area information to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service. FFS whether this is up to UE implantation or needs to be further specified.

Meanwhile, MCCH uses a modification period and MCCH contents are only allowed to be modified at each modification period boundary. In NTN UE needs to ensure a valid UE location information and a valid ephemeris data (in SIB19) for uplink synchronization. UE re-acquiring SIB19 in NTN may possibly miss the modification period boundary. In such case, enhancements may also be considered to accelerate UE in awareness of MCCH change.

Observation 4: UE may miss a MCCH modification period boundary when reacquiring SIB19 for UL synchronization.

Proposal 3: RAN2 to discuss whether UE can initiate MCCH acquiring after re-acquiring SIB19, e.g., when the duration of reacquiring SIB19 overlaps with an MCCH modification period boundary.

## Service continuity regarding broadcast service area

For MBS broadcast service continuity in TN, mobility procedures for MBS reception allow the UE to start or continue receiving MBS service when changing cells. The gNB may indicate in SIB21 the mapping between the current and/or neighboring carrier frequencies and MBS Frequency Selection Area Identities (FSAI). The FSAI is derived from the serviceArea and radiofrequency elements received in the USD (User Service Description) via NAS signalling from network, wherein the serviceArea element declares one or more service areas designated by the SAI (Service Area Identity) which is mapped onto one or more cells.

If UE is receiving or interested to receive an MBS broadcast service and can only receive this MBS broadcast service by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session. The UE may also consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session.

This legacy mechanism works well under the assumption that an MBS broadcast service area is mapped to one or more TN cells. The UE can simply determine the frequency priority based on the receiving or interested MBS broadcast service and the frequency it associates to, and reselects to a cell that supports that service within its whole coverage. When considering that an NTN cell coverage could be larger than an MBS broadcast service area, which cannot be mapped to the cell’s whole coverage, UE cannot simply determine the frequency priority only based on the association between service and frequency, and service area information of neighbour cell may be needed.

Observation 5: In NTN wherein broadcast service area could cover a portion of an NTN cell, UE cannot simply determine the frequency priority only based on the association between service and frequency.

Figure 1 shows such differences between TN and NTN. For TN in a), UE1 from Cell #1 to Cell #2 can continue to receive MBS broadcast service #A as it is supported in Cell #2, and UE2 from Cell #1 to Cell #3 cannot continue to receive MBS broadcast service #A as it is not supported in Cell #3. For NTN in b), MBS broadcast service #A is supported in partial coverage of Cell #2 and Cell #3, and UE1 from Cell #1 to Cell #2 is expected to continue to receive MBS broadcast service #A as UE is still in its service area, while UE2 from Cell #1 to Cell #3 is expected not to continue to receive MBS broadcast service #A as UE is moving out its service area. If the legacy mechanism for TN applies, UE2 will also prioritize a frequency associated to the broadcast service but it will not be authorized to receive as it moves out of service area.

Figure 1. Example of relation between cell coverage and MBS broadcast service area in TN and NTN

Therefore, additional MBS broadcast service area information for neighbour cell may also be needed at UE for service continuity and how to indicate such information needs to be discussed as well.

Proposal 4: RAN2 to discuss how to indicate the broadcast service area information of a neighbour NTN cell to UE for service continuity e.g., frequency priority handling.

- Conclusion

In this contribution we discuss the further issues of broadcast service area indication and service continuity in NTN. The following observations are given:

Observation 1: Introducing a new SIB to include the broadcast area information needs to consider the number of areas to be indicated.

Observation 2: MCCH has the advantages in size and MBS session ID association to include the broadcast area information.

Observation 3: Whether and how often the broadcast service area could change in an NTN cell will impact on how UE can skip MCCH re-acquisition.

Observation 4: UE may miss a MCCH modification period boundary when reacquiring SIB19 for UL synchronization.

Observation 5: In NTN wherein broadcast service area could cover a portion of an NTN cell, UE cannot simply determine the frequency priority only based on the association between service and frequency.

And it is proposed that:

Proposal 1: The intended service area(s) is included in MCCH in association with MBS session ID.

Proposal 2: UE can use the indicated broadcast area information to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service. FFS whether this is up to UE implantation or needs to be further specified.

Proposal 3: RAN2 to discuss whether UE can initiate MCCH acquiring after re-acquiring SIB19, e.g., when the duration of reacquiring SIB19 overlaps with an MCCH modification period boundary.

Proposal 4: RAN2 to discuss how to indicate the broadcast service area information of a neighbour NTN cell to UE for service continuity e.g., frequency priority handling.