3GPP TSG-RAN WG2 Meeting #128	R2-2410270

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:		8.9.2

Source:		Lenovo

Title:		Further considerations on S&F operation

Document for:		Discussion

- Introduction

The latest RAN2#127bis meeting made the following agreements regarding Store and Forward (S&F) operations in IoT NTN:

- The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed) - RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided. - An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed - If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure - When present, the “S&F operation” indication has two possible settings: ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell) ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

In this contribution we discuss issues and solutions in cell barring, cell reselection and operation information exchange for S&F.

- Discussion

## Cell barring for Rel-19 NTN UEs

Based on current RAN2 discussion, cellBarred-NTN introduced in Rel-17 IoT NTN can be reused to restrict Rel-17 & Rel-18 NTN UEs from accessing an NTN cell operating in S&F. For Rel-19 NTN UEs, an indication that “the cell is operating in S&F mode” was agreed to be introduced and the following descriptions were decided:

- If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

- When present, the “S&F operation” indication has two possible settings:

- ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

- ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

Following the above description, the cell barring logic for S&F can be decided.

Proposal 1: If the “S&F operation” indication is present, an NTN-capable UE supporting S&F in RRC_CONNECTED ignores cellBarredNTN, and considers cell as not barred.

Proposal 2: If the “S&F operation” indication is present, an NTN-capable UE supporting S&F in RRC_IDLE ignores cellBarredNTN, and determines cell barring status based on its value (1 as not barred and 0 as barred).

Based on the above discussions for NTN cell barring, Rel-19 NTN UEs also face the similar issues during the cell reselection procedures. More specifically, if a neighbour NTN cell is operating in S&F, the corresponding cell barring scheme shall be considered by the UE when performing cell reselection.

Proposal 3: If a neighboring NTN cell operating in S&F is considered as not barred, an NTN-capable UE supporting S&F considers prioritizes the frequency of the cell for cell reselection.

Proposal 4: If a neighboring NTN cell operating in S&F is considered as barred, an NTN-capable UE supporting S&F considers the cell as barred for cell reselection.

Proposal 5: If a neighboring NTN cell is operating in S&F, an NTN-capable UE NOT supporting S&F considers the cell as barred.

## Uu information exchange for S&F operation

Referring to SA2 works as in TR22.865, at least the following scenarios or use cases can be considered for S&F operations: MO, MT, data transfer for IoT devices in remote areas, and emergency report. The main different between S&F operation and normal operation in NTN is whether there is a continuous end-to-end connectivity path between the UE, the satellite and the ground network i.e., whether it requires the service and feeder links to be active simultaneously. Such differences have brought in some new requirements as consolidated by SA2:

- support S&F operation for authorized UEs;

- support to inform a UE whether S&F Satellite operation is applied;

- support to allow the operator or a trusted 3rd party to apply an S&F data retention period;

- support to allow the operator or a trusted 3rd party to apply an S&F data storage quota;

- support a mechanism to configure and provision specific required QoS and policies for UE’s data;

- support to provide related information (e.g. estimated delivery time to the authorised 3rd party) to an authorized UE.

- support to inform an authorised 3rd party whether S&F Satellite operation is applied for communication with a UE and to provide related information (e.g. estimated delivery time to the authorised UE).

- support forwarding of the stored data from one satellite to another satellite (e.g., which has an available feeder link to the ground network), through ISLs.

- support suitable means to resume communication between the satellite and the ground station once the feeder link becomes available.

- support mechanisms for a UE to register with the network when the network is in S&F Satellite operation.

- support mechanisms to authorize subscribers for receiving services when the network is in S&F Satellite operation.

Observation 1: At least from RAN2 perspective, solutions may also be developed to fulfil some requirements of S&F operation as consolidated by SA2.

According to the listed cases and requirements, the services that can leverage S&F operations have some specific characteristics including delay-tolerant (could be hours) and remote (source or target could be reached only be limited satellite coverage), and the UE may need some availability information about the S&F operation supported by the network, e.g., for which area or for how long the data can be delivered, or some functional information about the S&F operation supported by the network, e.g., support of providing delivery status, support of cast types for delivery (unicast, multicast and broadcast), support of eNB/gNB-only S&F (no CN entity is involved during the S&F operation).

Observation 2: For S&F operations, it may also be necessary to indicate UE some availability information about the S&F operation.

On the other hand, network may also need to be aware of UE requirements for S&F operation, e.g., to configure UE for S&F data transmission/reception or to understand for which area or for how long the data shall be delivered, or whether providing delivery status is needed, or which cast types for delivery is expected.

Observation 3: For S&F operations, the network may need to know UE’s service requirements for S&F operation.

When both the network and UE supports S&F operation, the UE needs to determine when an S&F operation service can be initiated e.g., whether a service can leverage S&F operation. From protocol design perspective, the AS-NAS functional spilt in UE for S&F shall be specified, e.g., which stratum is responsible for the initiation and the other stratum can provide assistances. Moreover, AS-NAS interaction could be needed to ensure appropriate S&F initiation and data delivery, e.g., start/suspend NAS->AS S&F data delivery when satellite coverage is valid/invalid to avoid congestion.

Observation 4: Metrics and interactions to initiate an S&F operation at UE may need to be specified.

Based on the above analysis, the following aspects can also be considered for RAN2’s study on supporting S&F operations:

Proposal 6: To support S&F operation and fulfill requirements from SA2’s work, RAN2 to study the following:

- how to indicate UE the availability information about the S&F operation (e.g., estimated delivery time);

- how to indicate network the UE’s service requirements for S&F operation;

- AS-NAS functional split and interactions to support S&F operation (may pending on SA2 future work).

- Conclusion

In this contribution we discuss issues and potential solutions in cell barring, cell reselection and operation information exchange for S&F operations in IoT NTN. The following observations are given:

Observation 1: At least from RAN2 perspective, solutions may also be developed to fulfil some requirements of S&F operation as consolidated by SA2.

Observation 2: For S&F operations, it may also be necessary to indicate UE some availability information about the S&F operation.

Observation 3: For S&F operations, the network may need to know UE’s service requirements for S&F operation.

Observation 4: Metrics and interactions to initiate an S&F operation at UE may need to be specified.

And we proposed:

Proposal 1: If the “S&F operation” indication is present, an NTN-capable UE supporting S&F in RRC_CONNECTED ignores cellBarredNTN, and considers cell as not barred.

Proposal 2: If the “S&F operation” indication is present, an NTN-capable UE supporting S&F in RRC_IDLE ignores cellBarredNTN, and determines cell barring status based on its value (1 as not barred and 0 as barred).

Proposal 3: If a neighboring NTN cell operating in S&F is considered as not barred, an NTN-capable UE supporting S&F considers prioritizes the frequency of the cell for cell reselection.

Proposal 4: If a neighboring NTN cell operating in S&F is considered as barred, an NTN-capable UE supporting S&F considers the cell as barred for cell reselection.

Proposal 5: If a neighboring NTN cell is operating in S&F, an NTN-capable UE NOT supporting S&F considers the cell as barred.

Proposal 6: To support S&F operation and fulfill requirements from SA2’s work, RAN2 to study the following:

- how to indicate UE the availability information about the S&F operation (e.g., estimated delivery time);

- how to indicate network the UE’s service requirements for S&F operation;

- AS-NAS functional split and interactions to support S&F operation (may pending on SA2 future work).