3GPP TSG-RAN WG2 Meeting #128	R2-2410273

Orlando, USA, November 18th – 22nd, 2024

Agenda item:		8.4.3

Source:		Lenovo

Title:	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Document for:		Discussion and Decision

- Introduction

In RAN#103 meeting, WID of Rel-19 Low-power wake-up signal and receiver for NR (LP-WUS/ WUR) [1] was discussed and agreed, the objectives related to RRM measurement for RRC IDLE/INACTIVE modes of the work items are given as following.

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In the previous RAN2 meetings [2]-[4], RRM measurement relaxation and offloading in RRC_INACTIVE/IDLE state has been discussed and following agreements have been reached:

RAN2#126 Agreements: - For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. - RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’. RAN2#127 Agreements： - RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results. RAN2#127bis Agreement: Working assumption: - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

This contribution aims to present our views on the RRM measurement relaxation and offloading from MR to LR for UEs in RRC_IDLE/INACTIVE mode.

- Discussion

According to the latest WID, both RAN4 and RAN2 are required to specify RRM measurement relaxation of UE MR for both serving cell and neighboring cell measurements, and UE serving cell RRM measurement offloading from MR to LR, including the necessary conditions. Currently, the potential cases to be considered for RRM relaxation and serving cell measurement offloading is under discussion in RAN4 [5]. It has been approved by RAN4 that the ‘fully offloading case’ and ‘relaxed case b’ are considered as feasible while other cases are pending for further study, the details can be seen as below:

Approved cases on RRM measurement relaxation/offloading.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | OFF | OFF: FFS the conditions and details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

FFS cases on RRM measurement relaxation/offloading.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | OFF | ON |
| #4 Relaxed case c | OFF | ON, FFS the condition and the details | ON |
| #5 Extra Relaxed case | On with relaxation measurement | Off | OFF |
| #6 Extra Relaxed case | On with relaxation measurement | On with relaxation measurement | OFF |

RAN4 is required to define the RRM measurement relaxation/offloading requirements while RAN2 focuses on specifying the relaxation/offloading criterion for UEs supporting LP-WUS. Based on RAN4’ conclusion, it is suggested RAN2 gives priority to discuss the entry/exit conditions for the following offloading/relaxation cases:

- Fully offloading case：serving cell measurement is fully offloaded from MR to LR, while MR performs no measurement on serving cell, FFS on neighboring cell measurement,

- Relaxed case b (Partial offloading case): serving cell measurement and neighboring cell measurement are relaxed on MR, while LR also performs serving cell measurement, this case is also called as partial offloading case.

Observation 1: RAN4 has approved fully offloading case and relaxed case b as below while other cases on RRM measurement relaxation and offloading are pending for further study:

- Fully offloading case: serving cell measurement is fully offloaded from MR to LR while MR performs no measurement on serving cell.

- Relaxed case b: serving cell measurement is relaxed on MR while LR also performs the serving cell measurement.

2.1 Fully offloading case

According to latest RAN2 agreements [3], the entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. Currently, the agreed entry condition aims to guarantee the UE with a good coverage performance for both MR and LR. However, the serving measurement offloading aims to achieve UE power saving gain but also needs to limit the impacts on mobility performance as much as possible. For this, the UE needs to at least stay in a relatively stable status and have a low possibility of performing cell (re)selection, then the serving measurement offloading from MR to LR could be considered. To achieve this, in addition to a great MR threshold, additional entry conditions for the fully serving cell measurement offloading can be considered. For example, the low mobility condition, which can be defined as the difference of RSRP threshold being lower than a (pre-)configured threshold within a time duration, and MR measurements are used for the entry condition evaluation for low mobility criterion.

Proposal 1: RAN2 to discuss additional entry condition for fully serving cell measurement offloading case, e.g., low mobility conditions evaluated by the MR.

Additionally, due to the minimalist design of the low power receiver, RRM measurement results performed by the LR may have loss on accuracy to some extent compared to the MR measurement results. For the fully offloading case, the exit conditions are defined to make UE to stop the fully offloading and fallback to legacy RRM measurements on MR to guarantee the mobility performances. In the last RAN2 meeting, it has been agreed that the exit condition is based on the LR measurement results. If the low mobility condition is supported as one of the entry conditions for the offloading, then the exit conditions should also consider the high mobility criterion evaluated by the LR.

Proposal 2: The exit conditions for serving cell measurement offloading considers high mobility condition evaluated by the LR.

2.2 Relaxed Case b

As defined by RAN4, for the relaxed case b, MR performs relaxation measurement on serving cell while LR is ON and performs the serving cell measurement, which can also be defined as the partial offloading case. Regarding the entry conditions for the relaxed case, generally, we understand that good coverage performance and low mobility condition can also be considered for the UE determining to perform the serving cell measurement relaxation, i.e., the good coverage refers to MR and/or LR measurement is greater than a certain RSRP threshold, while the low mobility condition refers to the MR measurement difference on RSRP threshold is lower than a (pre-)configured threshold within a time duration. Whether to consider LR measurement can be further discussed. However, compared to fully offloading serving cell measurement to the LR, since both MR and LR can perform serving cell measurement for the relaxed case, the entry conditions can be defined to be less stringent than the fully offloading case, for example, the UE needs to at least stay in the coverage of LP-WUS, then UE may partially offload the measurement from MR to LR. But only in the case of the coverage performances are good enough, i.e., more stringent requirement than LR to perform serving cell measurement partially, MR may fully offload the serving cell measurement to LR. Therefore, we proposed that for the relaxed case b, the entry conditions on serving cell measurement relaxation can reuse the entry conditions of fully offloading case, but with different thresholds configuration.

Proposal 3: For the relaxed case b, the entry conditions on serving cell measurement relaxation can reuse the entry conditions of fully offloading case, with different thresholds configuration.

Regarding the exit conditions for the relaxed case b, i.e., MR stops performing the relaxation measurement on serving cell and can fallback to the legacy RRM measurement, while LR also stops performing the measurement on serving cell. Different with the offloading case that exit condition is based on the LR measurement results, since both MR and LR performs serving cell measurement for the relaxed case, RAN2 to discuss whether the MR measurement results should also be considered in the exit conditions in addition to the LR measurement results.

Proposal 4: For the exit conditions of relaxed case b, RAN2 to discuss whether to consider MR measurement results in addition to the LR measurement results.

Based on above analysis for the fully offloading case and the relaxed case b, the entry conditions define the criterion for UEs to start performing the serving cell measurement offloading or relaxation from the legacy RRM measurement mode. While the exit conditions define the criterion for UEs to stop performing the serving cell measurement offloading or relaxation and fallback to legacy RRM measurement mode directly. However, whether the UE can perform measurement transition directly between the fully offloading case and relaxed case b can be further discussed. For example, UE is performing the serving cell measurement relaxation b and then transits to the fully offloading case directly when some conditions are fulfilled, or UE is performing the fully offloading measurement on the serving cell and then fallbacks to the serving cell measurement relaxation based on some defined conditions. If the transition is identified feasible, then corresponding conditions for the transition between the two cases may need to be further defined, or the existing separate entry/exit conditions for fully offloading case and relaxed case b can still be applied to the transition case based on UE implementation. Therefore, it is suggested RAN2 to discuss the necessity/feasibility of the measurement status transition between the fully offloading case and relaxed case b directly.

Proposal 5: RAN2 to discuss the necessity/feasibility of the measurement status transition between the fully offloading case and relaxed case b directly.

2.3 Neighboring cell measurement relaxation

As concluded in RAN2#126 meeting [2], the RRM measurement of the neighboring cell can only be performed by MR. And in RAN2#127bis meeting, RAN2 assumes ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. In our understanding, ‘low mobility’ criterion guarantees that the UE has low possibility to perform the cell reselection, then the neighboring cell measurement can be relaxed from the MR side. It is suggested RAN2 to agree that ‘low mobility’ criterion is reused for neighboring cell measurement relaxation for UEs capable of LP-WUS in addition to ‘not at cell edge’ criterion.

Proposal 6: RAN2 to agree the ‘low mobility’ criterion is reused for neighboring cell measurement relaxation for UEs capable of LP-WUS.

More specifically, according to the neighboring cell measurement behaviors defined by RAN4, for the fully offloading case MR neighboring cell measurement is off and FFS on the conditions and details. In this case, LR is performing serving cell measurement while the MR may not perform any neighbor cell measurements. In such a case, we think that it may be necessary to define LR based measurements (e.g., PSS/SSS/PBCH DMRS, LP-SS, etc.) to reevaluate whether neighbor cell measurement on the MR need to be resumed/triggered. Furthermore, in the case that neighboring cell measurement is performed, it is unclear on whether the serving cell measurement of MR can be available for this scenario, if it is no available, then the neighboring cell measurement relaxation criterion on MR needs to be evaluated based on the LR measurement results e.g., PSS/SSS/PBCH DMRS, LP-SS, etc. Based on legacy neighbor cell measurement relaxation criteria i.e., low mobility condition, not-at-cell edge conditions, some examples have been provided below based on the LR measurement:

| Low mobility criterion (SrxlevRef-lr – Srxlev-lr) < SSearchThresholdP-lr | - Srxlev-lr =cell selection Rx level value of the serving cell (dB) performed by LR. - SrxlevRef-lr = reference Srxlev value of the serving cell (dB) performed by LR. - SSearchThresholdP-lr= the Srxlev threshold (in dB) for relaxation on LR neighboring cell measurement. |
| --- | --- |
| Not-at-cell-edge criterion: Srxlev-lr > SSearchThresholdP-lr, and Squal-mr > SSearchThresholdQ-lr.. | - Srxlev-lr = cell selection Rx level value of the serving cell (dB) performed by LR. - Squal-lr = cell selection quality value of the serving cell (dB) performed by LR. - SSearchThresholdP-lr = the Srxlev threshold (in dB) for relaxation on LR neighboring cell measurement. - SSearchThresholdQ-lr = the Squal threshold (in dB) for relaxation on LR neighboring cell measurement. |

Therefore, for fully offloading case, MR may resume/trigger the neighbor cell measurement when the LR measurement results on serving cell satisfy the configured criteria, whether the resumed neighbor cell measurement needs to be relaxed or performed as legacy may be left to FFS. If further relaxation is needed for the neighboring cell, the LR measurement results on serving cell can be used to evaluate above criteria if configured in the case of serving cell measurement on MR is not available.

Proposal 7: For fully offloading case, RAN2 may consider the feasibility of the case that neighboring cell measurement on MR is triggered or resumed by the LR measurement result on the serving cell.

Proposal 8: For fully offloading case, LR measurement results on serving cell can be used to evaluate the ‘not-at-cell-edge’ criterion and/or ‘low mobility’ criterion for neighboring cell measurement relaxation (if performed) in the case of the serving cell measurement on MR is not available.

For the relaxed case b, as defined by RAN4, the MR neighboring cell is ON with the relaxation measurement. In this case, MR serving measurement is also ON to perform the relaxation measurement. Then the neighboring cell measurement relaxation conditions can fully follow legacy, when MR measurement results satisfy ‘low mobility’ or ‘not-at-cell-edge’ criterion if configured, the neighboring cell measurement relaxation is performed.

Proposal 9: For the relaxed case b, MR performs neighboring cell measurement relaxation as legacy, i.e., when MR measurement results satisfy ‘not-at-cell-edge’ criterion and/or ‘low mobility’ criterion if supported.

2.4 Cell (re)selection impacts

As indicated in the TR 38.869[6], in RRC_IDLE and RRC_INACTIVE state, RAN2 to specify how UE performs RRM measurements and corresponding cell selection/re-selection to ensure that UE is camping on the best cell. The procedures may need to be modified to accommodate operation using LP-WUS/WUR. In the case of UE configured with LP-WUS/WUR is triggered to perform cell (re)selection in RRC_INACTIVE/RRC_IDLE, to ensure the LR availability and minimize the power consumption caused by MR monitoring in the new camping cell, UE may prioritize the cell(s) supporting LP-WUS/WUR operation.

Proposal 9: When cell (re)selection is performed, UE configured with LP-WUS/WUR may prioritize the cell(s) supporting LP-WUS/WUR operation.

- Conclusion

Observation 1: RAN4 has approved fully offloading case and relaxed case b as below while other cases on RRM measurement relaxation and offloading are pending for further study:

•	Fully offloading case: serving cell measurement is fully offloaded from MR to LR while MR performs no measurement on serving cell.

•	Relaxed case b: serving cell measurement is relaxed on MR while LR also performs the serving cell measurement.

Proposal 1: RAN2 to discuss additional entry condition for fully serving cell measurement offloading case, e.g., low mobility conditions evaluated by the MR.

Proposal 2: The exit conditions for serving cell measurement offloading considers high mobility condition evaluated by the LR.

Proposal 3: For the relaxed case b, the entry conditions on serving cell measurement relaxation can reuse the entry conditions of fully offloading case, with different thresholds configuration.

Proposal 4: For the exit conditions of relaxed case b, RAN2 to discuss whether to consider MR measurement results in addition to the LR measurement results.

Proposal 5: RAN2 to discuss the necessity/feasibility of the measurement status transition between the fully offloading case and relaxed case b directly.

Proposal 6: RAN2 to agree the ‘low mobility’ criterion is reused for neighboring cell measurement relaxation for UEs capable of LP-WUS.

Proposal 7: For fully offloading case, RAN2 may consider the feasibility of the case that neighboring cell measurement on MR is triggered or resumed by the LR measurement result on the serving cell.

Proposal 8: For fully offloading case, LR measurement results on serving cell can be used to evaluate the ‘not-at-cell-edge’ criterion and/or ‘low mobility’ criterion for neighboring cell measurement relaxation (if performed) in the case of the serving cell measurement on MR is not available.

Proposal 8: For the relaxed case b, MR performs neighboring cell measurement relaxation as legacy, i.e., when MR measurement results satisfy ‘not-at-cell-edge’ criterion and/ or ‘low mobility’ criterion if supported.

Proposal 9: When cell (re)selection is performed, UE configured with LP-WUS/WUR may prioritize the cell(s) supporting LP-WUS/WUR operation.

- References

3GPP RAN#103, RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR).

3GPP RAN2#126, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

3GPP RAN2#127, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

3GPP RAN2#127bis, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

3GPP RAN4 #112 RRM session meeting report

3GPP TR 38.869, Study on low power wake up signal and receiver for NR; (Release 18).