3GPP TSG-RAN WG2 Meeting #128			                                 	R2- 2410284

Orlando, USA, Nov.  18th – 22nd, 2024

Agenda Item:	8.5.2

Source: 	Lenovo

Title: 	Issues on the procedure of on-demand SSB SCell operation

Document for:	Discussion and decision

# Introduction

In RAN2#127bis, some agreements were achieved [1].

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

And the issue on L3 RRM framework is FFS including two potential options:

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

In RAN1#118, some agreements were achieved [2].

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

In RAN1#118b, some agreements were achieved [3].

Agreement

- For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options.

  - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication

  - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - Option 3: Configuration/indication of the duration of on-demand SSB transmission window

  - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell

  - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired

  - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed

  - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI

  - FFS: Each option is applicable to which Cases or Scenarios

  - FFS: Details related to each of the above options

In this contribution, we provide analysis and proposals on procedures and signalling method(s) to support on-demand SSB SCell operation for UE in CONNECTED mode configured with CA, for both intra-/inter-band CA, also we discuss the impact on UE in IDLE/INACTIVE mode.

# Discussion

Configuration of OD-SSB

Referring to RAN1 progress, it was agreed to provide at least Frequency of the on-demand SSB, SSB positions within an on-demand SSB burst by using signalling similar to ssb-PositionsInBurst and Periodicity of the on-demand SSB via higher layer RRC signalling. Also, in RAN1#118, it was agreed to configure multiple candidate values by RRC for on-demand SSB at least including Periodicity of the on-demand SSB.Since the existing RRC message can configure cell specific parameters of a UE's serving cell, e.g. when configuring a UE with an SCell or with an additional cell group (SCG), we can reuse the RRC message to configure the on-demand SSB related information.

Proposal 1: Reuse existing RRC message to configure the on-demand SSB related information to UE.

Considering the existing configuration of periodicity for SSB is enumerated which cannot provide multiple values, a new IE may be added to provide multiple candidate values of periodicity of the on-demand SSB.

Proposal 2: A new IE may be added to provide multiple candidate values of periodicity of the on-demand SSB.

L3 measurement

Once informed about serving and/ or neighbor cell transmitting SSB on on-demand basis, UE prioritizes measurement of such object: UE may start with the measurement of the said object before any other measurements. Network may use measSequence to achieve this purpose and UE shall prioritize measurement of such an object. Otherwise, UE signals a start time to measure a measurement object (SSB provided on-demand) and the end time for the same can also be indicated accordingly, to enable the network to stop SSB transmission as soon as possible.

Proposal 3: A UE informs the network about the start time and end time of L3 measurement on a frequency with at least one cell providing SSB on-demand.

Considering the dynamical change of OD-SSB transmission e.g. OD-SSB transmission, it would be complicated to configure RRM measurement with SMTC. To simplify the implementation and reduce spec impact, performing RRM measurement based on OD-SSB pattern and RAN4 requirement when OD-SSB is transmitted.

Proposal 4: RRM measurement is performed based on OD-SSB pattern and RAN4 requirement when OD-SSB is transmitted.

# Conclusion

In summary, we have following observations and proposals on the operation of on-demand SSB for SCell:

Proposal 1: Reuse existing RRC message to configure the on-demand SSB related information to UE.

Proposal 2: A new IE may be added to provide multiple candidate values of periodicity of the on-demand SSB.

Proposal 3: A UE informs the network about the start time and end time of L3 measurement on a frequency with at least one cell providing SSB on-demand.

Proposal 4: RRM measurement is performed based on OD-SSB pattern and RAN4 requirement when OD-SSB is transmitted.

# References

- RAN2#127bis chairman note.

- RAN1#118 chairman note.

- RAN1#118b chairman note.