3GPP TSG-RAN WG2 Meeting #128				        		       R2- 2410285

Orlando, USA, Nov.  18th – 22nd, 2024

Agenda Item:	8.5.4

Source: 	Lenovo

Title:  	Paging and PRACH adaptation

Document for:	Discussion and decision

# Introduction

In RAN2#128 [1], paging adaptation was discussed, and following agreements were achieved:

Agreements on paging adaptation

- Select option-b as baseline for R19 NES paging enhancement.

- R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

- Legacy UE is not barred.

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

Companies have already proposed bundling of Paging Frames in the last meetings [2][3], even keeping the total number of Paging occasions same. This was also studied and captured in the TR 38.864.

This paper intends to check if it is possible to achieve better energy saving with respect to paging transmission. Also, PRACH adaptation related issues are discussed in this paper.

# Discussion

## Paging enhancement

Some logs from different cities and operators were previously presented in [4] and based on this we think that option-b i.e., extended values of N should be used to have increased interval between PFs (e.g. T/64, T/128 …T/N). The value T/N will allow to have just one PF in the Paging DRX cycle.

Proposal 1: RAN2 decide on values of N is extended to have increased interval between PFs (e.g. T/64, T/128 …T/N).

As legacy UEs can’t know about the R19 changes, the legacy paging occasions (PF+POs) are kept as it is and since their number will dwindle with time, the required paging for such UEs will go down further.

Proposal 2: For legacy UEs the legacy paging mechanism can be continued.

## RACH enhancement

In RAN1#118bis discussion on the RACH adaptation for Rel-19 took place with the following agreements:

Agreement

For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources.

Agreement

For adaptation of PRACH in time-domain, support both of the following

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources

- FFS: Additional details

Agreement

For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different

- FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement).

- Discuss further following options for signaling

- Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources.

  - msg1-FrequencyStart at least for 4-step RACH

  - FFS: other applicable legacy frequency domain parameter(s)

- Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources

  - Note: Offset to legacy frequency domain parameter(s) is a new parameter

  - FFS: applicable legacy frequency domain parameter(s)

According to RAN1 agreement, the configuration of additional PRACH resources is provided by semi-static signaling. From point view of RAN2 specification, the configuration of additional PRACH resources is separately configured from the legacy RACH configuration in SIB 1, wherein the configuration of additional PRACH resources needs to indicate at least the PRACH configuration index, Number of SSBs per RACH occasion. The other config(s) e.g., msg1-FDM, msg1-FrequencyStart, may be also separately configured from the legacy RACH configuration for flexibility. If the other config(s) are not separately configured, UE default reuses the config in legacy RACH configuration.

Proposal 3: RAN2 to agree the additional RACH occasion for adaptation in time domain is separately configured in RACH configuration from legacy RACH configuration in SIB1.

Proposal 4: RAN2 to agree the separate RACH configuration indicates at least Number of SSBs per RACH occasion and the other config(s) e.g., msg1-FDM, msg1-FrequencyStart, may be also separately configured. If the other config(s) are not separately configured, UE default reuses the config in legacy RACH configuration.

# Conclusion

In summary, we have following observations and proposals for adaptation of common signal/channel transmissions:

Proposal 1: RAN2 decide on values of N is extended to have increased interval between PFs (e.g. T/64, T/128 …T/N).

Proposal 2: For legacy UEs the legacy paging mechanism can be continued.

Proposal 3: RAN2 to agree the additional RACH occasion for adaptation in time domain is separately configured in RACH configuration from legacy RACH configuration in SIB1.

Proposal 4: RAN2 to agree the separate RACH configuration indicates at least Number of SSBs per RACH occasion and the other config(s) e.g., msg1-FDM, msg1-FrequencyStart, may be also separately configured. If the other config(s) are not separately configured, UE default reuses the config in legacy RACH configuration.

# References

[1] RAN2#128 chairman notes.

[2] RAN2#127 chairman notes.

[3] RAN2#126 chairman notes.

[4] R2-2406890 Paging statistics from field and PRACH adaptation Lenovo

[5] Summary of [109] OPPO