3GPP TSG-RAN WG2 Meeting #128	R2-2410288

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.13.2

Source:	Nokia

Title:	Relay discovery aspects for multi-hop relay

WID/SID:	NR_SL_relay_multihop - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RANP#104, the WID on multi-hop relay is approved in RP-241609. In this contribution, we present our initial thought on the potential/necessary enhancement for relay discovery and reselection to support multi-hop solution.

# 2	Discussion

At least for approach 1 in the post RAN2#127 email discussion, where the U2N relay solution is extended to multi-hop U2N operation, we think re-use of the Rel-17 U2N Remote UE and U2N Relay UE functionality is desired as it would simplify both the specification and deployment of multi-hop relay solution. It will also enable wider applicability of UEs to support multi-hop relay. Therefore, we think that RAN2 should attempt to enable the re-use of the Rel-17 U2N Remote and U2N Relay UEs at least for the case when there is only one additional hop.

Observation 1: Reusing the existing Rel-17 U2N procedures as much as possible makes for a simplified solution while allowing existing Rel-17/18 L2 U2N Remote/Relay UE to benefit from multi-hop solution.

Proposal 1: At least for Rel-17/18 UEs, legacy relay discovery messages should be reused.

However, the Rel-17 U2N relay solution might not be optimized for MH relay scenarios. For example, some RRC procedures would not be work well with multi-hop transmission since RRC timer values may not be long enough to cover the latency caused by MH transmission. In order to enable efficient MH relay operation while maintaining backwards compatibility, it is reasonable to investigate whether to differentiate procedures when the relay and intermediate UE is either a Rel-17/18 UE or a Rel-19 UE.

Observation 2: The MH relay operation may benefit from advanced procedures when the Relay UE is a release 19 UE.

Proposal 2: RAN2 can enhance the relay discovery message in case of Rel-19 Remote and intermediate Relay UEs to include i.e. number of hops. FFS whether to use existing or new message.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: Reusing the existing Rel-17 U2N procedures as much as possible makes for a simplified solution while allowing existing Rel-17/18 L2 U2N Remote/Relay UE to benefit from multi-hop solution.

Proposal 1: At least for Rel-17/18 UEs, legacy relay discovery messages should be reused.

Observation 2: The MH relay operation may benefit from advanced procedures when the Relay UE is a release 19 UE.

Proposal 2: RAN2 can enhance the relay discovery message in case of Rel-19 Remote and intermediate Relay UEs to include i.e. number of hops. FFS whether to use existing or new message.