3GPP TSG-RAN WG2 Meeting #128	R2-2410291

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	8.9.3

Source: 	NEC

Title:	        Discussion on UL Capacity Enhancement for IoT-NTN

Document for:	 Discussion and Decision

- Introduction

In RAN2#127bis, the Msg3 transmission without msg1/ RAR has reached the following agreements [1]:

Agreements:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

2.	RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

3.	At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

6.	At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

8.	CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

In this contribution, we continue our discussion on enhancing UL capacity through resource configuration, contention resolution, efficient delivery of DL messages, failure handling for the CB-msg3 EDT, and NPRACH enhancement with OCC.

- Discussion

As both DSA and OCC2 are considered in RAN2 [1], the CB-msg3 EDT with multiple resources in the time, frequency, and code domains can be illustrated in Fig. 1 below.

Fig. 1 CB-msg3 EDT

When M opportunities are considered in the time domain for DSA and/or SA, there will be an EDT occasion group with a duration equal to or larger than M multiply a unit EDT occasion duration.

For OCC2, there will be two individual opportunities for each EDT occasion in the time-frequency domain.

The configured number of subcarriers, SCS, and RU definitions determines the frequency domain's freedom.

  - Trigger condition

The CB-msg3 EDT is a type of NPUSCH transmission that requires a higher SNR for successful decoding compared to NPRACH. Since the uplink signal is not measured before transmitting the CB-msg3 EDT, the network cannot control the transmit power, modulation coding scheme (MCS), or the number of repetitions in a closed-loop manner. Consequently, there is no guarantee that the CB-msg3 EDT will be successfully decoded, even if it does not conflict with other CB-msg EDTs. To better manage potential conflicts with the CB-sg3 EDT at an appropriate rate, the network must control the number of enabled CB-msg3 EDT terminals. Giving the terminals under better channel conditions with the CB-msg3 EDT opportunities can restrict the conflict on the network side and save the power on the terminal side.

Similar to the legacy NPRACH CE level determination, enabling the CB-msg3 EDT can be based on the UE-measured RSRP and the network-configured RSRP threshold.

Proposal 1: Support the RSRP threshold for enabling CB-msg3 EDT.

  - Resource configuration

For the legacy NB-IoT random access procedure, the number of RUs and TBS for EDT are defined in Table 16.3.3-2 and Table 16.3.3-3 in [2], and EDT-TBS-InfoList-NB-r15 in [3] as below:

Table 16.3.3-2: EDT TBS for Msg3 NPUSCH with edt-SmallTBS-Enabled set to 'true' [2]

| edt-TBS | edt-SmallTBS-Subset | Allowable TBS values |
| --- | --- | --- |
| 408 | not configured | 328, 408 |
| 504 | not configured | 328, 408, 504 |
| 504 | enabled | 408, 504 |
| 584 | not configured | 328, 408, 504, 584 |
| 584 | enabled | 408, 584 |
| 680 | not configured | 328, 456, 584, 680 |
| 680 | enabled | 456, 680 |
| 808 | not configured | 328, 504, 680, 808 |
| 808 | enabled | 504, 808 |
| 936 | not configured | 328, 504, 712, 936 |
| 936 | enabled | 504, 936 |
| 1000 | not configured | 328, 536, 776, 1000 |
| 1000 | enabled | 536, 1000 |

Table 16.3.3-3: MCS index for Msg3 NPUSCH and EDT [2]

| MCS Index | Number of RUs |  |  |
| --- | --- | --- | --- |
|  | edt-TBS = 328, 408, 504, or 584 | edt-TBS = 680 | edt-TBS = 808, 936, or 1000 |
| '011' | 3 | 3 | 4 |
| '100' | 4 | 4 | 5 |
| '101' | 5 | 5 | 6 |
| '110' | 6 | 8 | 8 |
| '111' | 8 | 10 | 10 |

EDT-TBS-InfoList-NB-r15 ::=	SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF EDT-TBS-NB-r15

EDT-TBS-NB-r15 ::=	SEQUENCE {

edt-SmallTBS-Enabled-r15		BOOLEAN,

edt-TBS-r15						ENUMERATED {b328, b408, b504, b584, b680, b808, b936, b1000}

}

Observation 1: The candidate RU numbers for EDT are 3, 4, 5, 6, 8, and 10, with each RU number covering various TBSs in the range of 328 to 1000.

The duration of a RU in different SCS and tone configurations for frame structure type1 and NPUSCH format 1 are defined in Table 10.1.2.3-1 of [4] as below:

Table 10.1.2.3-1: Supported combinations of , , and  for frame structure type 1[4]

| NPUSCH format |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3.75 kHz | 1 | 16 | 7 |
|  | 15 kHz | 1 | 16 |  |
|  |  | 3 | 8 |  |
|  |  | 6 | 4 |  |
|  |  | 12 | 2 |  |
| 2 | 3.75 kHz | 1 | 4 |  |
|  | 15 kHz | 1 | 4 |  |

Observation 2: The duration of one EDT depends on the required number of RUs and the corresponding number of slots per RU.

To fulfill the coverage requirements in IoT-NTN, whether in LEO or GEO scenarios, repeating NPUSCH transmission is necessary. For CB-msg3 EDT repetition, this can be achieved as illustrated in Fig. 2 and Fig. 3 below:

Fig. 2 Intra-occasion repetition

Fig. 3 Inter-occasion group repetition

According to the channel described in [5], the duration of one EDT significantly exceeds the coherent time of the NTN channel. As a result, executing coherent combinations for inter-occasion group repetition is not feasible, meaning that the full benefits of repetition gain cannot be realized.

Proposal 2: RAN2 will continue their work on CB-msg3 EDT, assuming that coverage enhancement is achieved through intra-occasion repetition, at least for DSA.

The CE level, which determines the number of repetitions, along with the required number of RUs (related to the TBS) and the required number of slots for each RU (determined by the SCS and the number of tones), determines the duration of one EDT occasion.

A terminal determines the CE level by measuring the DL reference signals. It then identifies the required number of slots for each RU based on the UL configurations broadcasted by the network. Additionally, the terminal calculates the required number of RUs using the boolean value of edt-SmallTBS-Enabled along with the TBS to be transmitted. Subsequently, the terminal can select the appropriate CB-message EDT resources from the resource list configured by the network to transmit the CB-msg3 EDT.

Proposal 3: CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level, the number of RUs required by the TBS, and the number of slots required by each RU.

For DSA, a terminal must be aware of the appropriate resources of the EDT occasion group with M opportunities.

Proposal 4: CB-msg3 EDT cell specific PUSCH resource groups for Msg3 transmission are provided per CE level, the number of RUs required by the TBS, the number of slots required by each RU, and the opportunity number.

Proposal 5: Each CB-msg3 EDT resource should be close to adjacent resources in the time domain for DSA resource group configuration.

  - Contention resolution

The legacy RA-RNTI is defined as [6, TS 36.321]:

RA-RNTI=1 + floor(SFN_id/4) + 256*carrier_id

where SFN_id is the index of the first radio frame of the specified PRACH and carrier_id is the index of the UL carrier associated with the specified PRACH. The carrier_id of the anchor carrier is 0.

Since the minimum number of required RUs for EDT is 3, the corresponding frame number is also 3 when using a 3.75 kHz SCS. Therefore, a CB-msg3 transmitted in frame 0 and frame 3 on the same carrier ID will have the same RA-RNTI value.

Observation 3: The legacy RA-RNTI has an ambiguity issue with frame number modulo 4 for scheduling CB-msg3 EDT responses.

Since the maximum number of required RUs for EDT is 10, the corresponding frame number is 8 under a 3.75 kHz SCS. When a repetition number of 128 is configured, the first CB-msg3 EDT transmitted in SFN 0 of hyper SFN 0 will have a second CB-msg3 EDT transmitted in SFN 0 of hyper SFN 1. The first and second transmissions will have the same RA-RNTI value when they are transmitted on the same carrier.

Observation 4: The legacy RA-RNTI has an ambiguity issue with frame number modulo 1024 for scheduling CB-msg3 EDT responses.

The resource of one CB-msg3 EDT group will span multiple frames; the opportunity ID can be utilized to derive the RNTI, addressing RNTI ambiguity issues, particularly for DSA.

Observation 5: The opportunity ID can be used to determine the RNTI for scheduling CB-msg3 EDT response, at least for DSA.

If the subcarrier ID is used to determine the RNTI, 6 bits are needed to identify the 48 subcarriers. However, since the total number of bits in an RNTI is 16, there may not be enough bits available in the RNTI to identify the subcarrier ID.

Proposal 6: The RNTI used to schedule the CB-msg3 response is derived based on the following list:

- SFN

- Hyper SFN

- Carrier ID

- Subcarrier ID

FFS: parameter downselection and detailed RNTI design

Since the CB-msg3 PUSCH resource is specific to the cell for Msg3 transmission and the RNTI is derived from the PUSCH resource according to the agreements from the last meeting, it is possible for multiple UEs to share the same RNTI. As a result, additional contention resolution is necessary for the CB-msg3 EDT procedure. In this context, the legacy 48-bit contention resolution ID may be reused. If the contention resolution ID from the network matches the first 48 bits of the CCCH SDU transmitted in the CB-msg3 PUSCH, the contention resolution is deemed successful.

Proposal 7: Contention resolution ID is introduced for further contention resolution.

After transmitting Msg3 PUSCH, a response window or timer must be introduced to receive the corresponding response message, similar to the RACH procedure.

For NB-IoT, only type-B half-duplex FDD operation is supported [4]. In this type of operation, guard periods (GP, illustrated in Fig. 4), each referred to as a half-duplex guard subframe, are created by the UE by

-	not receiving a downlink subframe immediately preceding an uplink subframe from the same UE, and

-	not receiving a downlink subframe immediately following an uplink subframe from the same UE.

Fig. 4 Type-B half-duplex FDD

Since multiple EDT occasions need to be timed closely together to manage the overall delay of the CB-msg3 EDT in DSA mode, altering the CB-msg3 EDT resource configuration will not resolve the half-duplex FDD subframe conflict issues.

The legacy RA Response window starts at the subframe that contains the end of the last preamble repetition plus X subframes plus UE-eNB RTT and has length ra-ResponseWindowSize for the corresponding enhanced coverage level, where value X is determined based on the used preamble format and the number of NPRACH repetitions[36.321,6].

For DSA, if the CB-msg3 EDT Response window starts at the subframe that contains the end of the last NPUSCH repetition plus X subframes plus UE-eNB RTT, it might 1) conflict with the terminal’s UL subframes with a small X value; or 2) introduce additional latency with a large X value as illustrated in Fig. 5.

To effectively control the overall CB-msg3 EDT delay in DSA mode, multiple EDT occasions must be configured closely together in the time domain. Therefore, changing the CB-msg3 EDT resource configuration is not an option to resolve the half-duplex FDD subframe conflict issues.

Observation 6: The legacy response window design has issues with half-duplex FDD subframe conflicts and additional latency, at least in DSA mode.

Proposal 8: Introduce a CB-msg3 EDT response window for UE to receive the response to CB-msg3 EDT.

To solve the above issues, the CB-msg3 EDT Response window can start at the last subframe that contains the last EDT occasion of one EDT occasion group plus Y subframes plus UE-eNB RTT and has length cbmsg3edt-ResponseWindowSize for the corresponding CB-msg3 EDT resource (group) configuration. This can be illustrated in Fig. 6.

Fig. 5 Legacy response window

Fig. 6 CB-msg3 EDT response window

Proposal 9: The CB-msg3 EDT response window starts at the last subframe that contains the last EDT occasion of one EDT occasion group plus Y subframes plus UE-eNB RTT and has length cbmsg3edt-ResponseWindowSize for the corresponding CB-msg3 EDT resource (group) configuration, at least for DSA mode.

FFS: the value list of Y and the cbmsg3edt-ResponseWindowSize

  - Efficient delivery of the response message

For DL response message, when multiple UEs select the same PUSCH resource, since RNTI is derived based on the PUSCH resource, the same RNTI may be used for multiple UEs. Then DL response messages from multiple UEs with same RNTI may be multiplexed into one MAC PDU for efficient delivery similar to MAC RAR/sucessRAR of RACH procedure.

- Proposal 10: The response messages from multiple UEs with the same RNTI can be multiplexed into one MAC PDU for efficient delivery.

For the content of DL response message, besides contention resolution ID, RRCEarlyDataComplete message including DL data/NAS signalling for CP-CIoT optimizations or RRCConnectionRelease message and possible DL data for UP-CIoT optimizations may be transmitted. If DL response messages from multiple UEs are multiplexed into one MAC PDU, and RRC message and/or DL data from multiple UEs are included, the MAC PDU will be very large. Hence, similar to MSGB of 2-step RACH procedure, C-RNTI can be included in MAC PDU, such that RRC message and/or DL data can be scheduled with the indicated C-RNTI in the following time.

- Proposal 11: C-RNTI can be included in the response message to further receive the possible RRC message and/or DL data.

  - Failure handling and fallback

The behavior of the UE needs to be defined if it fails to receive the corresponding contention resolution/response in the response window, similar to the 2 or 4-step RACH procedure. One potential solution is to allow the UE to re-initiate CB-msg3 transmission for a configured number of times before declaring complete failure. Then, whether and how to implement power control and power ramping for CB-msg3 transmission and retransmission should be investigated.

The network must control and distribute the load over the CB resources for CB-msg3 transmission. For instance, when the load is temporarily too high, the network could instruct the UE to apply a backoff for upcoming transmission over CB-msg3 resources. A procedure for the network to generally control a reasonable amount of UEs to use CB-msg3 transmission and configure a proper amount of CB PUSCH resources for it should be introduced. However, this may not solve the issue of high user density in the long term.

If CB-msg3 fails, the UE can also fall back to the normal RACH-based solution, similar to the 2-step RACH falling back to 4-step RACH. The conditions and triggers of this fallback need to be discussed, for example, after the maximum time of CB-msg3 retransmission or after receiving the fallback command from the network.

- Proposal 12: RAN2 to discuss the following failure handling and fallback issues:

- CB-msg3 retransmission

- NW controlled backoff

- The condition/trigger for a UE to fallback from CB-msg3 to RACH-based solution

  - NPRACH enhancement with OCC

Companies have observed the capacity benefits of using intra-symbol group OCC to enhance the NPRACH and have provided detailed design proposals in RAN1[7]. When a terminal transmits NPRACH using a UL subframe, it cannot simultaneously transmit NPUSCH or receive any DL signals (as shown in Fig. 4). NPRACH enhancement with OCC, could better utilize the limited UL resource by reducing the UL subframe for NPRACH, and increasing the transmission opportunities for NPUSCH and/or NPDSCH. Therefore, enhancing NPRACH with OCC is beneficial. Following the statement in the WID [8], the extension of NPRACH with OCC should be specified.

Proposal 13: RAN2 further discusses the NPRACH capacity enhancement with OCC.

- Conclusion

This contribution provides our analysis and views for the CB-msg3, the following observations and proposals are listed below:

Observation 1: The candidate RU numbers for EDT are 3, 4, 5, 6, 8, and 10, with each RU number covering various TBSs in the range of 328 to 1000.

Observation 2: The duration of one EDT depends on the required number of RUs and the corresponding number of slots per RU.

Observation 3: The legacy RA-RNTI has an ambiguity issue with frame number modulo 4 for scheduling CB-msg3 EDT responses.

Observation 4: The legacy RA-RNTI has an ambiguity issue with frame number modulo 1024 for scheduling CB-msg3 EDT responses.

Observation 5: The opportunity ID can be used to determine the RNTI for scheduling CB-msg3 EDT response, at least for DSA.

Observation 6: The legacy response window design has issues with half-duplex FDD subframe conflicts and additional latency, at least in DSA mode.

Proposal 1: Support the RSRP threshold for enabling CB-msg3 EDT.

Proposal 2: RAN2 will continue their work on CB-msg3 EDT, assuming that coverage enhancement is achieved through intra-occasion repetition, at least for DSA.

Proposal 3: CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level, the number of RUs required by the TBS, and the number of slots required by each RU.

Proposal 4: CB-msg3 EDT cell specific PUSCH resource groups for Msg3 transmission are provided per CE level, the number of RUs required by the TBS, the number of slots required by each RU, and the opportunity number.

Proposal 5: Each CB-msg3 EDT resource should be close to adjacent resources in the time domain for DSA resource group configuration.

Proposal 6: The RNTI used to schedule the CB-msg3 response is derived based on the following list:

- SFN

- Hyper SFN

- Carrier ID

- Subcarrier ID

FFS: parameter downselection and detailed RNTI design

Proposal 7: Contention resolution ID is introduced for further contention resolution.

Proposal 8: Introduce a CB-msg3 EDT response window for UE to receive the response to CB-msg3 EDT.

Proposal 9: The CB-msg3 EDT response window starts at the last subframe that contains the last EDT occasion of one EDT occasion group plus Y subframes plus UE-eNB RTT and has length cbmsg3edt-ResponseWindowSize for the corresponding CB-msg3 EDT resource (group) configuration, at least for DSA mode.

FFS: the value list of Y and the cbmsg3edt-ResponseWindowSize

- Proposal 10: The response messages from multiple UEs with the same RNTI can be multiplexed into one MAC PDU for efficient delivery.

- Proposal 11: C-RNTI can be included in the response message to further receive the possible RRC message and/or DL data.

- Proposal 12: RAN2 to discuss the following failure handling and fallback issues:

- CB-msg3 retransmission

- NW controlled backoff

- The condition/trigger for a UE to fallback from CB-msg3 to RACH-based solution

Proposal 13: RAN2 further discusses the NPRACH capacity enhancement with OCC.

- References

[1] RAN2#127 chairman notes

[2] TS 36.213

[3] TS 36.331

[4] TS 36.211

[5] TR 38.821

[6] TS 36.321

[7] R1-2409309, Final FL Summary for IoT-NTN, Moderator (Sony)

[8] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. (Rapporteur)