3GPP TSG-RAN WG2#128											      R2-2410292

Orlando, USA, Nov.  18th – 22nd , 2024

Agenda Item:	8.9.4

Source: 	NEC

Title:	        Discussion on PWS broadcast for NB-IoT

Document for:	 Discussion and Decision

- Introduction

In the last RAN2 meeting, regarding PWS of NB-IoT, the following agreements were reached.

Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

In this contribution, we will discuss the remaining issues and provide our views.

- Discussion

In the current specification [2], for DCI format N2, when the format CRC is scrambled by P-RNTI and Flag =0, direct indication information can be transmitted by DCI. When format CRC is scrambled by P-RNTI and Flag = 1, paging message can be transmitted. In other words, scheduling information for paging message and direct indication information cannot be transmitted to the UEs by the same DCI information.

Observation 1: Scheduling information for paging message and direct indication cannot be transmitted to the UEs by the same DCI information.

According to the last meeting’s agreement, PWS indication including etws-indication and cmas-indication can be added in direct indication information for NB-IoT. Whether PWS indication is also added in paging-NB can be further discussed. In our understanding, it is beneficial to include PWS indication in paging-NB. When there is both paging record list and ETWS/CMAS indication, according to the current specification, only one of them can be transmitted to the UEs due to scheduling information for paging message and direct indication information cannot be transmitted by the same DCI. If paging message includes PWS indication, paging record list and PWS indication can be simultaneously transmitted to the UEs by paging-NB, which is very efficient. Besides, for eMTC UEs, PWS indication can be included in paging message and direct indication information, we think the same methods should also be used for NB-IoT UEs. Therefore, PWS indication should also be added in paging-NB.

Proposal 1: PWS indication including etws-indication and cmas-indication should also added in Paging-NB.

In the section 5.3.2.3 of TS 36.331 [2], when PWS notification from eMTC and LTE cell is received, the corresponding procedure description is as below:

1> if the etws-Indication is included and the UE is ETWS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary; 2> if the schedulingInfoList indicates that SystemInformationBlockType10 is present: 3> acquire SystemInformationBlockType10; 2> if the schedulingInfoList indicates that SystemInformationBlockType11 is present: 3> acquire SystemInformationBlockType11; 1> if the cmas-Indication is included and the UE is CMAS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary as specified in 5.2.1.5; 2> if the schedulingInfoList indicates that SystemInformationBlockType12 is present: 3> acquire SystemInformationBlockType12;

When etws-indication/cmas-indication is received, the UE can re-acquire SIB1 immediately. Immediately means that the UE can re-acquire SIB1 without waiting until the next system information modification period boundary according to the above description. Then for NB-IoT, the same description is also used. For example, upon receiving etws-indication/cmas-indication, the PWS-capable NB-IoT UE re-acquires SIB1 immediately, i.e. without waiting for the next system information modification period. Then the UE shall start receiving ETWS primary/secondary notification and/or CMAS notification according to schedulingInfoList contained in SIB1.

Proposal 2: Upon receiving etws-indication/cmas-indication, the UE re-acquire SIB1 without waiting the next system information modification period boundary.

Besides, whether and how to support PWS in connected mode can be further discussed as below.

Continue the discussion in the next meeting(s) on whether and how to support PWS for NB-IoT NTN UEs in Connected mode

In the current specification [2], NB-IoT UEs in RRC_CONNECTED cannot receive paging. Then when PWS notification arrives, if the NW wants to inform NB-IoT UE to receive PWS notification, it can only release UE to RRC_IDLE, which can create signalling storm issues. In addition, PWS acquisition in RRC_CONNECTED has been supported in Rel-16 for non-BL UE in CE. A similar mechanism may be used for NB-IoT UE. Therefore, it would be better to support PWS acquisition in RRC_CONNECTED for NB-IoT UE.

Proposal 3: PWS should be supported for NB-IoT NTN UEs in RRC-CONNECTED.

Similar to eMTC, in order to support PWS acquisition for NB-IoT UE in RRC_CONNECTED, a new direct indication information including etws-indication and cmas-indication should be introduced as shown in Table 1.

Table 1 Direct Indication information

| Bit | Direct Indication information |
| --- | --- |
| 1 | etws-Indication |
| 2 | cmas-Indication |
| 3, 4, 5, 6, 7, 8 | Not used, and shall be ignored by UE if received. |

Proposal 4: Introduce a new direct indication information including etws-indication and cmas-indication for NB-IoT UE in RRC_CONNECTED.

It is possible to blindly detect CSS and USS on the same narrowband for NB-IoT UE in RRC_CONNECTED. Then, in order to reduce UE power consumption, it makes sense for UE to know whether the network can support delivery of ETWS/CMAS indication in RRC_CONNECTED. Dedicated signalling may be used for informing whether ETWS/CMAS notification is enabled.

Proposal 5: A dedicated indication whether ETWS/CMAS notification is enabled for UE in RRC_CONNECTED mode should be introduced.

- Conclusion

This contribution provides our analysis and views for broadcast of PWS message, the following proposals are listed below:

Observation 1: Scheduling information for paging message and direct indication cannot be transmitted to the UEs by the same DCI information.

Proposal 1: PWS indication including etws-indication and cmas-indication should also added in Paging-NB.

Proposal 2: Upon receiving etws-indication/cmas-indication, the UE re-acquire SIB1 without waiting the next system information modification period boundary.

Proposal 3: PWS should be supported for NB-IoT NTN UEs in RRC-CONNECTED.

Proposal 4: Introduce a new direct indication information including etws-indication and cmas-indication for NB-IoT UE in RRC_CONNECTED.

Proposal 5: A dedicated indication whether ETWS/CMAS notification is enabled for UE in RRC_CONNECTED mode should be introduced.

- References

[1] Draft_RAN2_127b_Meeting_Report_v1.

[2] TS 36.212 i10.

[3] TS 36.331 i31.