3GPP TSG-RAN WG2 Meeting #128 	R2-2410300

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.2.2

Source:	Panasonic

Title:	Discussion on Ambient IoT segmentation functionality

Document for:	Discussion

# Introduction

In the previous meeting, the following agreements have been made regarding A-IoT segmentation functionality.

- Agreements on segmentations

- RAN2 will study segmentation and RAN2 will need to give some guidance once there is more clarity on solution, on the minimum TB size.

- Study the functionality of segmentation in the MAC.   We focus on D2R and can consider the support for R2D later depending on SA2 and RAN1.   The solution should aim to be applicable to both direction (i.e. unified solution), if we decide to support on both directions.

- Do not support sequence number for A-IoT segmentation functionalities.

- Do not support segment number and the number of segments for A-IoT segmentation functionalities.

- Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment.    FFS the details on how this is designed and number of bits (one or two).

- RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

In this paper, we would like to provide our views on how segmentation can work based on the above agreed direction, and in particular, to address the above FFS point.

# Discussion

In NR, the RLC protocol is responsible for segmentation of RLC SDUs from the PDCP into suitably sized RLC PDUs. It also handles retransmission of erroneously received PDUs, as well as removal of duplicate PDUs. To provide error-free delivery of segments, sequence number and segmentation offset are included in the header, based on which the receiver can then reassemble these segments into original packet. For A-IoT, since it has been agreed RLC layer is not supported, the segmentation will be performed by A-IoT MAC as mentioned in above agreement (see section 1).

Further, it is understood that segmentation and reassembly would add complexity. Therefore, the A-IoT segmentation should aim for simple solutions. Along this line of thinking, the A-IoT segmentation will not support sequence number, segment number and the number of segments for simplicity, as mentioned in section 1. With these targets, it needs to be further discussed how the receiver can identify any missing or duplicated segment and further recover from the error cases. The candidate solutions described in the following sections can be considered. We focus the discussion below on the D2R direction as it is prioritized for the study.

## ACK/NACK indication from reader to control segment delivery

In this solution, each segment transmitted by device will be explicitly acknowledged by the reader by ACK/NACK indication. If an ACK is received by device, device will move forward to the next segment transmission. If a NACK is received by device, device will transmit the same segment again. The ACK/NACK indication can be provided together with a scheduling grant to schedule the next D2R transmission.

Since the reader has no idea whether the device has completed all segments, one-bit indication should be sent from the device to let reader know whether the current segment is the last segment (including the case where there is no segmentation performed by device). Therefore, to address the FFS point in the previous agreement (see section 1), only one-bit is needed from device to reader to indicate whether the data is segmented and whether it is last segment.

The following Fig.1 illustrates the operation of this solution.

Fig.1. Illustration of solution 1 – ACK/NACK to control segment delivery

Observation 1: The following solution 1 is identified for D2R segmentation:

- ACK/NACK indication is used for reader to control whether device to transmit the next new segment or re-transmit previous segment.

- Device uses one-bit to indicate whether the data is segmented and whether it is the last segment.

Although being simple, the above solution cannot avoid error in segment delivery in some scenarios. If it occurs, neither reader or device can detect it due to lack of support of segment index and indication of total number of segments. One error case is illustrated in following Fig.2.

Fig.2. Illustration of an error case in solution 1

As shown in Fig.2, after successfully receiving 1st segment, the reader sends a new grant with ACK to request the device to send 2nd segment. However, this new grant is missed by the device. Consequently, the reader will request the re-transmission by sending another grant with NACK. Since the device has missed previous grant, the device would interpret this grant with NACK as requesting the retransmission of 1st segment. As a result, the device will proceed to send 1st segment again. There is no way for reader to find out such misalignment. In the end, the duplicated segment will be stored by the reader as a new segment, resulting in packet assembly error.

Observation 2: If ACK is missed by the device, solution 1 will result in duplicated segment without awareness of the receiver.

## Even/odd segment indication from reader to control segment delivery

In this solution, instead of sending ACK/NACK, the reader would explicitly request an even or odd segment. For example, the segments are counted as odd, even, odd, even, etc., corresponding to 1st, 2nd, 3rd and 4th, etc. But this does not mean that device (or reader) needs to remember the segment index as 1, 2, 3, 4, etc. The device may need to store one-bit information, e.g. to represent the last transmitted segment is odd or even (or alternatively and equivalently, the next un-transmitted segment is even or odd).

For example, the even/odd segment indication can be represented by the following table:

| Even/odd segment indication | Scheduler’s intention | Device behavior |
| --- | --- | --- |
| 0 | The reader is requesting the even number segment | If the next un-transmitted segment has the same even/odd value as the indicated one, the device transmits this segment; otherwise, device re-transmits the previous segment. |
| 1 | The reader is requesting the odd number segment |  |

Same as solution 1, the device also reports 1-bit indication to indicate whether the data is segmented and whether it is last segment, in order to let reader know whether more grants are needed.

Observation 3: The following solution 2 is identified for D2R segmentation:

- Even/odd segment indication is used for reader to request an even or odd segment, based on which device can decide to transmit the next new segment or retransmit the previous segment.

- Device uses one-bit to indicate whether the data is segmented and whether it is the last segment.

The following Fig.3 illustrates how solution 2 can overcome the issue for solution 1. As shown, after missing one grant for new transmission, the device and reader can still achieve the same understanding on which segment to transmit/receive. Compared to solution 1, solution 2 requires additional one-bit stored in device memory for even/odd segment counting. But such added complexity can be justified for more robust segment delivery compared to solution 1.

Observation 4: Solution 2 is more robust than solution 1 for segment delivery, with marginal price for additional one-bit storage in device memory for even/odd counting.

Fig.3. Illustration of solution 2 - even/odd segment indication

## Two-bit indication from device to report segment delivery

In this solution, device will report two-bit indication to the reader. The first bit is a new segment indication and the second bit is end mark indication. If the new segment bit is toggled (i.e. from 1 to 0, or from 0 to 1), it means a new segment is sent. On the other hand, if the new segment bit is the same as previous transmission, it means the previous segment is re-transmitted. This solution can be viewed as enhancement on solution 1 (using ACK/NACK indication from reader), because the new segment indication from the device can act as a confirmation on “whether I am actually moving to the next segment”. With this additional information, the chance of misunderstanding of segment delivery between device and reader is reduced.

The following Fig.4 illustrates how this solution 3 can solve the identified issue on solution 1.

Fig.4. Illustration of solution 3 – two-bit indication from device

Observation 5: The following solution 3 is identified for D2R segmentation:

- ACK/NACK indication is used for reader to control whether device to transmit the next new segment or re-transmit previous segment.

- Device uses two-bit indication:

  - New segment bit to indicate whether a new segment is transmitted by device

  - End mark bit to indicate whether the data is segmented and whether it is the last segment

Observation 6: Solution 3 is more robust than solution 1 for segment delivery, with marginal price for additional signaling overhead.

We suggest RAN2 to capture all 3 solutions in the TR and further down-select in the work item phase.

Proposal 1: Capture solutions 1, 2 and 3 in the above observations in the TR as candidate solutions for A-IoT segmentation functionality.

# Conclusion

In this paper, we have discussed A-IoT segmentation. We have the following observations and proposals.

Observation 1: The following solution 1 is identified for D2R segmentation:

- ACK/NACK indication is used for reader to control whether device to transmit the next new segment or re-transmit previous segment.

- Device uses one-bit to indicate whether the data is segmented and whether it is the last segment.

Observation 2: if ACK is missed by the device, solution 1 will result in duplicated segment without awareness of the receiver.

Observation 3: The following solution 2 is identified for D2R segmentation:

- Even/odd segment indication is used for reader to request an even or odd segment, based on which device can decide to transmit the next new segment or retransmit the previous segment.

- Device uses one-bit to indicate whether the data is segmented and whether it is the last segment.

Observation 4: Solution 2 is more robust than solution 1 for segment delivery, with marginal price for additional one-bit storage in device memory for even/odd counting.

Observation 5: The following solution 3 is identified for D2R segmentation:

- ACK/NACK indication is used for reader to control whether device to transmit the next new segment or re-transmit previous segment.

- Device uses two-bit indication:

  - New segment bit to indicate whether a new segment is transmitted by device

  - End mark bit to indicate whether the data is segmented and whether it is the last segment

Observation 6: Solution 3 is more robust than solution 1 for segment delivery, with marginal price for additional signaling overhead.

Proposal 1: Capture solutions 1, 2 and 3 in the above observations in the TR as candidate solutions for A-IoT segmentation functionality.