3GPP TSG-RAN WG2 Meeting #128			R2-2410315

Orlando, USA, Nov. 18th-22nd

Agenda item:	8.2.4

Title:	Further discussion on ambient IoT random access

Source:	CMCC

Document for:	Discussion

Introduction

In the last RAN2 meeting, RAN2 discuss the e-mail discussion of random access, leaving the following agreements and FFS [1]:

Agreements for Random Access - In case of D2R data transmission failure, device follows the reader instruction: it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R). - Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose. FFS for following D2R data. - For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access. The re-access is always controlled by reader. - For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure. - R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message. FFS additional indications to differentiate if needed in paging message. FFS if other message can be used. - From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA. FFS if this is applicable to FDMA. Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

As it is the last meeting before SI endorsement, this contribution tries to propose technical solution that will not impact SI endorsement and accept by most companies.

Discussion

  - Functionality aspect

    - FDMA in slotted-ALOHA

For multiple access, TDMA and FDMA are both studied in RAN1. RAN2 agrees that device can random select access occasion at least in TDMA. But for FDMA, there are some arguments in RAN2 to wait for RAN1’s conclusion on whether device can also randomly select a frequency-domain access occasion. Those arguments suggest device may not have the same frequency shifting related capability.

After checking with our colleague in RAN1 and RAN1 chair notes, small frequency shifting is applied mainly with Manchester or Miller line codes instead of any hardware or RF component specific design. Hence it should be also possible for device 1/2a to randomly select frequency-domain access occasion. For large frequency shift, RAN1 is also not sure whether it can be applied for device 2a, but it is not feasible for at least device 1. Hence, all devices are capable of small frequency shifting.

Observation 1: Based on RAN1 agreement, small frequency shifting is applied mainly by Manchester or Miller line codes and feasible for devices 1/2a/2b. Large frequency shifting (e.g. tens of MHz) is not feasible for at least device 1.

The inventory time efficiency can be much improved with FDMA. Therefore, there is no harm to study it if time allows. But with the introduction of frequency shifting, as a start, RAN should discuss whether the shifting range is configured by reader or all devices share a unified range. As a start, RAN2 can assume that device 1/2a has the same maximum shift range for small frequency shifting. Then, maybe RAN2 can discuss whether reader should configure a shifting range of devices, in order to lower interference and better accommodate device population; or all devices should use a unified shifting range anytime when FDMA enables, for simplicity.

Obviously, there may also have impact on RAN1/RAN4 and still exists uncertainty. RAN2 can start with some feasible and simple approaches in the first release.

Proposal 1: Device can also randomly select access occasion in FDMA using small frequency shifter.

Some details, for example, whether and how to indicate center and shifting range can be discussed in WI.

    - Segmentation

One of the agreements on RAN2’s last meeting is to study the functionality of segmentation in MAC for Ambient IoT. This raises the possibility of impacts on random access procedures, specifically whether segmentation could be applied to random access. Additionally, RAN2 has agreed to prioritize focus on D2R communication, while R2D considerations may depend on input from SA2 and RAN1. Therefore, discussion on segmentation for Msg1 and Msg3 should be prioritized.

For Msg1 without data, i.e., Msg1 in the 3-step CBRA, its intention is to detect collision early with lower energy consumption than using a device ID. If Msg1 were segmented, the reader would need to wait until all segments are received to determine if there is a collision. Considering a device energy constraint case, device may need to recharge itself serval times before sending all segments. As a result, some timer or time relationship defined by RAN1 for random access may be impacted. Hence, it’s better to inform RAN1 if time allows.

Besides, the length of Msg1 without data should not be very long, it could just include a 16-bit random ID and maybe a very short subheader. Hence, it is not suggested to segment Msg1 without data. In contrast, Msg1 with data can be much longer than Msg1 without data, segmentation can be more beneficial. However, since contention has not yet been resolved, segmenting it in CBRA has the same challenges as segmenting Msg1 without data.

Regarding Msg1 with data and Msg3, segmenting can make it harder to define the boundaries of access occasion. For example, if device needs to recharge itself for certain time before sending all segmentations, should RAN1/2 expand the access occasion, or let device to re-access? It would leave with more open issues. Considering the SI endorsement, we suggest not to introduce segmentation for Ambient IoT random access.

Observation 2a: For Msg1, where contention is not resolved yet, segmenting Msg1 can make it difficult for reader to detect whether there is a collision or not.

Observation 2b: For Msg1 with data and Msg3, segmentation might be beneficial, but it will have impact on timer and timing relationship defined by RAN1, boundary of access occasion, and other device in the same inventory round, e.g., one occasion may be not enough to send all segmentations as device needs to recharge several times.

Hence, it would be better to postpone the discussion on segmentation in random access to next release where there would more solid conclusions on both segmentation and random access.

Proposal 2a: Msg1 without data should not be segmented.

Proposal 2b: Msg3 and Msg1 with data should not be segmented at least in Release-19.

Consider a case that device doesn’t have enough energy to send Msg1 and/or Msg3 but has energy to send some but not all segmentations, reader may trigger it to re-access and device may exhaust its energy in re-access and have no opportunity to recharge itself. Therefore, the device's behaviour when it determines it to segment Msg1 and Msg3 due to low energy should be specified by RAN2.

Given the device’s limited capability and complexity, the feasible solution is that device should not initiate an access procedure if it has to segment due to low energy. Moreover, low energy device should not attempt to re-access as well. Instead, it should wait for the next available access opportunity when segmentation is not necessary or it has enough energy.

Proposal 3: Device should not attempt to access or re-access if it has to segment Msg1 and/or Msg3 due to low energy.

For segmentation due to coverage, RAN2 can wait for RAN1 progress on whether there are any coverage specific parameters.

    - D2R message size indication

Based on the agreements of RAN2#127bis, device may be able to report D2R message size. This feature is introduced for segmentation from our perspective, because reader needs to know whether all segmentations are sent.

However, in previous RAN2 discussion, there do exist proposals that device can report uplink data size to be transmitted after contention resolution to reader, which is pretty similar to the legacy BSR procedure.

Considering there are only two use case, i.e., inventory and command, in this release, the data transmission for both use cases is either to read or write device’s NVM. Reader may already know the length and address to reader or write data in device by CN. Having device report data size seems like a sensor or monitor use case where device will buffer some data and report them to reader, which is outside the scope of current release.

Observation 3a: CN may provide message size for Command use case, and the demand to report D2R message size mainly comes from segmentation in undefined use cases, e.g., sensor and monitor, which are not supported in this release.

Regarding that segmentation happens in upper layer data transmission after random access or at least contention resolution and Msg1/Msg3 probably has a fixed size, the earliest opportunity to report D2R message size is anyway after random access, for example, maybe Msg5 in 3-step CBRA. Therefore, at least Msg1 and Msg3 doesn’t need to send message size indication.

Observation 3b: With message size indication, the size of Msg1 and Msg3 will be variable and device may need to store a buffer size table, which will make the low-cost device 1 more complex.

Proposal 4: D2R message size indication only applies after contention resolution, e.g. in CBRA Msg5. Msg1 and Msg3 will not report any D2R message size indication for simplicity.

RAN2 might consider utilizing any padding bits in Msg3 to capture additional assistance information in stage-2 or 3. However, for the first release, simplicity and robustness are the top priorities.

    - Device population estimation

There is an agreement captured in TR that the approximate number of target A-IoT device per service is useful in RAN. The number can be provided by CN if possible.

The following information are considered useful to be visible to the reader from CN: - The A-IoT service type (e.g., inventory, command) - Whether the service is targeted for one or more than one A-IoT devices - The approximate number of target A-IoT devices of this service (if available)

However, if CN cannot provide such information, reader may need to estimate the number of target device by itself.

And in some privacy concern use case, reader may also need to estimate device number by itself. For example, counting the number of people in a building, it’s impossible and maybe illegal to reader their ID by inventory, estimating their population can be the only option in this case. Hence device population estimation by reader should be studied and specified.

Proposal 5: RAN2 should specify how reader estimates the number of target device per service if it’s not provided by CN in stage-2.

As a start, in UHF RFID, EPC C1G2 uses a collision-based binomial algorithm, sometimes referred as Q-algorithm to estimate device population. Reader will adjust the Q value based on there is collision or not in a time slot to get a proper value of Q, typically, 2Q should be roughly equals to the number of target device.

There are many problems with Q-algorithm, for example, it’s not accurate, and in Ambient IoT, it will consume much device energy. The annex provides a both accurate and energy-efficient approach to estimate device population.

Reader can indicate the length of random number, target number and comparison operation to device. For every occasion, device generates a RN with length indicated by reader, then compares it to the target number. Only If the result matches the operation, device will send Msg1 without data. In this way, devices response in every access occasion independently with the target response probability.

By counting the number of empty occasions, successful occasions, collided occasions, the reader can calculate the estimated population based on the method in Annex A.

Proposal 6: RAN2 is suggested to capture Annex A in TR as a candidate solution for device population estimation, which introduces a dedicate R2D message containing length of RN, target number and operator.

  - Signaling aspect

    - As layer ID

There is an FFS on AS layer ID in RAN2#127, but this FFS is not discussed in RAN2#127bis.

- FFS if reader assigns the AS ID for scheduling purposes.

After checking with our colleague in RAN1, the ID associated with device(s), agreed in RAN1#118, is the not exactly the same AS layer ID which is under discussion in RAN2 now. The ID discussed in RAN1 is the device or group ID in the Ambient IoT paging message, which can be invisible to reader.

Agreement For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions FFS: other information FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

Observation 4: The ID associated with device(s) discussed in RAN1 is not the AS layer ID.

With the introduction of segmentation, an AS layer ID which is visible to reader becomes more essential. For example, in a read use case where device is energy constraint, reader may need to schedule device multiple times to read all data as device lacks enough energy to send all data in one message. Additionally, multiple devices may need to transmit their data simultaneously. In this case, a reader visible ID to identify each device is necessary for further resource allocation and data transmission.

Besides, in command use case including read and write, device or reader may need to ACK/NACK each other to indicate whether prior data transmission is success or not. In this case, the ACK/NACK should capture device-specific identifier to prevent confusion with other devices that may also be transmitting data.

An easy solution is to reuse device ID, but this can be less efficient, as device IDs are typically longer than AS layer IDs, reducing both time and energy efficiency. Moreover, device IDs may sometimes be invisible to reader considering its maybe NAS encrypted. The 16-bit random ID in Msg1 can be the baseline of AS layer ID, it shorter and much simpler.

Proposal 7a: RAN2 agrees to introduce an AS layer ID for resource allocation and ACK/NACK purpose, its baseline can be the 16-bit random ID in Msg1.

But the random ID generated by device may collides with other devices, i.e., multiple devices report the same random ID. Therefore, to obtain a unique AS layer ID, the device can either generate a new random ID or have a new AS layer ID assigned by the reader.

Proposal 7b: In case multiple devices report a same random ID, reader can either request device to generate and report a new ID, or assign an ID to device.

Reader to request device to report new ID is widely used in UHF RFID, e.g., Req_RN command, but reader to assign a new ID can be more efficient since the ID generated by device can still collides with other devices. We don’t have strong preference here, both options are fine with us.

    - Feedback indication

Let’s start with a fact is that the Msg2 in 2-step and 3-step RA is actually an explicit ACK message to Msg1. Therefore, it seems that there is no reason to prevent Msg2 from being used as an explicit ACK message in data transmission after contention resolution.

Observation 5: Msg2 including random ID serves a similar functionality as the explicit ACK message to Msg1.

And therefore, for simplicity, Msg2 can be reused as ACK message for Msg1, Msg3 and data transmission after random access.

Proposal 8: Msg2 including random ID can be potentially reused as ACK message for Msg3 and D2R data transmission after random access.

The “unique” AS layer ID is useful to indicate device a failure/success data transmission, especially when D2R data are segmented. For example, an explicit NACK would help device to stop send the remaining segments after reader fails to receive a prior segment. This would prevent device from wasting energy to send remaining segments, because anyway segment-based retransmission is not support, even retransmission is not supported yet.

The explicit ACK may be not necessary since the resource allocation indication/message can be regarded as an implicit ACK. The last segment of D2R data may be different.

Observation 6: Considering there is no segment-based retransmission, if reader fails to receive one segmentation of an D2R message, there is no need for device to send remaining segmentations.

Hence, it would be better if the ACK/NACK message for msg3 can be reused. We don’t have strong preference here, both ACK and NACK are fine with us.

Proposal 9: Introduce an explicit NACK message that can be potentially used as the D2R message reception failure indication for Msg3 and data transmission after contention resolution.

With introduction of both ACK and NACK message, RAN2 can further discuss whether to apply ACK/NACK message for Msg3 and upper layer data transmission, for example, whether the lack of indication implies an ACK or NACK, which could be a stage-3 issue.

Proposal 10: Whether to use ACK and NACK message for Msg1, Msg3, data transmission and other messages can be further discussed in Stage-3.

Please note that the “potentially” here mean ACK/NACK is not mandatory, it can also be indicated implicitly, as RAN2 agreed before.

    - RA Type

There is also a remaining FFS on RA type supported by device in RAN2#127, which is also not discussed in RAN2#127bis.

- FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.

Currently, RAN2 is discussing CFRA, as well as 2-step and 3-step CBRA. As a starting point, the demand for locating or finding a specific device is persistent; therefore, all devices should support CFRA.

In certain privacy-sensitive use cases, such as personal item tracking or locating, some devices may only support CFRA and paged by user itself, enabling them to be paged solely by the CN and not inventoried by unknown reader.

Observation 7: There is privacy concern in personal item tracking or locating. Some device may only response if reader already has its information, e.g., device ID.

Proposal 11: All devices should support CFRA by mandatory, some devices may only support CFRA for privacy.

For CBRA, the main difference of 2-step and 3-step is that 2-step CBRA enables quicker data transmission than 3-step, while the 3-step method allows for earlier collision detection. With regarding to the requirements from SA1 and inventory/command use case, the access delay requirements for A-IoT are relatively lenient.

Observation 8a: 2-step CBRA might send upper layer data faster than 3-step if no collision, however, there is no delay-sensitive use case for Ambient IoT identified so far.

Furthermore, if there are numerous devices that only support the 2-step CBRA are selected and the reader cannot allocate enough access occasions for them, i.e., collision is more common per access occasion, 2-step will consume more energy than 3-step CBRA per device. This is because 2-step CBRA will transmit both random ID and device ID in Msg1. Msg1 with data will consume more device energy, whereas the 3-step method only transmits the random ID. If collided, 3-step CBRA is more energy efficient. Additionally, it’s not sure whether reader can terminate D2R transmission at any point to avoid device sending remaining collided messages.

Observation 8b: In collision, 2-step CBRA will consume more device energy since devices has to send data. If not, 2-step and 3-step CBRA has basically the same energy consumption. 3-step CBRA is overall more energy efficient.

Our concern arises when devices can only support either the 2-step or 3-step CBRA in a mixed deployment. In such cases, the reader may need to initiate at least two rounds of inventory, one for 2-step device, the other for 3-step device, which is really not desirable.

Observation 8c: If device only supports 2-step or 3-step CBRA is mixed deployed, reader will have to initiates multiple access procedures per CBRA type.

And the majority view of RAN2 is not to introduce a device capability information. So, it is suggested for RAN2 to determine the mandatory CBRA type that devices must support.

Proposal 12: RAN2 should decide the CBRA type(s) that CBRA capable devices support by mandatory, can consider the following options,

Opt.1: Support only 3-step CBRA by mandatory.

Opt.2: Support both 2-step and 3-step CBRA by mandatory.

However, if it’s possible, RAN2 should down select between 2-step and 3-step CBRA. Honestly, 2-step is kind of a delay enhancement of 3-step CBRA which has no use case at least in this release. Supporting just one type of CBRA is beneficial for both a unified design and device simplicity.

Proposal 13: RAN2 is suggested to recommend only 3-step CBRA for normative work, and only support 3-step CBRA by mandatory in this release.

If not, RAN2 should focus on the reader behavior if 2-step and 3-step CBRA device is mixed deployed, i.e., to inventory twice or avoid this deployment.

    - Re-access indication

In RAN2#127bis, it is agreed to reuse paging message as re-access trigger message, but there are still some ambiguities in re-access procedures. For example, it is unclear how the device will recognize the need to re-access and how the trigger message can exclusively select devices that failed in the previous access attempt.

Currently, a NACK or the absence of ACK/subsequent message can be regarded as an access failure indication to inform device that this round of random access fails and it may need to re-access. but the failure indication doesn’t mean that device can start re-access immediately. Instead, device has to wait for re-access trigger message. Consequently, there will be a period that device will “remember” its failure status, which means device will stores a status or flag for re-access at least temporarily.

Observation 9: Device might have to store some information to distinguish initial access and re-access at least temporarily if it fails in last access attempt.

The status or flag for re-access, for our perspective, should be simple and short because device may need to store it in its NVM. AS a start, the information in paging can be reused:

Agreements on AIoT paging - The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase.

For simplicity, instead of introducing a new flag/indication, it would better to reuse existing information/indications.

Proposal 14: Upon random access failure, the short information to avoid duplicate response in paging message can be reused for re-access purpose, e.g., reader can use it to trigger re-access, failed device can use it to determine whether to re-access.

The details can be further discussed in stage-3 with regarding to the progress of paging and maybe other WGs input. And whether to introduce dedicated re-access trigger message can be discussed in Stage3.

Proposal 15: Whether to introduce dedicated R2D message for re-access can be discussed in Stage-3.

From our view, paging message might be enough.

Conclusion

This contribution discusses the remaining issue before SI endorsement and propose the following,

FDMA:

Observation 1: Based on RAN1 agreement, small frequency shifting is applied mainly by Manchester or Miller line codes and feasible for devices 1/2a/2b. Large frequency shifting (e.g. tens of MHz) is not feasible for at least device 1.

Proposal 1: Device can also randomly select access occasion in FDMA using small frequency shifter.

Segmentation:

Observation 2a: For Msg1, where contention is not resolved yet, segmenting Msg1 can make it difficult for reader to detect whether there is a collision or not.

Observation 2b: For Msg1 with data and Msg3, segmentation might be beneficial, but it will have impact on timer and timing relationship defined by RAN1, boundary of access occasion, and other device in the same inventory round, e.g., one occasion may be not enough to send all segmentations as device needs to recharge several times.

Proposal 2a: Msg1 without data should not be segmented.

Proposal 2b: Msg3 and Msg1 with data should not be segmented at least in Release-19.

Proposal 3: Device should not attempt to access or re-access if it has to segment Msg1 and/or Msg3 due to low energy.

Message size indication:

Observation 3a: CN may provide message size for Command use case, and the demand to report D2R message size mainly comes from segmentation in undefined use cases, e.g., sensor and monitor, which are not supported in this release.

Observation 3b: With message size indication, the size of Msg1 and Msg3 will be variable and device may need to store a buffer size table, which will make the low-cost device 1 more complex.

Proposal 4: D2R message size indication only applies after contention resolution, e.g. in CBRA Msg5. Msg1 and Msg3 will not report any D2R message size indication for simplicity.

Population estimation:

Proposal 5: RAN2 should specify how reader estimates the number of target device per service if it’s not provided by CN in stage-2.

Proposal 6: RAN2 is suggested to capture Annex A in TR as a candidate solution for device population estimation, where a dedicate R2D message containing length of RN, target number and operator is introduced.

As layer ID:

Observation 4: The ID associated with device(s) discussed in RAN1 is not the AS layer ID.

Proposal 7a: RAN2 agrees to introduce an AS layer ID for resource allocation and ACK/NACK purpose, its baseline can be the 16-bit random ID in Msg1.

Proposal 7b: In case multiple devices report a same random ID, reader can either request device to generate and report a new ID, or assign an ID to device.

ACK/NACK:

Observation 5: Msg2 including random ID serves a similar functionality as the explicit ACK message to Msg1.

Proposal 8: Msg2 including random ID can be potentially reused as ACK message for Msg3 and D2R data transmission after random access.

Observation 6: Considering there is no segment-based retransmission, if reader fails to receive one segmentation of an D2R message, there is no need for device to send remaining segmentations.

Proposal 9: Introduce an explicit NACK message that can be potentially used as the D2R message reception failure indication for Msg3 and data transmission after contention resolution.

Proposal 10: Whether to use ACK and NACK message for Msg1, Msg3, data transmission and other messages can be further discussed in Stage-3.

RA type:

Observation 7: There is privacy concern in personal item tracking or locating. Some device may only response if reader already has its information, e.g., device ID.

Proposal 11: All devices should support CFRA by mandatory, some devices may only support CFRA for privacy.

Observation 8a: 2-step CBRA might send upper layer data faster than 3-step if no collision, however, there is no delay-sensitive use case for Ambient IoT identified so far.

Observation 8b: In collision, 2-step CBRA will consume more device energy since devices has to send data. If not, 2-step and 3-step CBRA has basically the same energy consumption. 3-step CBRA is overall more energy efficient.

Observation 8c: If device only supports 2-step or 3-step CBRA is mixed deployed, reader will have to initiates multiple access procedures per CBRA type.

Proposal 12: RAN2 should decide the CBRA type(s) that CBRA capable devices support by mandatory, can consider the following options,

Opt.1: Support only 3-step CBRA by mandatory.

Opt.2: Support both 2-step and 3-step CBRA by mandatory.

Proposal 13: RAN2 is suggested to recommend only 3-step CBRA for normative work, and only support 3-step CBRA by mandatory in this release.

Re-access:

Observation 9: Device might have to store some information to distinguish initial access and re-access at least temporarily if it fails in last access attempt.

Proposal 14: Upon random access failure, the short information to avoid duplicate response in paging message can be reused for re-access purpose, e.g., reader can use it to trigger re-access, failed device can use it to determine whether to re-access.

Proposal 15: Whether to introduce dedicated R2D message for re-access can be discussed in Stage-3.

References

[1]	R2_127bis_ChairNotes_24-10-18_15-45_eom_clean.docx

Annex A (informative): Device population estimation method

Suppose there are x devices and m access occasion to estimate device population. Device can response in every access occasion independently with probability p, which can be done with a random number generator (RNG) used to randomly select access occasion. RNG should be able to generate uniformly distributed random number (RN). Reader can indicate the length of RN, target number and operator to device.

For every occasion, device generates a RN with length indicated by reader, then compares it to the target number. Only If the result matches the operator, device will send Msg1 without data. The following table is an example.

| Response probability | Length of random number | Target number | Operator |
| --- | --- | --- | --- |
| 0.125 | 3 | ‘000’ | ‘==’ |
| 0.25 | 2 | ‘00’ | ‘==’ |
| 0.375 | 3 | ‘010’ | ‘<=’ |
| 0.5 | 1 | ‘0’ | ‘==’ |
| 0.625 | 3 | ‘101’ | ‘<=’ |
| 0.75 | 2 | ‘00’ | ‘!=’ |
| 0.875 | 3 | ‘000’ | ‘!=’ |
| 2-k | k | ‘000…0’k zeros | ‘==’ |

Reader can count the number of empty occasions where no device responses, donated as e; successful occasions where only one device response, donated as s; and collided occasions where more than one device response, donated as c. Then we have,

1=p+q,  p>0, q>0

m=e+s+c,  e>0, c>0,s>0

And,

Pe=qx≈em

Ps=1x∙p∙qx-1=x∙p∙qx-1≈sm

Pc=i=2x1x∙pi∙qx-i≈cm

Hence,

x1=logemlogq

Where x1 is a rough estimation on device population. To get a more precise value, c can be used since it is negatively correlated with e.

fx=1-Pc=Pe+Ps=qx-1∙q+x∙p≈1-cm,  x∈N+

It can be proven that fx is constant decreasing in its domain (see Annex B), thus, a numerical result can be obtained using the c method. Let x2 be the numerical result obtained using the bisection method. The more accurate estimated value of x can be the mean of  x1 and  x2.

x=x1+x22

Reader typically uses small p and/or large m when there many devices, to get a more precise estimation value, reader can use a larger m.

Annex B (informative): Proof of monotonicity

Here is the proof of the monotonic decrease of fx in Annex A.

fx=Pe+Ps=qx-1∙q+x∙p,  x∈N+

Its differentiation,

f'x=qx-1∙lnq∙q+x∙p+p≤qx-1∙lnq+p,  x∈N+

And the sign of f'x is the same as,

gt=1-t+lnt,  t∈(0, 1)

Observing that,

limt→1-g(t)=0

And its differentiation,

g't=1t-1>0,  t∈(0, 1)

This completes the proof.