3GPP TSG-RAN WG2 Meeting #128			R2-2410317

Orlando, USA, Nov. 18th-22nd, 2024

Agenda item:	8.7.4.2

Title:	Further consideration on DSR enhancement

Source:	CMCC

Document for:	Discussion

Introduction

In the last meeting, RAN2 discusses DSR with multiple pairs of reporting values, and has the following agreements and FFS [1]

Agreements on DSR enhancements - We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

This contribution discusses some details of DSR enhancement.

Discussion

To facilitate future discussions and differentiate it from the DSR in Rel-18, we propose introducing the term "Refined DSR," similar to Refined BSR, to discuss the enhancements to DSR in Rel-19.

Proposal 1: For RAN2 discussion, the term “Refined DSR” is introduced for Rel-19 enhanced DSR with multiple pairs of remaining time and buffer size.

  - Triggering and reporting threshold(s)

As agreed in RAN2, dedicated triggering threshold and reporting threshold(s) will be introduced for Rel-19 DSR enhancement. But whether there will be any restrictions on their configuration is not clear yet.

For comparisons, if triggering threshold is smaller than the largest reporting threshold, non-delay-critical data can be reported via DSR. It’s a bit different from current DSR definition in 38.321, i.e., only delay-critical data will be reported.

More aggressively, if triggering threshold is smaller than any reporting threshold, DSR will report mostly non-delay-critical data. Although BSR can report both delay-critical and non-delay-critical data and we see no need to report non-delay-critical data in DSR, gNB should have such flexibility for further scheduling optimization with non-delay-critical data reporting. At least it is not necessary to restrict gNB or NW for more detailed information.

If triggering threshold is larger than reporting threshold, DSR will provide more detailed piecewise delay status and buffer size information, which is also beneficial for gNB scheduler optimization.

Observation 1a: Configuring triggering threshold lower than reporting threshold can potentially capture non-delay-critical data in DSR, while configuring triggering threshold higher than reporting threshold can provide information with finer granularity on delay status and buffer size.

Observation 1b: Information on non-delay-critical data and detailed information on delay-critical data can be helpful for gNB scheduler optimization.

Hence, gNB should still be capable of setting suitable threshold based on its implementation.

Proposal 2: The configuration of reporting threshold(s) and triggering threshold is up to NW, there is no constraint for that.

As for DSR cancellation, currently, pending DSR can be cancelled when all delay-critical SDUs are discarded or transmitted, or MAC CE itself is transmitted. Since Rel-19 DSR enhancement has no impact on the legacy triggering threshold, the triggering mechanism of Refined DSR should be aligned with Rel-18 legacy DSR, i.e., reporting threshold is has no impact on DSR triggering and cancellation, it only depends on whether DSR is sent and there is delay-critical SDU or not.

Proposal 3: Reporting threshold(s) has no impact on DSR triggering and cancellation, i.e., Refined DSR is triggered and cancelled as Rel-18 legacy DSR.

  - Segmented/Truncated Refined DSR

Another noteworthy issue is that with the introduction of multiple reporting thresholds, the length of the DSR will increase. Supposing that each LCG has 3 reporting thresholds and every portion has a non-zero buffer size, the size of Refined DSR MAC CE plus its subheader could reach up to 408 bits.

Sometimes allocated UL resource may be not enough for the complete Refined DSR. For example, if an SR is triggered by pending BSR, gNB, by implementation, will allocate resource enough for at least a BSR.

However, it might be impossible for gNB to know the size of DSR when there is a SR triggered by DSR.  If the UL grant size is less than the size of full Refined DSR, UE may send padding BSR, re-trigger a new SR and has to wait for next UL grant. Overall, without DSR, it may cause discarding for delay-critical data.

Observation 2a: With the introduction of multiple reporting thresholds, Refined DSR can be much longer than legacy DSR.

Observation 2b: In Rel-18, if UL grant cannot accommodate DSR, UE will not send DSR but re-trigger a new SR and wait for new UL grant.

Considering that padding DSR was not introduced in Rel-18 and Truncated DSR was not defined, RAN2 may need to discuss the UE behaviour when the MAC PDU cannot accommodate a complete Refined DSR.

The simplest option is to temporarily withhold the sending of pending DSR and wait for gNB to allocate a UL grant that is large enough to accommodate the complete pending Refined DSR. However, this option carries some uncertainty, as it is possible that gNB may never allocate sufficient resources, preventing the UE from ever sending the DSR.

Another approach is to introduce Truncated refined DSR. When the UL grant is insufficient to accommodate the complete Refined DSR, the truncated refined DSR can be used to send part of MAC CE. Subsequently, the UE can either cancel the pending DSR, similar with Truncated BSR, or keep the remaining pending DSR and continue sending the remaining pending DSR in the next UL grant. This method may require the introduction of segmentation indicators but would ensure that the gNB can at least receive some information from the DSR.

Proposal 4: RAN2 should specify UE behavior if UL grant cannot accommodate the complete Refined DSR, FFS whether it can apply to legacy DSR. May consider the following options and discuss other options:

Opt.1: As legacy, UE triggers a new SR and waits for new UL grant.

Opt.2: UE sends Truncated/Segmented Refined DSR, FFS whether to send the remaining Refined DSR segmentations.

We slightly prefer Opt.2 since Opt.1 will also cause discarding of delay-critical data.

  - Refined DSR MAC CE format

Similar with legacy DSR and BSR, only one DSR should be included in a MAC PDU for efficiency considerations. It would avoid the overhead caused by the subheader of the MAC CE. Additionally, multiple DSRs could potentially cause ambiguity in both UE and gNB MAC scheduler.

Proposal 5: As legacy, a MAC PDU contains at most one DSR MAC CE, refined or not.

With at most one DSR in a MAC PDU, RAN2 should focus on a unified DSR MAC CE format that can support UE to report delay status for LCGs configured with either Refined DSR or legacy DSR. Fortunately, the Rel-18 legacy DSR MAC CE can be compatible with both refined DSR and legacy DSR, i.e., new and old DSR, with only minor modifications.

Proposal 6a: To avoid multiple DSR MAC CEs in one MAC PDU, the Refined DSR MAC CE should be compatible with legacy DSR MAC CE, i.e. it can support LCG configured with single or multiple reporting threshold(s).

Within each pair of remaining time and buffer size from LCG, legacy DSR MAC CE has an R field as the reserved bit. This R field can be reused to indicate whether the consecutively distributed pairs of remaining time and buffer size belongs to the same LCG, i.e., whether the corresponding LCG is configured with multiple reporting threshold and the LCG has multiple pairs of nonzero value to report in this DSR.

Proposal 6b: Reuse the R field in legacy DSR MAC CE to indicate whether there are multiple pairs of remaining time and buffer size for the corresponding LCG.

The following is a proposed DSR MAC CE where the R field is replaced by the T field (stands for Threshold, final terminology can be further discussed) for multiple reporting threshold. The proposed DSR MAC CE can support both LCGs configured with Refined DSR and LCGs configured with Legacy DSR.

Figure 1: Proposed DSR MAC CE

The following is the description of the introduced T field:

- T: This field is present only if the corresponding LCG is configured with multipleReportingThresholds and the buffer size indicated by the corresponding reporting threshold of the corresponding LCG is not zero; otherwise, this field is reserved and set to 0. If present, the T field set to 1 indicates that the pair of remaining time and buffer size is the first or the only one of the corresponding LCG, while the BT field set to 0 indicates otherwise;

Another threshold related issue is the maximum number of reporting thresholds that can be configured for an LCG. Since the portion with a buffer size of zero no longer needs to be reported in the DSR, the DSR length can be effectively shortened. As a result, the limitation on the maximum number of reporting thresholds can be appropriately relaxed.

Based on the contribution of RAN2-127bis, there are proposals supporting 2, 3 and 4 reporting thresholds per LCG. As kind of a majority view, it is proposed that there will be at most 5 reporting thresholds for each LCG. Although the proposed DSR MAC CE can support a lot of reporting thresholds per LCG, configuring too many thresholds would be inappropriate due to the overhead of the DSR itself. RAN2 can further discuss to reduce the maximum number of reporting threshold per LCG.

Proposal 7: As a start, RAN2 agrees that there should be at most 5 reporting thresholds per LCG. RAN2 can further discuss whether to reduce the maximum number of reporting thresholds.

  - DSR Cancellation

With the introduction of multiple reporting threshold, gNB or NW may dynamically adjust reporting threshold(s) based on their implementation, e.g., RAN load. gNB may reconfigure UE MAC entity to get delay status with regarding to current radio resource.

Therefore, when DSR thresholds are updated, i.e., reporting or triggering threshold is changed, whether to send or cancel pending DSR will be a new issue and Rel-18 doesn’t specify it.

Observation 3: If NW reconfigures triggering or reporting threshold(s) of DSR, pending DSR can be obsolete and may not need to send it.

An obvious option is to just cancel all triggered delay status reporting procedure and discarding all pending DSR for the corresponding LCG as that in MAC reset. It will make sure that DSR configuration is effective immediately.

Proposal 8: UE should cancel pending Refined DSR if any of triggering or reporting threshold(s) is changed for the corresponding LCG. FFS whether it applies to legacy DSR.

Then, UE can trigger and generate new DSR with regarding to the new configuration.

  - UE capability

Considering that refined DSR is actually an enhancement of legacy DSR, and it will be also reported in UE capability. And some time gNB may just need very basic information reported in legacy DSR, it may only allow UE to report legacy DSR. Hence, UE should not only support refined DSR.

Proposal 9: If UE supports Refined DSR, it should also support Rel-18 legacy DSR by mandatory.

Conclusion

This contribution discusses some remaining issues on DSR enhancement, and propose the following,

Terminology:

Proposal 1: For RAN2 discussion, the term “Refined DSR” is introduced for Rel-19 enhanced DSR with multiple pairs of remaining time and buffer size.

Thresholds configuration:

Observation 1a: Configuring triggering threshold lower than reporting threshold can capture non-delay-critical data in DSR, while configuring triggering threshold higher than reporting threshold can provide more detailed information on delay status and buffer size.

Observation 1b: Information on non-delay-critical data and detailed information on delay-critical data can be helpful for gNB scheduler optimization.

Proposal 2: The configuration of reporting threshold(s) and triggering threshold is up to NW, there is no constraint for that.

Proposal 3: Reporting threshold(s) has no impact on DSR triggering and cancellation, i.e., Refined DSR is triggered and cancelled as Rel-18 legacy DSR.

Truncated Refined DSR:

Observation 2a: With the introduction of multiple reporting thresholds, Refined DSR can be much longer than legacy DSR.

Observation 2b: In Rel-18, if UL grant cannot accommodate DSR, UE will not send DSR but re-trigger a new SR and wait for new UL grant.

Proposal 4: RAN2 should specify UE behavior if UL grant cannot accommodate the complete Refined DSR, FFS whether it can apply to legacy DSR. May consider the following options and discuss other options:

Opt.1: As legacy, UE triggers a new SR and waits for new UL grant.

Opt.2: UE sends Truncated/Segmented Refined DSR, FFS whether to send the remaining Refined DSR segmentations.

Refined DSR MAC CE format:

Proposal 5: As legacy, a MAC PDU contains at most one DSR MAC CE, refined or not.

Proposal 6a: To avoid multiple DSR MAC CEs in one MAC PDU, the Refined DSR MAC CE should be compatible with legacy DSR MAC CE, i.e. it can support LCG configured with single or multiple reporting threshold(s).

Proposal 6b: Reuse the R field in legacy DSR MAC CE to indicate whether there are multiple pairs of remaining time and buffer size for the corresponding LCG.

Proposal 7: As a start, RAN2 agrees that there should be at most 5 reporting thresholds per LCG. RAN2 can further discuss whether to reduce the maximum number of reporting thresholds.

Refined DSR Cancellation:

Observation 3: If NW reconfigures triggering or reporting threshold(s) of DSR, pending DSR can be obsolete and may not need to send it.

Proposal 8: UE should cancel pending Refined DSR if any of triggering or reporting threshold(s) is changed for the corresponding LCG. FFS whether it applies to legacy DSR.

UE capability:

Proposal 9: If UE supports Refined DSR, it should also support Rel-18 legacy DSR by mandatory.

References

[1]	R2_127bis R18 MBS, R18 QoE and R19 XR session notes (Dawid)_EOM