3GPP TSG-RAN WG2 Meeting #128       	                      R2-2410343

Orlando, US, November 18th-22nd, 2024

Agenda item:	8.1.3

Source:	CMCC

Title:	Discussion on NW side data collection

WID/SID:	NR_AIML_air-Core

Document for:	Discussion

- Introduction

In RAN2#127 and RAN2#127bis meeting [1,2], RAN2 discussed the NW side data collection and achieved the following agreements.

RAN2#127 Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

RAN2#127bis Agreements

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

This contribution will discuss the remaining issues for NW side data collection.

- Discussion

- 2.1 NW side data collection for beam management

- Issue#1: Availability indication and event-triggered reporting

- In last meeting, RAN2 agreed that the UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. FFS trigger/definition of availability indication and FFS how data availability indication is sent to the network.

- In RAN2#127 meeting, RAN2 agreed that measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state. FFS whether the UE stops autonomously or if it reports to the network. In addition, RAN2 also agreed that when UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. Similarly, if the power state of the UE is low, we think that the UE can stop autonomously, but it needs to report to the network, otherwise, there is misunderstanding between UE and NW.

- Proposal 1: If the power state of the UE is low, the UE can stop measurement autonomously, which is similar to the case when the AS buffer is full.

- Based on the above agreement, we think the power state and AS buffer can be used to trigger availability indication. That is, when the power state is low or when the AS buffer is full, the UE can stop measurement and send the availability indication to the network.

- Proposal 2: Availability indication should be triggered upon the AS buffer is full or the power state of the UE is low.

- RAN2 has no consensus on event-triggered reporting. In our understanding, when the event is satisfied if event based reporting is supported, the UE will send BSR to the network for UL transmission scheduling. However, the network cannot be aware that the data is for training which is usually not urgent, and the transmission of logged data may cause congestion in the network. Furthermore, RAN2 has agreed to introduce the availability indication to indicate the availability of logged data for training, we think it is enough for the network to know the logging in UE side is stopped and determine when to request the data considering the current channel condition which can improve the flexibility of network scheduling.

- Therefore, we think it is no need to introduce event-triggered reporting, e.g. AS buffer, power state event based reporting.

- Proposal 3: Event based reporting (e.g. AS buffer, power state) is not supported.

- In legacy logged MDT, the UE can indicate the available measurements via UE-measurementAvailable [3] in msg5, e.g. RRCSetupComplete, RRCResumeComplete, RRCReestablishComplete message.

- For NW data collection in AI/ML, considering the UE is in connected mode, msg5 is not applicable for availability indication. We think UAI can be used to send the availability indication to the network.

- Proposal 4: The availability indication can be sent to the network via UEAssistanceInformation massage.

- Issue#2: FFS on multiple configurations and dynamic activation/deactivation

- In last meeting, RAN2 agreed that for the purpose of discussion of AI/ML BM LCM operations, the existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semi-persistent on PUCCH, semi-persistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1). In legacy CSI measurement/report mechanism, there can be multiple CSI configurations provided to the UE, and the UE would report multiple CSI reports to the network.

- For training data collection for beam management, we think the similar mechanism can be reused, i.e. there can be multiple measurement configurations provided to the UE.

- Proposal 5: There can be multiple measurement configurations provided to the UE.

- In current immediate MDT mechanism, the UE follows the RRM measurement behavior, i.e., the UE starts to collect measurement data upon reception of the measurement configuration. We think the motivation and gain of introducing dynamic activation/deactivation is unclear, and prefer that the UE starts the measurement immediately upon reception of the measurement configuration.

- Proposal 6: The UE starts the data collection immediately upon reception of the measurement configuration. The dynamic activation/deactivation is not supported.

- Issue#3: FFS on memory size

- RAN2 agreed that UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. In legacy logged MDT and QoE measurement, the minimum AS layer memory size is 64KB [4].

- TS 38.306, Section 4.2.18 Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The memory size of MDT logged measurements is 64KB. For eRedCap UE supporting this feature, the minimum memory size of MDT logged measurements is 16KB.; UE; No; No; No. - TS 38.306, Section 6 Features; Condition. AS layer memory size for QoE paused measurement reports; It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17.. AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE; It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports".

- For NW data collection, considering there are multiple use cases which needs training data, we think the legacy value is baseline and other larger value is preferred, i.e. the minimum AS layer memory size is at least 64KB.

- Proposal 7: The minimum AS layer memory size for training data logging is at least 64KB.

- Issue#4: FFS on logged time

- For temporal beam prediction, the time info is necessary for model training. In legacy logged MDT reporting, the absolute time info absoluteTimeStamp (48bits, as specified in [3]) when the logged measurement is provided and the relative time info relativeTimeStamp (as specified in [3]) which indicates the time of logging measurement results would be included in UEInformationResponse message. The similar principle can be reused for training data reporting to reduce the signaling overhead.

- Proposal 8: The absolute time stamp is used to indicate the time of report, and relative time stamp is used to indicate the time of logged measurement results, similar as logged MDT.

- 2.2 NW side data collection for Positioning

- 2.2.1 Positioning with gNB-sided model (case 3a)

- Figure 3. Positioning with gNB-sided model (case 3a)

- During the R18 study, the entity for gNB-sided model training can reside at gNB or OAM. Based on the following RAN1 agreement, the label and its related data (e.g., time stamp) for training can be generated by at least LMF. So the training data will be transmitted to gNB from LMF, and this has no impact on LPP.

RAN1#116bis Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope.

- Observation 1: For case 3a, the label and its related data for training can be generated by LMF then sent to gNB based on RAN1 agreement, which has no impact on LPP.

- Based on the following RAN1 agreement and conclusion, the measurements for determining model input are based on the existing SRS in R19 discussion, and the existing procedure can be reused in terms of SRS configuration for case 3a.

RAN1#116 Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. - Note: The use of SRS for MIMO resource is transparent to UE. RAN1#118 Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications

- Observation 2: The existing procedure of UL SRS configuration and measurement can be reused for case 3a based on RAN1 agreement.

- Based on observation 1 and 2, we think there is no additional impacts on Uu interface for case 3a for now. If further agreement is made in RAN1, we can revise it based on the RAN1’s progress.

- Proposal 9: For positioning with gNB-sided model (case 3a), there is no additional impact on Uu interface for now based on RAN1’s agreement. And we can revise it if RAN1 has further progress on this case.

- 2.2.2 Positioning with LMF-sided model (case 3b)

- During the R18 study, the entity for LMF-sided model training resides at LMF and the LMF is the termination point for training data. Therefore the LMF-centric data collection should be considered, i.e. LMF configures the UE/gNB to initiate/terminate the data collection procedure.

- Figure 4. Positioning with LMF-sided model (case 3b)

For the content of collected data for LMF-sided model training, at least measurements (corresponding to model input, e.g. timing, power, and/or phase info) and location coordinates as model output are needed based on RAN1 reply LS [5]. During the R19 discussion, RAN1 has achieved the following agreements on the collected data for LMF-sided model.

RAN1#116bis Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. RAN1#118 Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

- For training data generation of case 3b, the label and its related data can be generated by PRU or LMF or TRP/gNB. If the training data is generated by gNB and terminated at LMF, NRPPa will be used and should be discussed in RAN3. If the training data is generated by PRU or non-PRU UE and terminated at LMF, LPP will be used and in RAN2 scope. Therefore, RAN2 should focus on the latter case.

- Proposal 10: For positioning with LMF-side model (case 3b), RAN2 focus on the case where the training data is generated by PRU or non-PRU UE and terminated at LMF.

# 3	Conclusion

Here are the proposals for NW side data collection.

Beam management

- Proposal 1: If the power state of the UE is low, the UE can stop measurement autonomously, which is similar to the case when the AS buffer is full.

- Proposal 2: Availability indication can be triggered when the AS buffer is full or the power state of the UE is low.

- Proposal 3: Event based reporting (e.g. AS buffer, power state) is not supported.

- Proposal 4: The availability indication can be sent to the network via UEAssistanceInformation massage.

- Proposal 5: There can be multiple measurement configurations provided to the UE.

- Proposal 6: The UE starts the data collection immediately upon reception of the measurement configuration. The dynamic activation/deactivation is not supported.

- Proposal 7: The minimum AS layer memory size for training data logging is at least 64KB.

- Proposal 8: The absolute time stamp is used to indicate the time of report, and relative time stamp is used to indicate the time of logged measurement results, similar as logged MDT.

Positioning

- Observation 1: For case 3a, the label and its related data for training can be generated by LMF then sent to gNB based on RAN1 agreement, which has no impact on LPP.

- Observation 2: The existing procedure of UL SRS configuration and measurement can be reused for case 3a based on RAN1 agreement.

- Proposal 9: For positioning with gNB-sided model (case 3a), there is no additional impact on Uu interface for now based on RAN1’s agreement. And we can revise it if RAN1 has further progress on this case.

- Proposal 10: For positioning with LMF-side model (case 3b), RAN2 focus on the case where the training data is generated by PRU or non-PRU UE and terminated at LMF.

# 4	Reference

- Draft_R2_127_Meeting_Report_v1

- Draft_R2_127bis_Meeting_Report_v1

- TS 38.331 NR; Radio Resource Control (RRC) protocol specification

- TS 38.306 NR; User Equipment (UE) radio access capabilities

- R2-2311720 Reply LS on Data Collection Requirements and Assumptions (R1-2310681)