3GPP TSG-RAN WG2 Meeting #128									   R2-2410355

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.12.3

Source: 	NEC

Title:	Discussion on MAC CE impact of Rel-19 MIMO

Document for:	Discussion/Decision

- Introduction

NR MIMO Phase 5 introduces new MIMO features [1] for beam management, CSI, 3Tx, and asymmetric DL sTRP/UL mTRP. In this contribution, we provide our views on MAC impacts of the Rel-19 enhancements, in particular, on UE-initiated/event-driven beam management and asymmetric DL sTRP/UL mTRP scenarios.

- Discussion

  - UE initiated/event trigger beam management

UE initiated/event trigger beam management is targeting overhead/latency reduction. One issue may have RAN2 impact is the triggering event determination for Event 2. The same issue may hold for other events (e.g., Event 7) depending on the final decision on whether to support those events in Rel-19.

Agreement @ RAN1#117 [2] Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

As in this agreement, configurable time window/configurable number Mis similar as timer/counter based mechanism which is more common in RAN2 and usually the MAC entity is responsible for the management of timer/counter. For UEI beam report which is targeting overhead and latency reduction it is preferred to be done in physical layer enhancement, which could avoid the interaction between physical layer and the higher layer. While RAN1 plans to treat this issue in RAN1#119 meeting to decide whether RAN1 or RAN2 captures the agreements, RAN2 can only start the work after the decision is made. As said above, technically, it is preferred to capture the configurable time window and the configurable number M for event triggering directly in RAN1, therefore, we have the following proposal.

It is up to RAN1 to capture the agreement on triggering event determination for Event 2.

  - Asymmetric DL sTRP and UL mTRP scenarios

It was agreed in RAN1 that a new MAC CE would be introduced to update the PL offset value(s) for joint or UL TCI state(s), as follows.

Agreement @ RAN1#117 [2] For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

Further discussions are needed on the content of the MAC CE and the detailed mechanism for updating. There could be multiple options, for example, the MAC CE only provide one value, or multiple values, the MAC CE can update the PL offset values for all configured TCI states, or only activated TCI states, the value can be an absolute value, or an accumulative value, and so on. For simplicity, it is preferred that the MAC CE provide one accumulative value which is applied to the configured/applied PL offset values for all configured TCI states.

Support the PL offset value update MAC CE to provide one accumulative value which is applied to the configured/applied PL offset values for all configured TCI states.

Meanwhile, it could be NW implementation to determine the value of update, which may not be very accurate. If the value is too large, it may suggest that UE relative positions to two TRPs change a lot, e.g., the UE is moving much closer to the micro node or the macro node, compared to the positions when the PL offset value is configured originally. In this case, it is useful to trigger PHR report to let the NW know more timely information about UE transmit power if the PL offset variation (e.g., indicated in the PL offset value update MAC CE) is very large. Therefore, we have the following proposal.

PHR is triggered when the PL offset value update MAC CE provides a PL offset variation larger than a threshold.

- Conclusion

Based on the discussion in this document, we have the following observations and proposals:

Proposal 1:	It is up to RAN1 to capture the agreement on triggering event determination for Event 2.

Proposal 2:	Support the PL offset value update MAC CE to provide one accumulative value which is applied to the configured/applied PL offset values for all configured TCI states.

Proposal 3:	PHR is triggered when the PL offset value update MAC CE provides a PL offset variation larger than a threshold.

- Reference

- RP-242394, WID revision: NR MIMO Phase 5

- Chair’s Notes, 3GPP TSG RAN WG1 #117, Fukuoka City, Fukuoka, Japan, May 20th - 24th, 2024