3GPP TSG RAN WG2 Meeting#128	   							                                     R2-2410370

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	8.2.2

Source:	ETRI

Title:	A-IoT functionalities

Document for:	Discussion and Decision

# Introduction

This contribution discusses the functionalities of A-IoT, with a particular focus on L2 transmission, protocol stack, segmentation, and Information visible to the reader.

# Discussions

## L2 retransmission

In RAN2#126 meeting[2], RAN2 agreed not to support RLC-type retransmission. However, it does not mean RAN2 exclude the retransmission at A-IoT MAC level because there may be cases where receiver does not receive messages or data. Additionally, based on the RAN1 discussions, L1 repetition is primarily considered for transmission reliability, and there is openness to introducing higher-layer retransmission[7].

For R2D transmission, if reader transmits a message to device and does not receive response for a time period, the reader can re-transmit the message.

Let’s consider a case where a reader does not receive msg3 from a device after transmission of msg2 in 3-step CBRA procedure. If retransmission is considered, the reader transmits msg2 again to receive msg3 from the device. Otherwise, the reader re-initiates the 3-step CBRA by sending a paging message.

For D2R transmission, a device may encounter a situation similar to the R2D case described above. However, Rel-19 A-IoT only considers inventory and command services, which are primarily reader-initiated use cases. Furthermore, from a complexity perspective, as there may be cases where a device lacks memory, D2R retransmission offers little significant benefit relative to the added complexity.

In our view, A-IoT MAC level repetition is beneficial for reliable transmission but it is achieved by reader implementation.

Proposal 1: R2D retransmission can be up to reader implementation.

## Protocol stack

In RAN2#125bis meeting[1], the followings were agreed for A-IoT protocol stack.

1 RRC connection management is not supported. FFS how the resource configuration is provided to the device (if needed based on RAN1 progress) 2 RRM L3 measurement reporting is not supported by Ambient IoT devices. 3 RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding. 4 Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals. 5 RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

According to the above agreement, RAN2 does not consider RRC protocol between the reader and the device. However, there may be cases where control plane operation is needed for service trigger or initial access in in A-IoT network. For CP operation, even though control plane protocol can be considered on the top of A-IoT MAC, we think the A-IoT MAC layer can have functionalities for control plane operation. Therefore, a new AS protocol on the top of A-IoT MAC is not necessary for complexity of A-IoT device and compact protocol stack.

Proposal 2: No need to introduce new AS control protocol between the reader and the device in A-IoT.

## Segmentation

In last RAN2#127bis meeting[4], the followings were agreed for A-IoT segmentations.

Agreements on segmentations - RAN2 will study segmentation and RAN2 will need to give some guidance once there is more clarity on solution, on the minimum TB size. - Study the functionality of segmentation in the MAC. We focus on D2R and can consider the support for R2D later depending on SA2 and RAN1. The solution should aim to be applicable to both direction (i.e. unified solution), if we decide to support on both directions. - Do not support sequence number for A-IoT segmentation functionalities. - Do not support segment number and the number of segments for A-IoT segmentation functionalities. - Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment. FFS the details on how this is designed and number of bits (one or two). - RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

According to the agreements, a device only transmits information to the reader indicating whether the SDU is segmented and if the segment is the last one.

For example, a SDU is split into four different segments and which are then transmitted over D2R. Consider a case where segment #2 is not transmitted for some reason, while the others(i.e. segment#1, #3, and #4) are successfully delivered to the reader. In this situation, the reader may unable to determine if all segments have been successfully received, as each segment is transmitted without a segment number. In other words, the reader cannot detect that segment #2 is missing. Therefore, in D2R transmission, the reader needs confirmation that all segments of an SDU have been received without any missing parts. To address this, feedback indicating whether each segment has been successfully received from the device should be transmitted from the reader to the device upon each segment's reception. Based on the feedback, the device determines next segment to be transmitted. I n the example above, the device does not transmit segment #3 until it receives feedback for segment #2, ensuring in-order delivery without any missing segments.

Proposal 3: Feedback is necessary to ensure in-order delivery without any missing segment.

## Information visible to the reader

During the RAN2#127 meeting[3], the following agreement was made regarding information visibility issue/[1]. However, we have not concluded whether it is necessary to introduce additional information from the CN that would be visible to the reader.

At least the following information are considered useful to be visible to the reader from CN - The service type of A-IoT (e.g. inventory, command). FFS if more information on command type (e.g. read/write/disable) is useful - targeted for one or more than one devices; - approximate number of target devices (if available). FFS on mandatory/optional

For inventory services, the D2R message size is primarily determined by the length of the device ID. Therefore, the service type is essential for scheduling the next D2R message, even though EPC(Electronic Product Code) is the main consideration and may vary in length according to RFID standards.[6].

For command services, the D2R message size can vary depending on the command type. For example, with a 'read' command, the reader may either request the exact data size to be read from the device or allocate sufficient radio resources for the D2R message to accommodate the response. For commands like 'write' or 'disable,' the reader may expect a small amount of radio resources to transmit a short message, such as an ACK/NACK. Therefore, for radio resource scheduling of the next D2R message, it would be beneficial for the reader to identify the command type (e.g., read, write, or disable).

Proposal 4: Command type (e.g. read/write/disable) should be visible to the reader.

With regard to 'targeted for one or more devices,' we believe this information can help determine whether the service is intended for a single device or multiple devices. Assuming there are multiple target devices for a service requested from the CN, the reader can choose to either perform a single transmission using a multicast AS ID or conduct multiple transmissions using unicast AS IDs to deliver the information. Therefore, we believe that 'targeted for one or more devices' could be useful for AS layer transmission.

Regarding the agreement on 'approximate number of target devices (if available),' this information is useful for estimating the radio resources needed to accommodate the service(s). For inventory services based on CBRA, if the number of target devices is provided by the CN, the reader can configure appropriate radio resources for the random access procedure, taking into account a certain collision rate. However, this is primarily related to optimization.

Therefore, we believe that 'service type,' 'command type,' and 'targeted for one or more devices' could be mandatory information visible to the reader. In contrast, 'approximate number of target devices (if available)' may be considered optional.

Proposal 5: The ‘service type’, ‘command type’ and ‘targeted for one or more than one devices’ can be considered mandatory information to be visible to the reader from CN.

In TR38.848, A-IoT devices are classified into three types—Device A, Device B, and Device C—based on their energy storage capacity and ability to generate RF signals for transmissions. Since energy storage may impact AS procedures, identifying the device type could be beneficial for AS operations from the reader’s perspective. Therefore, RAN2 is kindly asked to discuss the device type as an additional information to be visible to the reader from CN.

Proposal 6: RAN2 to discuss the device type as an information visible to the reader from the CN.

# Conclusions

Based on the discussion in the contribution, we have the following proposals:

Proposal 1: R2D retransmission can be up to reader implementation.

Proposal 2: No need to introduce new AS control protocol between the reader and the device in A-IoT.

Proposal 3: Feedback is necessary to ensure in-order delivery without any missing segment.

Proposal 4: Command type (e.g. read/write/disable) should be visible to the reader.

Proposal 5: The ‘service type’, ‘command type’ and ‘targeted for one or more than one devices’ can be considered mandatory information to be visible to the reader from CN.

Proposal 6: RAN2 to discuss the device type as an information visible to the reader from the CN.

# References

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #125bis”, April 2024

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #126”, May 2024

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #127”, August 2024.

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #127bis”, October 2024.

- 3GPP TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR, Sep., 2024.

- EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID, 2013.

- Draft report of 3GPP TSG RAN WG1 #116, MCC Support, Athens, Greece February 26th - March 1st, 2024.