3GPP TSG RAN WG2 Meeting#128	      								                             R2-2410371

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	8.2.5

Source:	ETRI

Title:	Discussion on ‘temporary out of connection’ in Topology 2

Document for:	Discussion and Decision

# Introduction

In last RAN2 meeting, we reached an agreement on the basic resource configuration as follows.

Agreements - 1. Dedicated resource configuration is only given to the UE reader via dedicated signalling. Mechanisms for shared resource pool amongst readers are not considered in this release. - 2. The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control. FFS how valid resource is determined – we will consider temporary out of connection scenarios. FFS on resource validity across multiple cells. - 3. The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

This contribution discusses operations in cases where the intermediate UE is in a temporary out-of-connection.

# Discussions

## Indication of ‘temporary out of connection’

In our understanding, ‘temporary out of connection’ refers to a state where the intermediate UE temporarily has no network connection for some reason. This includes cases such as RLF (Radio Link Failure) or handover of the intermediate UE providing A-IoT services. In a temporary out-of-connection, the configured radio resources for an A-IoT service may not be utilized due to the potential for interference and interruption in data transmission between the intermediate UE and gNB.

In the case of a handover, this means that the intermediate UE moves to a different cell. In this scenario, we can consider two cases: one where the A-IoT device moves with the intermediate UE, and one where it does not. When the A-IoT device moves with the intermediate UE, causing a cell change, radio resource configuration for A-IoT services should be considered through information exchange between gNBs during the handover.

However, in Rel-19 A-IoT, there is no use case that considers the mobility of the device, so we can exclude a case where the A-IoT device moves along with the intermediate UE. Therefore, there is a need for discussion on how to handle ongoing A-IoT services when the intermediate UE moves to a different cell. When the intermediate UE moves to another cell, it may be replaced and the radio resources allocated to the device for a A-IoT service may be no longer be use. Consequently, it becomes necessary to release the radio resources for the A-IoT device after the handover. It is also essential to notify the device during or after the intermediate UE’s handover procedure that these radio resources are no longer valid.

In the case of RLF, the connection is temporarily released due to reasons such as out of synchronization, expiration of the TA timer, or excessive RLC retransmission. Thus, in this case, there is also a need for discussion on the operation of the ongoing A-IoT service. RLF indicates that a connection is temporarily failed, so the radio resources for the intermediate UE should not be used until re-establishment is completed. Therefore, in the case of RLF, the intermediate UE should inform the device that the radio resources configured for the A-IoT service are no longer valid.

Proposal 1: To ensure the validity of the allocated resources, the intermediate UE should notify the device when it enters a temporary out-of-connection.

Some companies have proposed using a timer to indicate how long the allocated radio resources remain valid. However, since handover and RLF events are unpredictable, this method cannot be used to indicate the ‘temporary out of connection’ of the intermediate UE.

Therefore, the most direct way to inform the device that the allocated radio resources are no longer valid is to send an explicit message to the device.

Proposal 2: The intermediate UE sends a message to the device indicating a ‘temporary out of connection’.

## Device behavior on reception of ‘temporary out of connection’

In case of ‘temporary out connection’ of intermediate UE, three possible operations can be considered for the ongoing A-IoT service: continuation, suspension and termination. For continuation, no action is needed. However, there should be an indication message for suspension and termination.

The message indicating ‘temporarily out of connection’ may contain different type of information such as termination and suspension. If the intermediate UE undergoes a handover, a new intermediate UE can be selected for the A-IoT service, so termination can be sent in the message. In contrast, if the intermediate UE experiences an RLF, the original UE can re-establish and resume the A-IoT service after re-establishment, so suspension can be included in the message.

A device that receives a message indicating ‘temporary out of connection’ will treat the radio resources as invalid and will either terminate or suspend D2R transmission based on the content of the message.

Proposal 3: A device that receives a message indicating ‘temporary out of connection’ from the intermediate UE will interpret the radio resources as no longer valid.

## Operations after ‘temporary out of connection’

Once the intermediate UE exits the 'temporary out of connection', the gNB can initiate procedures to resume the A-IoT service.

In the case of a handover, the gNB can select a new UE that can operate as the intermediate UE and resume the A-IoT service. In the case of an RLF, the original UE can be selected to resume service after re-establishment, or, if the RLF is not resolved, a new intermediate UE may be selected.

If the A-IoT service is resumed, the intermediate UE can notify the device to resume the A-IoT service.

Proposal 4: After a ‘temporary out-of-connection’, the gNB requires a mechanism to (re)select an intermediate UE and to resume the A-IoT service.

When resuming the A-IoT service, it is necessary for the intermediate UE to (re)configure radio resources. To accomplish this, the gNB can either reconfigure the radio resources on the original intermediate UE or allocate new radio resources. Therefore, radio resource configuration should be included in the resume message.

Proposal 5: For a resumption of A-IoT service, a resource (re)configuration should be included in resume message.

# Conclusions

Based on the discussion in the contribution, we have the following proposals:

Proposal 1: To ensure the validity of the allocated resources, the intermediate UE should notify the device when it enters a temporary out-of-connection.

Proposal 2: The intermediate UE sends a message to the device indicating a ‘temporary out of connection’.

Proposal 3: A device that receives a message indicating ‘temporary out of connection’ from the intermediate UE will interpret the radio resources as no longer valid.

Proposal 4: After a ‘temporary out-of-connection’, the gNB requires a mechanism to (re)select an intermediate UE and to resume the A-IoT service.

Proposal 5: For a resumption of A-IoT service, a resource (re)configuration should be included in resume message.

# References

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #127bis”, October 2024.