3GPP TSG RAN WG2 Meeting #128		R2-2410374

Orlando, U.S., 18th – 22nd November 2024

Agenda Item: 	8.2.2

Source:		Sony

Title:			Considerations on functionality aspects for Ambient IoT

Document for:	Discussion & Decision

# Introduction

At RAN#103 meeting, a revised version of the SID for Ambient IoT in NR was approved including the following objectives [1].

The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm,DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). - From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case. Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum. The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1. - RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential. - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. - RAN4-led: - Coexistence study of Ambient IoT and NR/LTE. - RF requirements study for Ambient IoT: - Ambient IoT BS transmission and reception - Ambient IoT Device, as per the General Scope, transmission and reception - Intermediate node (UE), as per the General Scope, transmission and reception RAN2 and RAN3 are expected to identify RAN-CN functional split in coordination with SA2. Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies. -

At RAN2#126 [2], the following agreements were made related to functionality aspects:

Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported.   FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported.  This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective.   FFS how we handle segmentation case (if needed)

At RAN2#127 [4], the following agreements were made;

Segmentation:

Agreements

-	Send an LS to RAN1 regarding TBS size.  Ask SA2 on the max and typical upper layer packet size.

Message Size/Status Indication

Agreements

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

-	RAN2 will study the need and use of a simple message “size” reporting to the reader

Visibility of AIoT Information

Agreements

- At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

2	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIoT interface as upper layer data.

At RAN2#127bis [6] the following agreements were made related to functionality aspects:

Agreements on AS ID RAN2 assumes that if “AS ID” is defined it is used at least for purpose of D2R scheduling and R2D reception. Up to RAN1 to decide whether a “AS ID” is defined. RAN2 assumes this “AS ID” should be a short AS layer ID, rather than the full upper layer device ID. FFS on the length. FFS if AS ID can be based on partial upper layer device ID. From RAN2 perspective, following options are possible for “AS ID”: - Option 1: a random ID if used in Msg1 can be reused; - Option 2: reader assigns this “AS ID”. FFS via which R2D message. RAN2 will aim to define one common design for all RA procedures (if technically possible)

Agreements on energy bit indication

- Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can.  1 bit indication can be captured in the TR.    We will capture the use case in the TR.   FFS whether it is reader controlled.

Agreements, Need of MSG Size

From RAN2 perspective, it is beneficial for the reader to know an estimate of expected D2R message size.  Two options can be captured: 1) from the CN and 2) from the device (simple message size indication).    Capture the advantages/disadvantages.

The D2R message size would be beneficial but it is not essential.

Ask SA2 if it is possible to provide the expected (e.g. approximate, estimate, exact, max) future response D2R message size.  Is it always available, sometimes, or never.

- Agreements on segmentations

- RAN2 will study segmentation and RAN2 will need to give some guidance once there is more clarity on solution, on the minimum TB size.

- Study the functionality of segmentation in the MAC.   We focus on D2R and can consider the support for R2D later depending on SA2 and RAN1.   The solution should aim to be applicable to both direction (i.e. unified solution), if we decide to support on both directions.

- Do not support sequence number for A-IoT segmentation functionalities.

- Do not support segment number and the number of segments for A-IoT segmentation functionalities.

- Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment.    FFS the details on how this is designed and number of bits (one or two).

- RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

- Fdf, DO-A Gap Analysis

From RAN2 perspective, at least the A-IoT paging is an aspect/part of the current design which is not sufficient for the DO-A use case.

From RAN2 perspective, DO-A use case cannot be supported with the current design discussed in the study item.

In this contribution, we discuss our view on some functionality related aspects such as status indication, segmentation and visibility of inventory/command information.

# Discussions on Functionality aspects

What functionality that is required for A-IoT devices are being discussed, and some agreements have been made on functionality that is not going to be supported, but some aspects need to be further discussed

At RAN2#127 it was agreed to study energy status report and to study the use of simple message size report to the reader.

It would be beneficial for the A-IoT device to be able to indicate that the currently stored energy is not sufficient to transmit the data available for transmission.

Based on this report a number of different methods or alternatives can be foreseen.

- a) indicate low energy,

- b) indicate low energy + more data/message size,

- c) indicate low energy/charge rate and suggest a segmentation and transmission scheme for remaining data.

## Energy indication

The simplest way forward is to let the A-IoT device be able to just indicate that it is low on energy. This could be done by using only one bit, and it would be up to the reader to interpret this indication, but one example would be that the A-IoT device may not be able to complete its transmission or that it may have to charge before commencing a subsequent transmission.

At RAN2#127bis, it was agreed to support this one bit indication. The potential issue or drawback with only one bit is that the device can only indicate two values, low energy or not low energy. By adding more bits also different levels of remaining energy could be indicated, as well as potential harvesting need. E.g., 2 bits could be used to indicate 4 energy levels, and then another 2 bits could be used to indicate remaining time for harvesting.

Proposal 1: Add a separate indication on remaining energy level and for energy harvesting.

## Message size

At RAN2#127bis, the following agreements were made related to message size:

Agreements, Need of MSG Size

From RAN2 perspective, it is beneficial for the reader to know an estimate of expected D2R message size.  Two options can be captured: 1) from the CN and 2) from the device (simple message size indication).    Capture the advantages/disadvantages.

The D2R message size would be beneficial but it is not essential.

Ask SA2 if it is possible to provide the expected (e.g. approximate, estimate, exact, max) future response D2R message size.  Is it always available, sometimes, or never.

It can be beneficial for the reader to know the D2R message size for resource allocation purpose. For option 1, as agreed, the message size can be known in the network and provided to the reader. This would be a rather straightforward solution and the reader can create the trigger message and allocate resources accordingly.

Observation 1: For option 1, the reader can create the trigger message and allocate resources accordingly.

For the second option, the message size indication should come from the device. The advantage of option 2 is if the D2R message size may vary, then the device may have the best knowledge of the size, but the disadvantage would be that the reader doesn’t know what size to expect and may not be able to allocate the proper number of resources.

For option 2, the conclusion may be that 1 step CFRA response message is not possible, and that only the 3 step CBRA procedure is needed to provide the data in message 3, after the device has indicated the message size in msg 1

Observation 2: The advantage of option 2 is if the D2R message size may vary, then the device may have the best knowledge of the size, but the disadvantage would be that the reader doesn’t know what size to expect and may not be able to allocate the proper number of resources.

If the A-IoT device gets low on energy, it may be beneficial for the reader to understand not only that the device is low on energy, but also how much remaining data is available. At least the UE could indicate that more data is available for transmission, but that this is not possible with the current energy level and that the additional data cannot be sent until charging has been done.

Proposal 2: The UE can indicate that there is a remining data and its amount when energy harvesting is needed.

## Segmentation and transmission scheme

The energy indication and message size issues discussed above can be reasons for the need for segmentation.

At the latest RAN2 meeting [6], the following agreements were made:

- Agreements on segmentations

- RAN2 will study segmentation and RAN2 will need to give some guidance once there is more clarity on solution, on the minimum TB size.

- Study the functionality of segmentation in the MAC.   We focus on D2R and can consider the support for R2D later depending on SA2 and RAN1.   The solution should aim to be applicable to both direction (i.e. unified solution), if we decide to support on both directions.

- Do not support sequence number for A-IoT segmentation functionalities.

- Do not support segment number and the number of segments for A-IoT segmentation functionalities.

- Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment.    FFS the details on how this is designed and number of bits (one or two).

- RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

As a further improvement or additional information, the device could be able to not only indicate remaining data, but also suggest a segmentation and/or transmission scheme.

The reason for segmentation could also be relevant in case the data to be transmitted is too large to fit in e.g., a TBS, transport Block size defined for msg 1 or msg 3 transmission, and hence a subsequent transmission may be needed.

RAN2 has sent an LS to RAN1 and SA2 [5] asking about the message size but irrespectively, segmentation should be possible for dividing the data into suitable packet sizes for transmission. RAN1 has responded in, [7]

From RAN1 perspective, a maximum TB size of around 1000 bits in PHY for R2D and D2R directions can be supported. How large TB that can be transported at a given time depends on target coverage/data rate, energy consumption/device availability, etc..From RAN1 perspective, there is no lower bound on the minimum TB size for R2D and D2R directions.

Segmentation could be used in case where:

- the device is low on energy, but still has enough energy to transmit at least a part of the data, or

- the device has limited energy storage, and the entire data information cannot be transmitted in one message.

- the size of the data is larger than the defined or configured TBS.

Observation 3: It has been agreed to support segmentation.

The issue may be how this should be indicated. E.g., if the number of segments should be indicated, and/or whether the current segment is the last segment or not.

The problem with only indicating that the current segment is the last segment, is that the Reader cannot know if a packet has been lost or not.

Observation 4: By only indicating last segment, the Reader cannot know if a packet has been lost of not.

Another aspect is energy harvesting. If the reason for segmentation is the need for energy harvesting, some sort of energy harvesting scheme indication may be beneficial. This can be done or discussed related to Energy bit indication.

Observation 5: If the reason for segmentation is the need for energy harvesting, some sort of energy harvesting scheme can be beneficial.

Proposal 5: Support segmentation and energy charging scheme for the remaining segmented data.

# Summary

In this contribution, we have discussed our views on aspects related to, random access, procedures.

Proposal 1: Add a separate indication on remaining energy level and for energy harvesting.

Observation 1: For option 1, the reader can create the trigger message and allocate resources accordingly.

Observation 2: The advantage of option 2 is if the D2R message size may vary, then the device may have the best knowledge of the size, but the disadvantage would be that the reader doesn’t know what size to expect and may not be able to allocate the proper number of resources.

Proposal 2: The UE can indicate that there is a remining data and its amount when energy harvesting is needed

Observation 3: It has been agreed to support segmentation.

Observation 4: By only indicating last segment, the Reader cannot know if a packet has been lost of not.

Observation 5: If the reason for segmentation is the need for energy harvesting, some sort of energy harvesting scheme can be beneficial.

Proposal 5: Support segmentation and energy charging scheme for the remaining segmented data.

# References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

- Minutes RAN2#126

- 23.700-13-v040

- RAN2#127

- LS to SA2 and RAN1

- RAN2#127bis

- R1-2409250, Reply LS to RAN2 on data block sizes for Ambient IoT