3GPP TSG-RAN WG2 Meeting #128  	                            R2-2410380

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.5.2

Source: 	Sony

Title:	On-demand SSB Scell operation discussion

Document for:	Discussion

# Introduction

In RAN2#127bis meeting, on-demand SSB Scell operation was discussed, and the agreements are as follows.

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this contribution, we discuss some open issues regarding the OD-SSB deactivation, and measurements.

# Discussion

2.1 Cell on/off indication via backhaul

RAN1#116 agreed to support on-demand SSB SCell operation triggered by gNB and cell on/off indication via backhaul is one of the candidate solutions. With the existing CU-DU architecture, it is the CU who is responsible for the cell on/off decision. In gNB-CU configuration update message sent from gNB-CU to gNB-DU, it contains cells to be activated/deactivated list which gNB-DU shall activate/deactivate the corresponding cells as indicated by the cell group and physical cell identity.

With the existing signalling, it is not possible to support dynamic cell on/off use cases where the cell on/off decision is made based on the current traffic and energy consumption status of the cell. It is the gNB-DU who is aware of the up-to-date status of the cells but the cell on/off indication comes from gNB-CU.

Cell on/off indication via backhaul will be a promising solution if it can be enhanced to support:

- Assistance information from gNB-DU to gNB-CU to help gNB-CU making cell on/off decision. We think this information is needed because gNB-DU has more awareness and gNB-DU should be involved in cell on/off decision as explained below.

- Certain level of autonomy is given to gNB-DU to allow it make its own cell on/off decision according to the cell status. The gNB-DU has the up-to-date information e.g., number of active UEs, radio resource utilization ratio, traffic status of the cell. Therefore, if certain level of autonomy is given to gNB-DU, it can make proper cell on/off decision based on the above cell status. But gNB-CU should always control this procedure and being informed of any gNB-DU’s decision.

RAN3 should lead the work and it may have some impacts on the assistance information which can be potentially supported by the UEs in the cell.

Proposal 1: Enhancements to support dynamic cell on/off indication via backhaul should be supported and RAN3 should be informed about it.

2.2 OD-SSB deactivation signalling

In RAN1#118bis, deactivation of OD-SSB transmission was discussed and RAN2 should discuss the options that were listed by RAN1.

Agreement - For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options

Option 1 and Option 1A are to support explicit deactivation via MAC-CE and RRC respectively. It was agreed in RAN2 that RRC can indicate OD-SSB transmission activation/deactivation state and RAN2 will introduce a new MAC CE to indicate the OD-SSB activation/deactivation. So both Option 1 and Option 1A are supported from RAN2’s point of view.

Option 2 and Option 3 are to define a time limit, in the form of number of SSB bursts or duration, so that the OD-SSB transmission is deactivated implicitly after the time limit. With these options, the signalling overhead on deactivating OD-SSB can be reduced. The RRC, for example, can configure the time limit. The specific time limit should be discussed in RAN1.

Option 4 and Option 4A are to couple the OD-SSB deactivation with SCell deactivation and deactivate the OD-SSB implicitly. Whereas in RAN2, it was agreed not to introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication. It is the understanding that activation/deactivation of OD-SSB transmission and Scell are decoupled, therefore Option 4 and Option 4A should be excluded from the list.

Option 5 may need more clarification. Our understanding of Option 5 is based on the assumption that OD-SSB is for activation of SCell only, so after SCell activation is completed, the OD-SSB should be deactivated implicitly. RAN2 is discussing the issue of how to perform OD-SSB measurement, so it requires more time on discussing if this option is feasible or not.

Option 6 needs to be discussed in RAN1.

Proposal 2: At least Option 1 and Option 1A should be supported to deactivate the OD-SSB transmission. Option 4 and Option 4A should be excluded.

Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication

2.3 OD-SSB measurement

In RAN2#127bis meeting, the L3 RRM framework on OD-SSB measurement has been discussed and two options are listed as follows.

Option1: Based on different measurement configuration when OD-SSB is transmitted.

Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted.

Our understanding is that Option 1 is more aligned with the legacy measurement framework while Option 2 may introduce more specification work. Furthermore, the SMTC configurations in Option 1 will need more RAN1 input on the detailed OD-SSB pattern discussion. On the other hand, if the OD-SSB pattern is shared with UE, it will be beneficial to both options, e.g., the OD-SSB pattern will be helpful for UE to choose from the configured multiple SMTC configurations in Option 1 and OD-SSB pattern sharing is essential in Option 2.

Proposal 3: OD-SSB patterns are informed to UE from network.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: Enhancements to support dynamic cell on/off indication via backhaul should be supported and RAN3 should be informed about it.

Proposal 2: At least Option 1 and Option 1A should be supported to deactivate the OD-SSB transmission. Option 4 and Option 4A should be excluded.

Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication

Proposal 3: OD-SSB patterns are informed to UE from network.