3GPP TSG-RAN WG2 Meeting #128  	                            R2-2410381

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.5.3

Source: 	Sony

Title:	WUS configuration details for on-demand SIB1

Document for:	Discussion

# Introduction

In RAN2#127bis meeting, the agreements on OD-SIB1 were agreed as follows:

Agreements on OD-SIB1

- We will inherit all agreements made during SI phase to WI phase.

- A NES cell can include neighbouring NES cell’s WUS configuration.

- NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

- In on-demand SIB1 procedure, the UE considers RACH failure when PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1.

- The MAC layer will indicate the RACH failure for SI request to upper layers. FFS: after that the UE upper layer will consider the cell as barred.

- The legacy UE behaviour can be reused upon on-demand SIB1 acquisition failure, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced.

- NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

- RAN2 will not start the discussion on the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, unless we’re asked to do that by other WG, e.g. RAN3.

In this contribution, we discuss WUS configuration aspects.

# Discussion

2.1 WUS configuration details

It was agreed that in RAN2#127bis meeting that a NES cell can include neighbouring NES cell’s WUS configuration and a NES cell’s WUS configuration is included in a new SIB (including its own WUS configuration).

For the UE connected to any cell i.e. NES cell or Cell A, it can receive the WUS configuration in RRC release message. On the other hand, as the Cell A may also provide the UL WUS configuration for the same NES cell, the mismatched configuration between Cell A and NES cell may not be an issue as dedicated configuration takes priority.

Proposal 1: Network can include WUS config in RRC Release message and this configuration takes priority over WUS configuration received from system information.

In RAN1#116 meeting, it was agreed that for study of UL WUS design, consider at least PRACH as a starting point. In Rel-15, on-demand SIB transmission is supported via IDLE/INACITVE UE to trigger a random access procedure. In MSG1 solution, a subset of the PRACH resources is associated with the requested SI, and network will reply a MAC subPDU with RAPID only for acknowledgement.

It is important that network can identify the request to transmit on-demand SIB1 from very beginning, in order to reduce the access delay for UE. And it was agreed in RAN2#125bis that UL WUS configuration includes at least RACH configuration. Therefore, the network should reserve PRACH resources that is associated with on-demand SIB1 transmission request.

Proposal 2: The network reserves PRACH resources associated with on-demand SIB1 transmission request. UE transmits UL WUS on the reserved PRACH resources to request the on-demand SIB1 transmission.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: Network can include WUS config in RRC Release message and this configuration takes priority over WUS configuration received from system information.

Proposal 2: The network reserves PRACH resources associated with on-demand SIB1 transmission request. UE transmits UL WUS on the reserved PRACH resources to request the on-demand SIB1 transmission.