3GPP TSG-RAN WG2 Meeting #128	R2-2410404

Orlando, USA, 18 – 22 November 2024	R2-2408698

Agenda item:	8.2.3

Source:	Nokia

Title:	Paging aspects of AIoT

WID/SID:	FS_Ambient_IoT_solutions - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we further discuss the considerations on paging for Ambient IoT and make some observations and proposals focusing on Topology 1.

In RAN2 #126, the following agreements were made related to Ambient IoT paging.

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

The following agreements were also made in RAN2 #126 with respect to Stage 2 general aspects of Ambient IoT.

Agreements 1 As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS 2 As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

In RAN2 #127, the following agreements were made related to Ambient IoT paging.

Agreements:

1	Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

In RAN2 #127bis, the following agreements were made related to Ambient IoT paging.

Agreements on AIoT paging - The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase. - Based on this information the device determines whether to skip sending the response to paging. - From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs. - If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA - For all use cases, device determines the random access type from the paging message. FFS whether it is explicitly or implicitly. FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

In this contribution, we further discuss the considerations on paging for Ambient IoT and make some observations and proposals.

# 2	Discussion

## 2.1	Support of “command only” procedure

There are no agreements yet on the steps associated with the procedure for the “command only” use case other than that RAN2 will study this use case. However, our understanding is that in this case, the procedure inherently does not include associated with “inventory”. Therefore, in principle, the base station may directly send a command and/or data to one or more devices in the initial trigger message.

It is clear from the agreements that the “inventory and command” procedure can be distinguished from the “inventory only” procedure through adding the steps associated with sending a command or data to the device(s) and subsequently receiving data back from the devices. For a “command only” service request, there may not be a need for separate steps associated with the “inventory” procedure. In other words, in this use case the network may want to directly send commands to devices without performing an inventory. For example, devices in this use case may be stationary or always within range of the base station. In other cases, it could be up to the base station or reader to first perform an inventory. Therefore, it is not always necessary to have separate steps for the purpose of obtaining device ID(s).

Observation 1: It is beneficial to support the “command only” procedure without including the steps associated with device ID delivery during “inventory” / “inventory & command” procedure.

In the “inventory only” and “inventory and command” use cases, the AIoT paging message triggers devices to respond with device ID. Thus, although control information related to the inventory procedure (e.g., random access parameters) may be included, there is no command or other data sent in the AIoT paging message in these use cases. On the other hand, for the “command only” procedure, the inventory procedure may be bypassed as discussed above, and the network may adopt a one-step approach for communication with AIoT devices. In this approach, both the device/group ID indication and the indication of the information related to data communication (e.g., “command”, DT data) can be provided in the initial trigger message.

However, including the DT traffic data in the AIoT paging message may lead to a failure to deliver the data if the device is not reachable. Moreover, delivering the data directly in the AIoT paging message may lead to some security issues and it may be necessary for SA3 to analyze the security issues associated with this approach. For example, sending a command or other data to an AIoT device may require prior authentication of the AIoT device by the network and exchange of security keys between the network and the AIoT device, where the security keys are necessary for decoding the command or data targeting the device. This may also mean that the AIoT device must store the security keys. As such, it is necessary to wait for SA3 inputs on the support of “command only” procedure without an inventory step or sending a command or data directly in the AIoT paging message.

Proposal 1: RAN2 to wait for SA3 inputs for considering inclusion of the DT information such as data and/or command in the AIoT paging message.

## 2.2	Indication of device ID(s)

One issue that is still open is, whether/how to indicate that the paging is addressing all devices with no restriction. From a unified design point of view, we believe that the simplest approach would be to always include the ID, but setting is to 0 in case all devices are addressed. Although this may cost bits, it will allow for easier decoding at device side.

Proposal 2: When an AIoT paging addresses all devices, the group/device ID is still contained in the paging message but set to 0.

The use of full-length device identifiers in AIoT paging messages may cost a high overhead, especially when listing a large number of identifiers in the paging message. Therefore, it would be beneficial to consider short identifiers for devices. Two options for how short identifiers are assigned to devices are: (a) the short identifiers are generated by the devices and shared with the reader (b) the short identifiers are generated by the reader and assigned to devices.  and details of how they can be used can be studied. Basing the short identifier on the full identifier would present a security/privacy and as such should be avoided.

Regarding RAN2 assuming a permanent ID, even though this will be different from the full ID, we believe that such an agreement may cause both privacy and security risk, and it should not be agreed without consulting SA3.

Observation 2: A permanent ID is not suitable for use as it provides privacy and security risk.

Proposal 3: RAN2 not to study permanent ID without consulting SA3.

Proposal 4: RAN2 to support the use of short device IDs for future access related procedures. FFS whether the ID is allocated by either the reader or device. FFS whether it is the short random ID.

Proposal 5: The short ID should not be based on the full ID due to security/privacy risk.

Proposal 6: The paging message needs to contain at least an ID type field indicating the type of identification.

## 2.3	Retransmission control

In order to improve resource efficiency, it is useful to consider preventing devices that have previously successfully responded to an AIoT paging message for an inventory or other service request from responding to a retransmission of the paging message for the same service request. Therefore, from a practical point of view, it may be also beneficial to provide an indication in an AIoT paging (re)transmission message related to the devices excluded from the list of paged devices which are not / no more expected to respond (e.g., since their response was successfully received in the past or is of lower priority) Additionally, methods for indication to trigger only those devices  whose responses were not received in the past can also be studied. Identifying devices through the access occasions previously used by them would reduce the signaling overhead.

Proposal 7: RAN2 to discuss whether to an AIoT paging message may also provide an indication to identify

- AIoT devices that are excluded from responding to a given AIoT paging request, i.e. devices that shall ignore the AIoT paging message.

- AIoT devices that shall respond to the AIoT paging message.

FFS whether there is a need for mapping to previous access occasions.

Proposal 8: The paging message may indicate a duration in which the device is excluded from responding e.g. a number of access occasions, number of paging occasions, or a time reference points.

As discussed above, a paging request may be retransmitted. It is further observed that a paging request may be reconfigured. For example, the targeted devices may be changed (e.g. based on priority if the response time is protracted due large-scale contention implying long / frequent backoff) or MAC parameters may be updated (e.g., backoff time is shortened if contention is low).

To this end, it would be beneficial to uniquely identify individual paging requests, e.g. by using a paging ID or session ID that is preferably unique over some time. Alternatively, single-bit indication of a new paging occasion may be used, similarly to New Data Indication.

Paging request identification is also needed when multiple paging requests occur simultaneously (be it by originating from the same reader, or multiple independent readers).

Proposal 9: RAN2 to agree that individual paging requests are identified using a session ID for retransmission or reconfiguration purposes as well as to permit the execution of multiple parallel paging requests. FFS whether multi-bit identifiers (e.g., random ID or sequence number) or single-bit flags (e.g., similarly to NDI) are used.

Alternatively, a paging request may be terminated if a sufficient number of devices have responded, or the available resources will become unavailable (e.g., the need to reserve them for SL communications). Also, massive contention and contention may result in excessive response times. If an upper-layer deadline elapses, there is no need to further collect and forward data from AIoT devices.

Proposal 10: RAN2 to study how to terminate ongoing responses from AIoT devices to a given paging request.

## 2.4	Multiple parallel sessions

While an AIoT device is engaged in a session associated with a service request, it is conceivable that the AIoTF may trigger another service request involving the same device. In this case, the device would need to be simultaneously engaged in multiple parallel sessions. For an AIoT device to be able to support this capability, the device complexity would be increased. Applications for which AIoT devices are used are delay tolerant. As such, in our view, there would not be a use case in which a service request for an AIoT device cannot wait until the device completes the procedure associated with an ongoing service request, even if that procedure involves a large number of devices and hence is expected to take a relatively long time. Furthermore, the AIoTF can avoid triggering such parallel sessions.

Proposal 11: RAN2 to agree that an AIoT device that is engaged in a procedure associated with a certain session ID is not required to respond to an AIoT paging message with a different session ID.

## 2.5	Multiple reader scenario

Another scenario for consideration is one where the AIoT device is within the range of multiple readers.  In our view, this may be a common scenario, especially in Topology 2, where readers may not always be deployed in a carefully planned manner. In such scenarios, an AIoT device may receive AIoT paging messages associated with the same service request (i.e., with the same session ID) from multiple readers. In this case, the device behavior could be similar to that when multiple paging (re)transmissions are received from the same reader. That is, it may not be relevant to the device to know whether the AIoT paging messages are received from the same reader or different readers as long as the session ID is included in the AIoT paging messages. On the other hand, the device may need to distinguish between a paging retransmission from one reader and the first paging message from another reader with the same session ID since the device may need to respond in the former case (in the absence of an acknowledgment to its past response) and not the latter case. Therefore, it is necessary to further study whether reader ID should be included in the AIoT paging messages.

Observation 3: Deployment of readers cannot always be planned to avoid the possibility that an AIoT device is within range of multiple readers, especially in Topology 2.

Proposal 12: RAN2 to further study whether it is necessary to include reader ID in the AIoT paging message to support the scenario where the AIoT device receives AIoT paging messages from multiple readers with the same session ID.

## 2.6	Support of access types

For AIoT paging of a single device, there is no need for consideration of contention-based random access (CBRA) since access resource can be granted to the device and hence contention-free access (CFA) can be the only type of access supported in this case. Likewise, for AIoT paging message targeting a device group (consisting of an arbitrary number of devices) or all devices, CFA is not an option and hence CBRA can be the only type of access in this case. However, both 2-step CBRA and 3-step CBRA are options. Therefore, if both these options are supported, the type of access needs to be indicated to the AIoT device in order for the device to correctly complete the procedure.  It was agreed in RAN2 #127bis that, if AIoT paging with indication of multiple device IDs is supported, either contention-free access or CBRA can be configured. In our view, however, we do not see a strong benefit with supporting CBRA when targeting multiple device IDs. Therefore, only CFA can be supported in this case. Furthermore, allocation of resources or access occasions to devices can be implicit and hence no additional signaling in the AIoT paging message is necessary. For example, resources or access occasions may be sequentially indexed and these are assigned to devices in the order in which the device IDs are listed in the AIoT paging message.

Similarly, service type may need to be also indicate the procedure type and length (e.g., number of subsequent steps until service termination). For example, the “command” scenario requires more steps compared to pure “inventory” use case.

Proposal 13: To simplify signaling, RAN2 to agrees that

- an AIoT paging of a single-device means “Contention-free access”,

- an AIoT paging of a device group or all devices means “Contention-based random access”.

- An AIoT paging of multiple device IDs (if supported) means “Contention-free access”.

Proposal 14: RAN2 to agree that, if AIoT paging of multiple device IDs in a single paging message is supported, assignment of resources or access occasions to device IDs for contention-free access is implicit.

Proposal 15: RAN2 to agree that an AIoT paging message contains at least the following

- access type, in case of AIoT paging message targeting a device group or all devices supporting multiple access types (“2-step CBRA”, “3-step CBRA)

- service type (FFS options “Inventory only”, “Command only”, “Inventory & command”)

## 2.7	Indication of access parameters

Dynamic configuration updates could be sent in the AIoT paging message to one or more devices. The information sent may comprise the full configuration or only the part of the configuration that has changed (differential or “delta” information). The information can include parameters for device access in response to inventory or “read” command requests. In case of paging multiple devices, parameters for contention-based random access (e.g., similar to a Q-parameter in the RFID standard or maximum number of access occasions) can be provided. Alternatively, when a single device is paged for a “read” command service request (as part of “command only” procedure), scheduling parameters for resource allocation with contention-free access can be provided. If dynamic configuration update through the paging procedure is supported, further study is needed on the frequency of such updates.

Proposal 16: AIoT paging message includes parameters indicating resources for contention-free access and contention-based random access (e.g., Q-parameter or maximum number of access occasions).

## 2.8	AIoT Device Availability

To ensure the availability of the AIoT device during the transmission and reception procedures, the following agreement was made during the RAN#103 meeting in RP-240854 (Section 8):

- The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study [RAN2, RAN1] - Duration of one device’s unavailability due to charging by energy harvesting can be assumed up to several tens of seconds - Note: this value can be revisited in future RAN plenary meetings, if necessary

In general, an AIoT device may experience failure during the random-access procedure due to their need to harvest energy between transmissions. In particular, AIoT devices may not harvest enough energy between two successive transmissions of the D2R access messages, or before the start and the completion of access (whether contention-based or contention-free) procedures. Not having enough energy for completing the access procedure may result in unexpected behaviour and can cause radio link failures.

In addition, AIoT devices (e.g., device 1 and device 2a) may not have the capability to send their D2R transmissions with constant transmission power which means that the maximum transmission power they could achieve at any given moment could vary. Backscattering the D2R signals with variable transmission power may negatively affect the access procedure for A-IoT devices. For example, AIoT devices may not have sufficient power to backscatter Msg1 with successive increases in power. Failure to complete a successful access procedure not only impacts the network performance but also creates unnecessary collisions and spectral inefficiency. It may also lead to excessive drain of stored energy at A-IoT devices.

Proposal 17: RAN2 to study how AIoT devices can successfully complete the access procedure under the constraint of device energy availability.

# 3	Conclusion

This paper has made the following observations and proposals:

Observation 1: It is beneficial to support the “command only” procedure without including the steps associated with device ID delivery during “inventory” / “inventory & command” procedure.

Proposal 1: RAN2 to wait for SA3 inputs for considering inclusion of the DT information such as data and/or command in the AIoT paging message.

Proposal 2: When an AIoT paging addresses all devices, the group/device ID is still contained in the paging message but set to 0.

Observation 2: A permanent ID is not suitable for use as it provides privacy and security risk.

Proposal 3: RAN2 not to study permanent ID without consulting SA3.

Proposal 4: RAN2 to support the use of short device IDs for future access related procedures. FFS whether the ID is allocated by either the reader or device. FFS whether it is the short random ID.

Proposal 5: The short ID should not be based on the full ID due to security/privacy risk.

Proposal 6: The paging message needs to contain at least an ID type field indicating the type of identification.

Proposal 7: RAN2 to discuss whether to an AIoT paging message may also provide an indication to identify

- AIoT devices that are excluded from responding to a given AIoT paging request, i.e. devices that shall ignore the AIoT paging message.

- AIoT devices that shall respond to the AIoT paging message.

FFS whether there is a need for mapping to previous access occasions.

Proposal 8: The paging message may indicate a duration in which the device is excluded from responding e.g. a number of access occasions, number of paging occasions, or a time reference points.

Proposal 9: RAN2 to agree that individual paging requests are identified using a session ID for retransmission or reconfiguration purposes as well as to permit the execution of multiple parallel paging requests. FFS whether multi-bit identifiers (e.g., random ID or sequence number) or single-bit flags (e.g., similarly to NDI) are used.

Proposal 10: RAN2 to study how to terminate ongoing responses from AIoT devices to a given paging request.

Proposal 11: RAN2 to agree that an AIoT device that is engaged in a procedure associated with a certain session ID is not required to respond to an AIoT paging message with a different session ID.

Observation 3: Deployment of readers cannot always be planned to avoid the possibility that an AIoT device is within range of multiple readers, especially in Topology 2.

Proposal 12: RAN2 to further study whether it is necessary to include reader ID in the AIoT paging message to support the scenario where the AIoT device receives AIoT paging messages from multiple readers with the same session ID.

Proposal 13: To simplify signaling, RAN2 to agrees that

- an AIoT paging of a single-device means “Contention-free access”,

- an AIoT paging of a device group or all devices means “Contention-based random access”.

- An AIoT paging of multiple device IDs (if supported) means “Contention-free access”.

Proposal 14: RAN2 to agree that, if AIoT paging of multiple device IDs in a single paging message is supported, assignment of resources or access occasions to device IDs for contention-free access is implicit.

Proposal 15: RAN2 to agree that an AIoT paging message contains at least the following

- access type, in case of AIoT paging message targeting a device group or all devices supporting multiple access types (“2-step CBRA”, “3-step CBRA)

- service type (FFS options “Inventory only”, “Command only”, “Inventory & command”)

Proposal 16: AIoT paging message includes parameters indicating resources for contention-free access and contention-based random access (e.g., Q-parameter or maximum number of access occasions).

Proposal 17: RAN2 to study how AIoT devices can successfully complete the access procedure under the constraint of device energy availability.