3GPP RAN WG2 Meeting #128	R2-2410409

# Orlando, U.S.A, November 18th- 22nd, 2024

Agenda Item:	8.7.5

Source:	InterDigital

Title:	Discussion on RLC enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion

# 1. Introduction

The WID for R19 XR has the following objective for RLC enhancements [1]:

- Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

In RAN2 #127-bis meeting, we made several agreements for RLC enhancement [2], which are provided the appendix. In this contribution we further discuss the RLC enhancement topics. Specifically, we further discuss enhancement to avoid unnecessary retransmission and timely RLC retransmission.

# Discussion

  - 2.1 Timely RLC retransmissions

In the last meeting, RAN2 confirmed that the existing mechanisms are insufficient to resolve the timely RLC retransmission problem and we agreed to focus on autonomous retransmission and polling enhancement. In the following, we discuss the necessity/benefits to support each option.

Autonomous retransmission:

In the current operation of RLC Acknowledgement Mode in NR, the UE first performs the initial transmission of an RLC PDU. Afterwards, the UE only triggers retransmission of an RLC PDU if it receives an RLC status report from the gNB indicating the NACK for the RLC PDU. For RLC status reporting, the gNB can send a NACK status for an RLC PDU when detecting an SN gap between two consecutively received RLC PDUs or a missing segment of a RLC SDU [4]. Such a procedure may cause a long delay for a retransmission of an RLC SDU (segment) which may not be suitable for an RLC SDU with a small packet delay budget (PDB). This is because, after such a long delay, the retransmission of the RLC SDU (segment) may arrive beyond the PDB of the SDU and, hence impacting the QoS.

Therefore, enhancing the RLC re-transmission for the operation of the RLC AM can be beneficial. To enhance the RLC re-transmission for an RLC SDU with small PDB, the delay gap between the initial transmission and the re-transmission of the RLC SDU can be reduced. One possibility is to allow the UE to retransmit an RLC SDU without waiting for the RLC status reporting from the network. For example, the UE can retransmit the RLC SDU if the remaining time of the SDU is smaller than a configured threshold. Such retransmission can help improve the successful reception of the RLC SDU with small delay budget. Alternatively, the UE may fail to transmit a resulting RLC PDU due to an improper scheduling of transmission parameters such as the resource size, MCS, beam, etc. Therefore, with multiple HARQ retransmissions, the UE may still fail to successfully transmit the RLC PDU. In such cases, it can be beneficial for the UE to trigger a retransmission of the RLC SDU using another set of transmission parameters. Specifically, the UE may retransmit the RLC SDU, if the number of HARQ retransmissions for the RLC PDU exceeds a threshold.

For autonomous RLC retransmission based on the remaining time of a SDU, the network can properly configure a delay threshold to minimize the capacity loss while still guarantee the QoS (e.g., reliability) of the service. Regarding retransmission based on the number of HARQ retransmissions, capacity loss is minimal as the UE may still need to retransmit the SDU as it has not been received successfully.

Observation 1:	With a proper configuration of the triggering condition for autonomous retransmission, the network can minimize the capacity loss while still guarantee QoS of a service.

Proposal 1:	The UE retransmits a RLC SDU if the remaining time of the SDU is smaller than a configured threshold or the number of HARQ retransmissions is greater than a configured threshold.

Polling enhancement:

Regarding the polling procedure in NR, the UE is allowed to poll the RLC reception status of the gNB by setting the polling bit in the RLC header to one in the following cases:

    - The amount of data without polling is larger than a threshold (i.e., PDU_WITHOUT_POLL >= pollPDU; or BYTE_WITHOUT_POLL >= pollByte).

    - Its buffer (initial and retransmission buffer) will be empty after the current RLC PDU.

    - Its window is stalling (e.g., the gap between the current SN and the ACKed SN is greater than the window size).

In XR applications, the receiver decodes the received data per PDU set. Therefore, the reception status of each PDU set is more important than the reception status of each PDU in a PDU set. Moreover, in an XR application, the PDU sets may arrive at the UE periodically with a certain jitter. It is also expected that the amount of data per PDU set may vary from one PDU set to another. Specifically, the number of PDUs and the number of bytes per PDU set can be different among PDU sets.

If the current polling procedure is used, in which the UE polls the gNB after a configured number of PDUs or bytes, it is most likely that the UE may poll the RLC status of the gNB in the middle of a PDU set. As a result, the reception status of each PDU set may not be reported. To reduce the delay in determining the reception status of a PDU set, the UE can be configured with a smaller values of pollByte and/or pollPDU. However, this configuration may result in high signaling overhead as the UE may need to receive many more RLC status reports when the values of pollByte and/or pollPDU is small. Therefore, it may be beneficial to enhance the polling procedure, which may help the UE to receive the RLC status of one or more PDU sets. For example, the polling bit can be set based on the transmission of a PDU set (e.g., the last PDU of a PDU set) or it can be set periodically based on the PDU set traffic pattern.

Observation 1: 	Polling the gNB after a configured number of PDUs or bytes does not inform the UE of the RLC reception status of each PDU set.

Proposal 2:	The UE enables the polling in the last transmission of a PDU set to request the status report after transmissions of each uplink PDU set.

For delay-critical RLC SDU, upon its transmission, it is beneficial to be aware of the reception status of the RLC SDU as soon as possible. It is because the Tx RLC entity may then trigger retransmission of the RLC SDU to guarantee that the RLC SDU can reach the Rx entity before the PSDB. However, in general, the Rx RLC entity transmits the status report based on the polling indication from the Tx entity. Therefore, it is beneficial to allow the Tx RLC to set the polling indication for delay-critical RLC SDU.

Proposal 3:	The Tx RLC entity includes a polling indication when the remaining time of a RLC SDU being smaller than a configured threshold.

  - 2.2 Avoiding unnecessary retransmissions

It was confirmed in the last meeting that, as baseline, the Rx window advances before Tx window advance. Therefore, the Tx RLC entity may need to wait for a status report from the Rx RLC entity before advancing its window. In our view, it is beneficial to support the Tx RLC entity to indicate the obsolete/discarded RLC SDUs. It is because the Tx RLC entity is fully aware of the status of each obsolete/discarded RLC SDUs and relying on the Rx RLC entity to send the status report based on its local timer may incur delay in advancing windows at both ends. If the Rx RLC entity fails to receive the indication from the Tx RLC entity, it still can rely on its own local timer to advance the receiving window. Therefore, the system still works regardless of whether the indication is successfully received or not.

Proposal 4: 	The Tx RLC entity indicates the Rx RLC entity about the obsolete/discarded SDUs.

Regarding how to send indication about obsolete/discarded SDUs, there are two possibilities to send the indication. Specifically, the indication may can be sent in an RLC header or via an RLC control PDU. However, in our view, using a control PDU is simpler compared to using a header. It is because using a control PDU may provide more flexibility in the content of the indication.

Proposal 5: 	The Tx RLC entity uses new RLC control PDU to indicate the Rx RLC entity about the obsolete/discarded SDUs.

It was agreed that the Rx RLC entity may advance the window before the Tx RLC entity does. Therefore, upon reception of the indication from the Tx RLC entity, the Rx RLC entity shall advance the window and trigger status report to the Tx RLC entity for advancing the Tx window.

Proposal 6: 	The Rx RLC entity triggers status report when it receives the indication about the obsolete/discarded SDUs from the Tx RLC entity.

It was agreed that the Rx RLC entity may stop to receives an obsolete SDU based on its local timer. Such timer may be used in case the Rx RLC entity fails to receive the indication of the obsolete/discarded SDUs from the Tx RLC entity or no indication is sent. In our view, the timer should be started upon detection of an SN gap in the received RLC buffer. The Rx RLC entity then move its receiving window upon expiry of the timer. To help moving the Tx window, the Rx RLC entity needs to send the status report to the Tx RLC entity after moving its receiving window. Therefore, the Rx RLC entity also needs to send the status report to the Tx RLC upon the expiry of the timer as well.

Proposal 7: 	To support moving the Tx and Rx windows, the Rx RLC entity starts a new timer upon detection of an SN gap and triggers status reporting upon expiry of the timer.

The purpose of the status report from the Rx RLC entity is to help the Tx RLC entity to move its transmitting window. In the legacy behavior, the Tx RLC entity forward its transmitting window based on ACK report for an SDU. Therefore, in the status report, the Rx RLC entity also can indicate the obsolete/discarded SDUs as ACK(s).

Proposal 8: 	The Rx AM RLC entity reports the obsolete/discarded RLC PDUs as ACK in the status report.

In the last meeting, RAN2 agreed to further study whether there is any issue with PDCP Control PDU (C-PDU) handling if RLC is configured with unnecessary re-transmission avoidance feature. In the legacy system, all PDCP C-PDUs (except UDC feedback) can be transmitted over both UM and AM mode bearers. Therefore, all those PDCP C-PDUs (except UDC) could already be lost over-the-air (OTA) in the legacy with UM mode bearer already. Furthermore, regarding UDC, it is used by NW to indicate UDC checksum error (indicates the compression buffer and de-compression buffer are out of synchronization). However, if this is lost OTA, the NW can still detect the out of sync and re-transmit the UDC feedback (the NW also configures the timer in the Rx RLC entity and is aware of the discard possibility).

Observation 2: 	No specific handling of PDCP Control PDUs is needed with the re-transmission avoidance feature.

# Conclusion

In this contribution, the following observations are made:

Observation 1: 	Polling the gNB after a configured number of PDUs or bytes does not inform the UE of the RLC reception status of each PDU set.

Observation 2: 	No specific handling of PDCP Control PDUs is needed with the re-transmission avoidance feature.

Based on this, the following proposals are made:

Proposal 1:	The UE retransmits a RLC SDU if the remaining time of the SDU is smaller than a configured threshold or the number of HARQ retransmissions is greater than a configured threshold.

Proposal 2:	The UE enables the polling in the last transmission of a PDU set to request the status report after transmissions of each uplink PDU set.

Proposal 3:	The Tx RLC entity includes a polling indication when the remaining time of a RLC SDU being smaller than a configured threshold.

Proposal 4: 	The Tx RLC entity indicates the Rx RLC entity about the obsolete/discarded SDUs.

Proposal 5: 	The Tx RLC entity uses new RLC control PDU to indicate the Rx RLC entity about the obsolete/discarded SDUs.

Proposal 6: 	The Rx RLC entity triggers status report when it receives the indication about the obsolete/discarded SDUs from the Tx RLC entity.

Proposal 7: 	To support moving the Tx and Rx windows, the Rx RLC entity starts a new timer upon detection of an SN gap and triggers status reporting upon expiry of the timer.

Proposal 8: 	The Rx AM RLC entity reports the obsolete/discarded RLC PDUs as ACK in the status report.

# References

- RP-241771, “Revised WID on XR (eXtended Reality) for NR Phase 3”, Nokia, 2024-09.

- RAN2#127-bis chair notes.

- 3GPP TS 38.322, Radio Link Control (RLC) protocol specification, v.18.0.0, 2023-12.

# Appendix: RAN2 #127-bis on RLC enhancement for XR

Agreements on RLC timely retransmissions - RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay Agreements on avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)