3GPP TSG-RAN WG2 Meeting #128										R2-2410412

Orlando, USA, Nov 18th– 22nd, 2024

Agenda item		: 8.4.2

Title	: Discussion on IDLE/INACTIVE procedures for LP-WUS

Source				: Tejas Networks Limited

Document for		: Discussion and Decision

# Introduction

During RAN2#127 meeting, RAN2 WG made the following agreements for IDLE and INACTIVE mode procedure for LP-WUS:

- Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE. - Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.. - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. - The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. - The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

And in RAN2#127bis meeting, RAN2 further agreed:

- If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). - The LPWUS monitoring exit condition does not include MR measurements. - For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion. - For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping. - RAN2 inform this conclusion to SA2/CT1/RAN3. - This LS is approved in R2-2409226

This contribution further addressed the procedure and configuration of LP-WUS for IDLE/INACTIVE modes.

# Discussion

## 2.1 Impact of UE not supporting LP-WUS on all bands

RAN1 sent an LS (R2-2407921) indicating that the LR at UE might not support all the bands supported by the MR. This could lead to a scenario where:

- MR is operating in a particular frequency f1 and

- LR does not support reception in f1 leading to 2 cases:

  - Case 1:LR supports and can operate on a different frequency f2 and the NW is transmitting LP-SS and LP-WUS signals in that supported frequency,

  - Case 2: LR does not support any of the bands in which the NW is transmitting LP-SS and LP-WUS

In case 1, the MR and LR would be operating in different frequencies and this needs special handling of the entry condition. Since the frequencies supported by the LR and MR are different, the thresholds to check for the entry condition could be different for LR and MR. It was agreed that NW can optionally broadcast separate thresholds for LR and MR, and If configured, UE checks for both the conditions to start using LR.

In case that NW does not broadcast separate thresholds for LR for that frequency that UE supports, UE does not use LR to monitor LP-WUS and used legacy Paging mechanism.

Alternatively, UE can prioritize a cell which uses a frequency supports by the LR but this might in some cases lead to over loading of that frequency as many UEs could reselect to that cell which is not a desirable behavior.

Proposal 1: The granularity of entry condition thresholds for LR and MR broadcasted by NW should be per frequency band supported by the NW.

Proposal 2: If the LR does not support frequency band that MR is receiving in, UE uses LR to receive LP-WUS only if the UE has valid entry condition threshold configured for LR for the frequency band that LR supports.

Proposal 3: If the LR does not support any of the frequency band that NW is using to transmit LP-SS/LP-WUS, UE does not use LR and uses MR for legacy paging mechanism.

## 2.2 Entry/Exit criteria for using LR:

The main advantage of using LR to monitor wake up indication is that MR can move to ultra-deep sleep/ deep sleep state to save power. The currently agreed entry and exit conditions only consider the signal qualities of LP-SS/SSB measured by LR/MR. Considering just the thresholds might lead to ping-pong effect of moving between use of LR and MR resulting in UE not having sufficient time to move to between sleep states. This can even result in adverse effect as there is power penalty associated with transition between sleep states. In order to overcome this, we propose to reuse the similar principle used in Rel16 relaxed measurement criteria where there is a time period configured for which the evaluation criteria need to be satisfied to consider the condition as fulfilled.

Proposal 4: RAN2 to consider evaluation time period for which the entry/exit conditions needs to be satisfied to switch between LR/MR.

## 2.3 LO and PO association

RAN1 in R1#118bis agreed to discuss the following options for determining the association of LO with a PO:

- Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. - Option 1: gNB configures a single offset value. - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS. - Otherwise, - Option 1-1: the UE follows the legacy paging monitoring procedure. - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - Option 2: gNB configures one or multiple offset values. - For the same PO, each offset corresponds to a LO. - This does not preclude the possibility that the same LO may correspond to different POs with different offset values. - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs. - FFS exactly how to choose the offset - Option 2B: - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay. - FFS exactly how to choose the offset - Otherwise, - Option 2B-1: the UE follows the legacy paging monitoring procedure. - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - FFS exactly how to choose the offset

The drawback with Option 1 is that it considers a single offset value which has to consider the worst case delay value of all the UEs in the cell. Since the NW will not have the knowledge of the UEs currently within its coverage, it will not be able to pick an optimal offset value. So picking an offset value becomes an issue. One more drawback if the NW picks a smaller offset value is that there will be additional delay incurred by the UEs which had wake-delay greater than the offset value as they will miss the associated PO and will have to monitor the next valid PO. Alternatively, if the NW picks a larger offset value, the UE receiving the LP-WUS will be moved out of sleep state soon even if its LR has a smaller the minimum wake up delay.

Observation 1: NW indicating a single offset value (Option 1 from RAN1) has many drawbacks.

Considering Option 2: From the network perspective, there can be multiple UEs having different wake-up delays in the cell, so NW indicates multiple offset values from the reference PO/PF. The UE monitors an LO by selecting an offset value which is at least greater than or equal to the wake delay of the LR.

Proposal 5: RAN2 to consider NW broadcasting multiple offset as baseline and further study the alternatives within option 2.

# Conclusion

Provision Impact of LR not supporting all bands as MR

Proposal 1: The granularity of entry condition thresholds for LR and MR broadcasted by NW should be per frequency band supported by the NW.

Proposal 2: If the LR does not support frequency band that MR is receiving in, UE uses LR to receive LP-WUS only if the UE has valid entry condition threshold configured for LR for the frequency band that LR supports.

Proposal 3: If the LR does not support any of the frequency band that NW is using to transmit LP-SS/LP-WUS, UE does not use LR and uses MR for legacy paging mechanism.

Entry/Exit criteria for using LR

Proposal 4: RAN2 to consider evaluation time period for which the entry/exit conditions needs to be satisfied to switch between LR/MR.

LO and PO association

Observation 1: NW indicating a single offset value between LO and PO (Option 1 from RAN1) has many drawbacks.

Proposal 5: RAN2 to consider NW broadcasting multiple offset values between a PO and LOs (Option 2 from RAN1) as baseline and RAN2 to further study how UE selects the offset value.