3GPP TSG-RAN WG2 Meeting #128										R2-2410413

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item		: 8.4.4

Title				: Discussion on CONNECTED mode procedures for LP-WUS

Source				: Tejas Networks Limited

Document for		: Discussion and Decision

# Introduction

The following agreements have been made so far in RAN2 for CONNECTED mode procedures for LP-WUS:

The Agreements at RAN2 #127: - For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1. Agreements at RAN2 #127bis: - For option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. - In option 1-2, a new timer triggered by LP-WUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LP-WUS monitoring. - When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

In this contribution, we will further discuss the RAN2 related issues on procedure and configuration of LP-WUS for IDLE/INACTIVE modes and give our proposals.

# Discussion

## 2.1 LP-WUS monitoring during CDRX:

For LP-WUS monitoring with CDRX, the following options were considered,

Option 1-1: A direct replacement of DCP where UE monitors LP-WUS before the C-DRX onDuration and if a LP-WUS intended for the UE is received, UE starts the onDurationTimer and monitors PDCCH as in legacy CDRX mechanism.

Option 1-2: UE monitors LP-WUS outside of the active time of CDRX. When UE receives wake up indication, a new timer is started and UE monitors PDCCH and UE is considered to be in CDRX active time.

In our understanding, option 1-2 is mainly meant for sending in delay sensitive data where the UE can be indicated to receive the data instead of waiting for the next CDRX cycle. If the UE is configured with a CDRX configuration, UE follows the legacy CDRX mechanism of starting the onDurationTimer based on the CDRX cycle. If UE is configured with option1-2, UE monitors for LP-WUS only in the periods outside of the CDRX active period as determined by the legacy timers.

Proposal 1: UE starts the onDurationTimer as per legacy mechanism even if option 1-2 is configured.

Proposal 2: UE monitors LP-WUS only in time periods outside of the CDRX active time as determined by the legacy CDRX framework

Proposal 3: If option1-2 is configured without the CDRX configurations, UE monitors for PDCCH only based on the new timer associated with the LP-WUS.

# Conclusion

Proposal 1: UE starts the onDurationTImer as per legacy mechanism even if option 1-2 is configured.

Proposal 2: UE monitors LP-WUS only in time periods outside of the CDRX active time as determined by the legacy CDRX framework

Proposal 3: If option1-2 is configured without the CDRX configurations, UE monitors for PDCCH only based on the new timer associated with the LP-WUS.