3GPP TSG-RAN WG2 Meeting #128	R2-2410436

Orlando, US, Nov 18th – 22th, 2024

Agenda item:	8.7.4.2

Source:	China Telecom

Title:	Discussion on Leftover Issues for DSR Enhancement

Document for:	Discussion and Decision

- Introduction

The DSR enhancement had been widely discussed in the last RAN2#127bis meeting, and the following agreements were achieved:

- RAN2#127 agreements We do not change the definition of delay-critical data For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. There is no need to include PSI in the enhanced DSR MAC CE. A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

This paper aims to discuss the remaining issues with respect to DSR enhancement for XR.

2 Discussion

In Rel-18, there is only one remaining time threshold. Once the remaining time falls below the threshold, the data becomes delay-critical data and a DSR is triggered to report the delay-critical data, which means that the remaining time threshold is used for both reporting threshold and triggering threshold. To increase the granularity of delay information, it was agreed in the RAN2#127 meeting that multiple reporting thresholds could be configured for each LCG to report multiple pairs of remaining time and buffer size information. However, it is undetermined how to configure the triggering threshold with multiple reporting thresholds.

In the last meeting, it has agreed that the triggering threshold can be configured as the value lower than the largest reporting threshold on DSR procedure. There would be following two cases.

- Case1 (TriggerThresh ≤ ReportThresh1(smallest) < ReportThresh2< ReportThresh3(largest) )

For simplicity, the delay critical data is also defined as the remaining time smaller than the DSR triggering threshold. In this case, once a DSR is triggered, the information of delay-critical data (with triggering threshold 1) and all non-delay critical data (with other triggering threshold) would be reported. In this way, the network can optimize the resource allocation for delay-critical data first. After that, the network can schedule resources for other non-delay critical data, which can avoid the case where most of the data becomes delay-critical data before transmission.

Observation 1: Setting the triggering threshold to a value equivalent to a lower reporting threshold can avoid the case where most of the data becomes delay-critical data before transmission.

- Case2 (ReportThresh1(smallest) < ReportThresh2 < TriggerThresh < ReportThresh3(largest) )

In this case, once a DSR is triggered, the UE will report the information of all delay-critical data (whose reporting threshold is smaller than/equal to the triggering threshold) and the non-delay critical data (with reporting threshold 3) to the network. If delay-critical data has not been reported previously, there would be less remaining time for resource scheduling and data transmission. Even worse, if there is a large amount of delay-critical data, the network may not have sufficient time or resources to guarantee its transmission.

Observation 2: Setting the triggering threshold to a value equivalent to a larger reporting threshold may leave less remaining time for delay-critical data transmission.

Based on the above description, setting the triggering threshold to a value equivalent to a lower reporting threshold (such as case 1) can provide the network with greater flexibility in resource allocation. However, if the triggering threshold is set to a value only smaller than the largest reporting threshold, some delay-critical data may not be transmitted timely. Therefore, it would be preferable to set the triggering threshold to a value equivalent to a lower reporting threshold.

Proposal 1: The triggering threshold is set to a value equivalent to a lower reporting threshold.

Per TS38.321, the legacy DSR MAC CE in Rel-18 is shown as follows. It can be observed that there is only one pair of remaining time and buffer size information can be reported for each LCG. From the specification point of view, it would be difficult to extend the legacy DSR MAC CE to contain multiple pairs of remaining time and buffer size information for each LCG. Therefore, it would be better to introduce a new DSR MAC CE to report multiple reporting thresholds associated with the LCG. Once an LCG is configured with multiple reporting thresholds, the new DSR MAC CE would be used. In some way, the legacy DSR MAC CE can also be considered as a special case of the new DSR MAC CE. Therefore, in case the new DSR format is used, a one-bit indication can be used to indicate whether one or more than one pair of remaining time and buffer size information exist for the associated LCG. In this way, all the LCGs with one or multiple pairs of remaining time and buffer size information can be reported by the new DSR MAC CE.

Proposal 2: Once an LCG is configured with multiple thresholds, only the new DSR MAC CE will be used. A one-bit indication can be used to indicate whether there is one or more reporting thresholds associated with the LCG.

Figure 1: Legacy DSR MAC CE

It is undetermined whether non-delay critical data ahead of delay critical data can be included during the buffer size calculation for DSR. Based on the current spec, the data within an LCH is assembled in a first-in and first-out manner. It means that the non-delay critical data ahead of delay critical will be assembled first and consume some UL grant. If the gNB is unaware of non-delay critical data, after non-delay critical data is assembled, the remaining resources may not be enough for delay critical data. As a result, the delay critical data transmission would be impacted.

Furthermore, if the non-delay critical data has higher priority than the delay critical data, upon reception of the UL grant, the non-delay critical data will be assembled first. In this case, the non-delay critical data information also needs to be reported to the gNB. Based on it, the gNB can allocate enough UL grant to ensure the delay critical data transmission.

From our point of view, reporting both non-delay critical data and delay critical data can help gNB have a better understanding of the whole buffer status of the UE. If the non-delay critical data is not included in the buffer size calculation, the non-delay critical data may consume the resource allocated for the delay critical data. As a result, the delay critical data would be impacted or even discarded.

Proposal 3: Non-delay critical data ahead of delay critical data or with higher priority also should be included in the buffer size calculation for DSR.

- Conclusions

Based on the above analysis, we have the following observations and proposals:

Observation 1: Setting the triggering threshold to a value equivalent to a lower reporting threshold can avoid the case where most of the data becomes delay-critical data before transmission.

Observation 2: Setting the triggering threshold to a value equivalent to a larger reporting threshold may leave less remaining time for delay-critical data transmission.

Proposal 1: The triggering threshold is set to a value equivalent to a lower reporting threshold.

Proposal 2: Once an LCG is configured with multiple thresholds, only the new DSR MAC CE will be used. A one-bit indication can be used to indicate whether there is one or more reporting thresholds associated with the LCG.

Proposal 3: Non-delay critical data ahead of delay critical data or with higher priority also should be included in the buffer size calculation for DSR.

- References

- R2-24095xx, Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China