3GPP TSG-RAN WG2 #128	R2-2410462

# Orlando, USA, 18th – 22nd November 2024

Agenda Item:	7.2.3

Source:	Ericsson

Title:	Summary of RRC proposals for feMob

Document for:	Discussion

# 1	Introduction

In this contribution is provide a summary of the RRC proposals for FeMob with some possible conclusion for each of them. Please, note that stage 2 corrections and IPA CRs are left out from this document.

# 2	Discussion

Hereafter the contributions submitted to AI 7.2.3 are listed and a possible outcome for what is proposed is suggested:

| Tdoc # | Company | Proposal(s) | Suggested conclusion |
| --- | --- | --- | --- |
| R2-2410055 | Qualcomm, Oppo | P1: Propose to clarify for several capabilities (e.g., ltm-BeamIndicationJointTCI, ltm-BeamIndicationSeparateTCI, ta-IndicationCellSwitch-r18, etc)) that: LTM capability XYZ….for LTM procedure is supported only if the UE sets the capability value for the source PCell and the target PCell bands. For inter-band operation, supported values of the parameters of the source PCell band apply. P2: Propose to clarify for rach-EarlyTA-Measurement that: the UE shall indicate the same number of candidate cells for all bands. | P1: To be discussed. The clarification seems strange and probably is unnecessary. Probably nothing is really needed. P2: To be discussed. The understanding is that this capability may be different depending on the bands, and thus probably this clarification is not needed. |
| R2-2410117 | MediaTek | P1: Propose to adopt changes receives by the RAN1 LS in R1-2409043; P2: Propose to extend the range of some capabilities to up to 32 (according to what has been received by the RAN1 LS) - Option 1: Dummify the whole parent field. - Option 2: Dummify only the capabilities field for which the range need to be extended. P3: Clarify for FG45-3a/4a, 45-5/5a/6, and FG45-3/4 that if UE supports ltm-InterFreq-r18, the support of above FGs implies the support for inter-frequency scenarios in addition to intra-frequency scenarios (according to what has been received in the RAN1 LS). | P1: To be agreed. This is in line with what RAN1 is asking us to clarify P2: To be agreed. This is in line with what RAN1 is asking us to clarify and probably option 1 is cleaner. P3: To be agreed. This is in line with what RAN1 is asking us to clarify See also tdoc in R2-2410460 |
| R2-2410063 | NEC | P1: In case of LTM cell switch without L2 reset, UE triggers PDCP recovery for an AM DRB, if one of its associated RLC bearer is released. The draft CR in Annex is adopted | P1. Not pursued. This change is NBC. Apart from this, it is questionable whether this change is really needed. |
| R2-2409922 | Google | P1: Propose to clarify in the field description of ltm-CandidateConfig that: “The RRCReconfiguration message contained in this field cannot contain the conditionalReconfiguration, daps-Config, and ltm-CandidateConfig” and also to add ltm-Config in the field description of condRRCReconfig | P1. Not pursued. We already have a clarification in 38.300 that DAPS cannot be used together with LTM and this should be enough, as this is a network restriction. Also, the co-existence of LTM and CHO has been discussed during the WI and we agreed that they can be configured together. |
| R2-2410018 | CATT | P1: Change the restriction in the field description of ltm-UE-MeasuredTA-ID “If the field is present for a LTM candidate configuration with tag2 configured, network ensures that this field is configured with a value different from the values configured for other LTM candidate configurations and the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID.” | P1: To be discussed. I guess the scenario highlighted makes sense, but then we should decide whether to go the way how is proposed by CATT or whether we should capture that if tag2 is configured for an LTM candidate cell, than this field should be absent for all other LTM candidate cell (but maybe this latter option restrict a bit too much) |
| R2-2410019 | CATT | P1: Clarify in the field description of ltm-NoResetID that the network ensures that the UE has stored a valid value for ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID when ltm-NoResetID is configured for candidates | P1: Not pursued. We can rely on network implementation and there is no need to further clarify this. |
| R2-2409900 | MediaTek | P1: Field descriptions of ltm-Config, mrdc-SecondaryCellGroupConfig, and reestablishRLC, and Cond 'MasterKeyChange' are corrected to take ltm-ReferenceConfiguration into account. P2: Cond 'LTM' for cg-LTM-Configuration within ConfiguredGrantConfig IE is corrected to capture that the field is optionally present, need R, in an LTM candidate configuration. It is captured using the same wording used for P1. P3: Correct field description of nrofHARQ-Processes by removing the unnecessary text. | P1: To be agreed. Probably current text already work, but the proposed clarification it makes sense. P1: To be agreed. I guess the clarification is okay. P3: Not pursued. The restriction should not be deleted but maybe it should be clarified that “The network sets the value of this field to 1 when cg-LTM-Configuration is configured.” |
| R2-2409777 | Vivo | P1: RAN2 confirms that LTM is supported by (e)RedCap UEs P2: Delete the wording that MAC entity indicates the SSB index to lower layer when UE performs RACH-less based LTM cell switch in current MAC spec | P1. Discuss this in the R18 common session. P2: To captured in MAC summary |
| R2-2410442 | Nokia | P1: Add reference to 38.133 in section 9.2.3.5.2 of 38.300 P2: Propose to clarify the following: “for each drb-Identity value that is part of the same RLC entity that is re-established:” | P1: To be addressed in stage 2 summary (correction seems straightforward). P1: Not pursued. Current text already works fine. One can discuss whether to refine it but probably it not worth to do down this road. |
| R2-2410513 | Huawei | P1: Propose the TPs (38.306 and 38.331) to introduce the new capability ltm-interFreqL1-OnlyInBC | P1: To be discussed. Change seems straightforward it would be good to check whether the current measurement inter-frequency capability description needs some further clarification. |
| R2-2410460 | Ericsson | P1: Clarify in TS 38.306 for FG45-3a/4a, 45-5/5a/6, and FG45-3/4 that if the UE indicates support of ltm-InterFreq-r18, the UE supports both intra-frequency and inter-frequency L1-RSRP measurements for LTM. P2: The range of the capabilities maxNumberJointTCI-AcrossCells, maxNumberDL-TCI-AcrossCells, maxNumberUL-TCI-AcrossCells is changed from 1 to 32 in a BC way. | P1: To be agreed. This is in line with what RAN1 is asking us to clarify P2: To be agreed. This is in line with what RAN1 is asking us to clarify Same topic (and some proposals) addressed also in R2-2410117 |
| R2-2410461 | Ericsson | P1: RAN2 to confirm that, during an LTM cell switch execution, once the UE releases all fields related to the SRB/DRB configuration, the UE keep for SRB/DRB in the current configuration the srb-Identity, drb-Identity, pcdp-config, and sdap-config. P2: RAN2 to confirm that, upon an LTM cell switch procedure with a L2 reset, the logicalChannelIdentity and logicalChannelIdentityExt of the radio bearer in current UE configuration can be reconfigured. P3: Clarify that s-measurement applies to both LTM L1 measurements and L3 measurements P4: The text related to LTM in section 5.4.4 is deleted as the UE will never execute those actions | P1: To be agreed. The change does not change what the UE does but clarify more from a logical order the UE actions. P2: To be discussed. This is not crystal clear from the procedureal text. Would be good to clarify (maye procedural P3: To be agreed. Clarification seems fine and in line with what was agreed in last RAN2 meeting. P4: To be included in the MAC summary. |
| R2-2410449 | Lenovo | P1: Simultaneous execution of CHO (e.g. CHO without SCG configuration, CHO with SCG configuration, CHO with candidate SCG) and Intra-CU SCG LTM is not supported in Rel-18 | P1: to be addressed in stage 2 discussions (this was already discussed and not agreed, not sure we need to re-discuss). |
| R2-2410707 | Nokia | P1: The field description for SK-counter configuration includes note that NW includes new values for SK-counter configuration on master-key change. P2: The securityCellSetID reference in SCPAC execution is clarified | P1: To be discussed. Probably there are alternative ways to solve this. I guess solutions can be discussed first. P2: Not pursued. This does not look like an essential correction. |
| R2-2410737 | Google | P1: Changes to what is proposed in R2-2409922 | P1. Not pursued. We already have a clarification in 38.300 that DAPS cannot be used together with LTM and this should be enough, as this is a network restriction. Also, the co-existence of LTM and CHO has been discussed during the WI and we agreed that they can be configured together. |

According to what is discussed above, the following proposals are made.

The following proposals are suggested to be agreed:

Clarify in TS 38.306 for FG45-3a/4a, 45-5/5a/6, and FG45-3/4 that if the UE indicates support of ltm-InterFreq-r18, the UE supports both intra-frequency and inter-frequency L1-RSRP measurements for LTM.

The range of the capabilities maxNumberJointTCI-AcrossCells, maxNumberDL-TCI-AcrossCells, maxNumberUL-TCI-AcrossCells is changed from 1 to 32 in a BC way.

Field descriptions of ltm-Config, mrdc-SecondaryCellGroupConfig, and reestablishRLC, and Cond 'MasterKeyChange' are corrected to take ltm-ReferenceConfiguration into account.

Cond 'LTM' for cg-LTM-Configuration within ConfiguredGrantConfig IE is corrected to capture that the field is optionally present, need R, in an LTM candidate configuration. It is captured using the same wording used for P1.

RAN2 to confirm that, during an LTM cell switch execution, once the UE releases all fields related to the SRB/DRB configuration, the UE keep for SRB/DRB in the current configuration the srb-Identity, drb-Identity, pcdp-config, and sdap-config.

Clarify that s-measurement applies to both LTM L1 measurements and L3 measurements

The following proposals are suggested to be discussed ONLINE:

RAN2 to discuss whether to propose to clarify for several capabilities (e.g., ltm-BeamIndicationJointTCI, ltm-BeamIndicationSeparateTCI, ta-IndicationCellSwitch-r18, etc)) that: LTM capability XYZ….for LTM procedure is supported only if the UE sets the capability value for the source PCell and the target PCell bands. For inter-band operation, supported values of the parameters of the source PCell band apply.

RAN2 to discussion whether to propose to clarify for rach-EarlyTA-Measurement that: the UE shall indicate the same number of candidate cells for all bands.

RAN2 to discuss whether to change the restriction in the field description of ltm-UE-MeasuredTA-ID “If the field is present for a LTM candidate configuration with tag2 configured, network ensures that this field is configured with a value different from the values configured for other LTM candidate configurations and the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID.”

RAN2 to discuss proposed TPs (38.306 and 38.331) to introduce the new capability ltm-interFreqL1-OnlyInBC

RAN2 to confirm that, upon an LTM cell switch procedure with a L2 reset, the logicalChannelIdentity and logicalChannelIdentityExt of the radio bearer in current UE configuration can be reconfigured.

RAN2 to discuss if the field description for SK-counter configuration includes note that NW includes new values for SK-counter configuration on master-key change.

The remaining proposals are suggested to be discussed OFFLINE:

Discuss the remaining RRC proposals (including the ones suggested to not be pursued) offline, except for stage2.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

The following proposals are suggested to be agreed:

Proposal 1	Clarify in TS 38.306 for FG45-3a/4a, 45-5/5a/6, and FG45-3/4 that if the UE indicates support of ltm-InterFreq-r18, the UE supports both intra-frequency and inter-frequency L1-RSRP measurements for LTM.

Proposal 2	The range of the capabilities maxNumberJointTCI-AcrossCells, maxNumberDL-TCI-AcrossCells, maxNumberUL-TCI-AcrossCells is changed from 1 to 32 in a BC way.

Proposal 3	Field descriptions of ltm-Config, mrdc-SecondaryCellGroupConfig, and reestablishRLC, and Cond 'MasterKeyChange' are corrected to take ltm-ReferenceConfiguration into account.

Proposal 4	Cond 'LTM' for cg-LTM-Configuration within ConfiguredGrantConfig IE is corrected to capture that the field is optionally present, need R, in an LTM candidate configuration. It is captured using the same wording used for P1.

Proposal 5	RAN2 to confirm that, during an LTM cell switch execution, once the UE releases all fields related to the SRB/DRB configuration, the UE keep for SRB/DRB in the current configuration the srb-Identity, drb-Identity, pcdp-config, and sdap-config.

Proposal 6	Clarify that s-measurement applies to both LTM L1 measurements and L3 measurements

The following proposals are suggested to be discussed ONLINE:

Proposal 7	RAN2 to discuss whether to propose to clarify for several capabilities (e.g., ltm-BeamIndicationJointTCI, ltm-BeamIndicationSeparateTCI, ta-IndicationCellSwitch-r18, etc)) that: LTM capability XYZ….for LTM procedure is supported only if the UE sets the capability value for the source PCell and the target PCell bands. For inter-band operation, supported values of the parameters of the source PCell band apply.

Proposal 8	RAN2 to discussion whether to propose to clarify for rach-EarlyTA-Measurement that: the UE shall indicate the same number of candidate cells for all bands.

Proposal 9	RAN2 to discuss whether to change the restriction in the field description of ltm-UE-MeasuredTA-ID “If the field is present for a LTM candidate configuration with tag2 configured, network ensures that this field is configured with a value different from the values configured for other LTM candidate configurations and the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID.”

Proposal 10	RAN2 to discuss proposed TPs (38.306 and 38.331) to introduce the new capability ltm-interFreqL1-OnlyInBC

Proposal 11	RAN2 to confirm that, upon an LTM cell switch procedure with a L2 reset, the logicalChannelIdentity and logicalChannelIdentityExt of the radio bearer in current UE configuration can be reconfigured.

Proposal 12	RAN2 to discuss if the field description for SK-counter configuration includes note that NW includes new values for SK-counter configuration on master-key change.

The remaining proposals are suggested to be discussed OFFLINE:

Proposal 13	Discuss the remaining RRC proposals (including the ones suggested to not be pursued) offline, except for stage2.