3GPP TSG-RAN2 Meeting #128	R2-2410478

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:	8.11.2

Source:	Qualcomm Incorporated

Title:	Views on random access for SBFD

WID/SID:	NR_duplex_evo-Core – Release 19

Document for:	Discussion and Decision

# Introduction

RAN plenary #102 approved the work item on NR duplex operation, Sub-Band Full Duplex (SBFD) and the latest WID is in [1]. The objective of the work item is to specify SBFD operation at the gNB side within a TDD carrier. One of the objectives is to specify SBFD operation to support random access in SBFD symbols for UE in RRC-connected UE while studying the benefit of extending the random-access operation in SBFD symbols to UE(s) in RRC idle and inactive state.

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: … - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] …

In RAN2 #127 meeting, the following agreements were made.

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed. Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

In RAN2 #127bis meeting, the following agreements were made regarding random access for SBFD operation.

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first. The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

In this contribution, we would like to discuss the configurations of random access resource in SBFD symbols for UE in RRC_CONNECTED and RRC_IDLE/INACTIVE.

# Discussion

There were already several basic agreements regarding supporting SBFD random access operation in RAN1.

Agreement Confirm the working assumption: Working assumption: For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols. Agreement For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH)

The working assumption to support both CBRA and CFRA in SBFD symbols were confirmed in RAN1. RAN1 also agreed to support Type-1 random access (4-step RACH) in SBFD symbols and will further discuss Type-2 random access (2-step RACH) after finalizing the design of 4-step RACH. RAN2 #127 meeting further agreed RAN2 will focus on the 4-step RACH for SBRA RACH, FFS on 2-step if needed.

Observation 1: For the RRC_CONNECTED UE, RAN1 has agreed to support both CBRA and CFRA in SBFD symbols.

Observation 2: Both RAN2 and RAN1 has agreed to at least support 4-step RACH and FFS on 2-step RACH.

It could be observed that there are several benefits if the SBFD symbols is enabled for random access such as access latency reduction, PRACH collision probability reduction and improved coverage of PRACH. In our view, all the existing RACH triggering events can be supported by random access procedure in SBFD symbols. One may argue that SI request may be the exception. However, we don’t see a reason to exclude it, since it is the same procedure if network can configure the SBFD resource for SI request and it is up to network to configure this. Thus, the RAN2 working assumption can be confirmed.

Proposal 1: RAN2 confirms the working assumption that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Regarding the RACH configuration for SBFD operation, there were two options under RAN1 discussion. One option is to use single RACH configuration (i.e., RACH configuration Option 1 with Alt 1-1 defined in RAN1) from legacy RACH configuration with modification, and another option is to have two separate RACH configurations (i.e., RACH configuration Option 2) including one legacy RACH configuration and one additional RACH configuration to configure ROs (RACH occasion) within UL subband in SBFD symbols which could be valid for SBFD-aware UE.

It can be observed that single RACH configuration would enable random access for SBFD operation for both SBFD-aware UE and legacy UE without extra signaling overhead and separate RACH configuration for SBFD operation may enable more flexible configuration on the PRACH format, PRACH time/frequency resource, and so on.

In RAN1 #118, RAN1 confirmed the working assumption on supporting both options by network configuration but UE is not required to support both options.

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

Further, RAN1 #118bis made following conclusion that it is not supported to enable both RACH configuration option 1 and option 2 for SBFD RACH operation in a cell.

Conclusion There is no consensus in RAN1 to support the following in Rel-19: - Enabling both RACH configuration Option 1 and Option 2 for SBFD random access operation in a cell from network side.

Observation 3: It is not supported to enable both RACH configuration option 1 and option 2 for SBFD RACH operation in a cell from network side.

For a SBFD-aware UEs in RRC_CONNECTED state, it may indicate whether supporting single or separate RACH configurations via UE capabilities. It is up to network to determine single or separate RACH configurations and network provides one RACH configuration for all UEs in the serving cell. For example, network may configure separate RACH configuration. However, if UE only supports single RACH configuration, UE has to use the legacy RACH resource to perform RACH.

Proposal 2: For a SBFD-aware UE in RRC_CONNECTED state and RRC_IDLE/INACTIVE state, it is up to network to configure the UE with either a single RACH configuration (i.e., RACH configuration Option 1 with Alt 1-1 defined in RAN1) or two separate RACH configurations (i.e., RACH configuration Option 2) for SBFD operation.

Proposal 3: If SBFD-aware UE supports the RACH configuration for SBFD according to UE capability and network configures the RACH resource in SBFD symbols matching the UE capability, UE can use SBFD RACH resource.

In addition, for SBFD-aware UE in RRC_IDLE/INACTIVE state, it is straightforward to determine whether the RACH is applicable to the SBFD symbol if network configures separate RACH configurations via the system information. However, for the single RACH configuration, network should also indicate the legacy RACH configuration as only legacy (no valid SBFD-ROs in SBFD symbols) or as single RACH configuration including SBFD resource (with valid ROs in SBFD symbols).

Proposal 4: For single RACH configuration (i.e., RACH configuration Option 1 with Alt 1-1 defined in RAN1), whether the configured ROs in SBFD symbols are valid or not is indicated in system information.

The discussions on two options of SBFD RACH configuration in RAN1 are still ongoing. RAN2 should wait for RAN1 further inputs on the detailed RACH configuration parameters.

Proposal 5: RAN2 waits for RAN1 further inputs on the detailed RACH configuration parameters for SBFD operation.

In RAN2 #127bis, it has been agreed that ‘Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.’

It should be possible that network may identify the type of UE (i.e., SBFD-aware UE) via the selected ROs, and further considerations that SBFD-ROs may be less congested than TDD-ROs and the probability of RACH collision can be reduced. In that sense, network may directly indicate which type of RO that SBFD-aware UE is allowed to initiate RACH.

Proposal 6: Upon initiation of RACH procedure for a SBFD-aware UE, if network provides the indication on the prioritization of the legacy ROs and additional ROs, UE selects the type of ROs based on network indication.

For single RACH configuration, the main motivation is to introduce additional-ROs to improve RACH capacity and reduce PRACH collision probability in legacy-ROs or reduce the random-access latency. If SBFD-aware UE is capable to support PRACH transmission in the additional-ROs, it would be beneficial if UE selects the additional-ROs in the SBFD symbols with the high priority. By using additional-ROs to transmit Msg1 is also beneficial for early indication of the UE SBFD-awareness. Thus, for single RACH configuration, it is good that UE has priority to select additional-ROs in the SBFD symbols.

For two separate RACH configurations, the additional PRACH resource like additional SBFD-ROs or different type of preamble supporting SBFD operation are separately configured. For RO selection, one possible way is based on certain criteria, e.g., the RSRP of the selected SSB for UE evaluation whether the link is good enough. For cell-edge SBFD-aware UE with RSRP below the threshold, UE selecting the additional-RO resource may cause inter-UE CLI due to higher transmission power from cell edge UE. But cell-edge SBFD-aware UE may would like to select long preamble to improve the PRACH cell coverage. However, if RO selection is based on the RSRP, it prohibits the cell-center SBFD-aware UE to do the SBFD-aware indication based on msg1 transmission in the additional-ROs. In that sense, a unified solution is preferred for both RACH configuration optional 1 and option 2, i.e., RO selection based on the prioritization of the additional-ROs over legacy-ROs, if no additional indication received from network.

Furthermore, RAN1 #118 has made the following agreement regarding the PRACH transmission with preamble repetitions.

Agreement For SBFD aware UEs, at least support the following: - PRACH transmission with preamble repetitions within additional-ROs (not across additional-Ros and legacy ROs) Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-ROs and legacy ROs) as in legacy releases.

RAN1 #118bis made further conclusion.

Conclusion PRACH transmission with preamble repetitions across additional-ROs and legacy-ROs is not supported.

It is supported that PRACH transmission with preamble repetitions configured within the additional-ROs, but it is not supported that PRACH transmission with preamble repetitions configured across additional-ROs and legacy-ROs. However, it is unclear whether it is possible the PRACH transmission with preamble repetitions can be configured only one type of ROs (e.g., only on the legacy ROs) or both types of ROs. If only one type of ROs is configured with PRACH transmission with preamble repetitions, the impact on RO selection for RACH initial attempt and RACH fallback should be considered.

Proposal 7: Upon initiation of RACH procedure for a SBFD-aware UE, if no additional indication is from network and PRACH transmission with preamble repetitions is not configured in either type of ROs, the initial attempt in the RACH procedure is determined based on the prioritization of the additional-ROs over legacy-ROs. FFS on that case that PRACH transmission with preamble repetitions is configured.

Since how to configure the PRACH transmission with preamble repetitions in the additional-RO and/or legacy-ROs has impact on the RO selection and RACH fallback, RAN2 should further study this issue.

Proposal 8: RAN2 to study how to configure the PRACH transmission with preamble repetitions in the additional-RO and/or legacy-ROs.

In case of PRACH attempt failures, UE may perform PRACH re-attempt. In RAN2 #127bis, it has been agreed that ‘after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.’

If network indicates the SBFD-aware UE select the legacy RACH occasions first, and after UE keeps attempting on the legacy RO with a number of times and if it is still failed, it means the legacy RACH occasions resource may be congested and overload. UE should be allowed to switch to the SBFD RACH occasions for RACH re-attempt.

Proposal 9: For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in legacy RACH occasions, SBFD-aware UE is allowed to switch to SBFD RACH occasions for RACH re-attempt.

It is important for gNB to know the UE SBFD-aware capability in early stage of the random-access procedure to leverage the SBFD operation. The SBFD-aware indication is useful to inform the UE SBFD capability at the earliest time during RACH, e.g., network may schedule the resource for Msg3 in the SBFD resource in Msg2 if network can be aware the SBFD capable UE performing the RACH.

If network has configured the RACH occasions on SBFD symbols and UE also supports it, the SBFD-aware UE sends the Msg1 in the SBFD RACH occasions as the SBFD-aware indication to network. So, by using the SBFD RACH occasions can be already an implicit way as the SBFD-aware indication.

If the SBFD-aware UE selects the legacy RACH occasion, the additional SBFD-aware indication is needed. If Msg1 based SBFD-aware indication is not supported, Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

Proposal 10: If Msg1 based SBFD-aware indication is not supported, Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

In RACH configuration option 2, when there are legacy-RO in legacy PRACH configuration and additional-RO in the SBFD PRACH configuration that are time overlapping and have same start symbol index in the uplink slot. In such case, if legacy UE selects the legacy RO and SBFD-aware UE selects the additional-RO, the computation from the existing RA-RNTI formulation will be the same. There is ambiguity on the reception of RA response. To resolve this issue, RAN2 to study either update the RA-RNTI formulation or rely on gNB implementation to avoid overlapping of legacy-RO and additional-ROs in the configuration

Proposal 11: RAN2 to study the RA-RNTI collision issue for RACH configuration option 2.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1: For the RRC_CONNECTED UE, RAN1 has agreed to support both CBRA and CFRA in SBFD symbols.

Observation 2: Both RAN2 and RAN1 has agreed to at least support 4-step RACH and FFS on 2-step RACH.

Observation 3: It is not supported to enable both RACH configuration option 1 and option 2 for SBFD RACH operation in a cell from network side.

Proposal 1: RAN2 confirms the working assumption that random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Proposal 2: For a SBFD-aware UE in RRC_CONNECTED state and RRC_IDLE/INACTIVE state, it is up to network to configure the UE with either a single RACH configuration (i.e., RACH configuration Option 1 with Alt 1-1 defined in RAN1) or two separate RACH configurations (i.e., RACH configuration Option 2) for SBFD operation.

Proposal 3: If SBFD-aware UE supports the RACH configuration for SBFD according to UE capability and network configures the RACH resource in SBFD symbols matching the UE capability, UE can use SBFD RACH resource.

Proposal 4: For single RACH configuration (i.e., RACH configuration Option 1 with Alt 1-1 defined in RAN1), whether the configured ROs in SBFD symbols are valid or not is indicated in system information.

Proposal 5: RAN2 waits for RAN1 further inputs on the detailed RACH configuration parameters for SBFD operation.

Proposal 6: Upon initiation of RACH procedure for a SBFD-aware UE, if network provides the indication on the prioritization of the legacy ROs and additional ROs, UE selects the type of ROs based on network indication.

Proposal 7: Upon initiation of RACH procedure for a SBFD-aware UE, if no additional indication is from network and PRACH transmission with preamble repetitions is not configured in either type of ROs, the initial attempt in the RACH procedure is determined based on the prioritization of the additional-ROs over legacy-ROs. FFS on that case that PRACH transmission with preamble repetitions is configured.

Proposal 8: RAN2 to study how to configure the PRACH transmission with preamble repetitions in the additional-RO and/or legacy-ROs.

Proposal 9: For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in legacy RACH occasions, SBFD-aware UE is allowed to switch to SBFD RACH occasions for RACH re-attempt.

Proposal 10: If Msg1 based SBFD-aware indication is not supported, Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

Proposal 11: RAN2 to study the RA-RNTI collision issue for RACH configuration option 2.

# References

- RP‑241614, Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei