3GPP TSG-RAN2 Meeting #128	R2-2410479

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.11.3

Source:	Qualcomm Incorporated

Title:	Other aspects of SBFD

WID/SID:	NR_duplex_evo-Core – Release 19

Document for:	Discussion and Decision

# Introduction

RAN plenary #102 approved the work item on NR duplex operation, Sub-Band Full Duplex (SBFD) and the latest WID is in [1]. The objective of the work item is to specify SBFD operation at the gNB side within a TDD carrier. The objectives include to specify the semi-static indication of time and frequency domain locations of the SBFD subbands to the UE and specify UE transmission, reception and measurement procedures in SBFD and non-SBFD symbols.

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

The other objectives of the work item are to study the enhancements for CLI handling, and RAN2 will focus on the signaling design for UL resource muting for PUSCH, and L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD.

In RAN2 #127 meeting, the following agreements were made.

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed. Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

In RAN2 #127bis meeting, the following agreements were made about other aspects of SBFD

For UL resource muting for PUSCH, the configuration of time and frequency location for UL resource muting should be introduced based on R1 agreement. For L1 based UE-to-UE CLI measurement mechanism, the configuration of periodic, semi-persistent or aperiodic UE-to-UE CLI measurement resource (set) should be introduced based on R1 agreement. For L1 based UE-to-UE CLI reporting mechanism, the configuration of report quantities should be introduced based on R1 agreement.

In this contribution, we would like to discuss the configuration of SBFD time and frequency locations, the potential RAN2 impacts on transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols and enhancement for CLI handling.

# Discussion

RAN1 has agreed to support at least cell-specific configuration of SBFD time and frequency and will further discuss the need for additional UE-specific SBFD time and frequency configuration.

Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

In our understanding, to accommodate SBFD operation for RRC_IDLE/INACTIVE, UE-dedicated frequency configurations are not applicable and cell-specific configuration frequency configuration are needed. For semi-static indication of SBFD subbands time location, it is desirable to configure together with the existing parameter with modification, i.e., TDD-UL-DL-ConfigCommon.

RAN2 #127 meeting agreed that cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signaling to covey cell specific configuration). Regarding the UE-specific configuration, RAN2 can wait for RAN1 further discussion and decision.

Proposal 1: For UE-specific dedicated RRC configuration on SBFD time/frequency configuration, RAN2 wait RAN1 further decision on whether support.

SBFD symbols can be configured in DL and/or FL symbols indicated by TDD-UL-DL-ConfigCommon. The SBFD-aware UE is half duplex UE which can either transmit or receive within the SBFD symbols. UE may utilize the UL resource configured in SBFD symbols to enjoy the benefits of the transmission latency reduction and low UL resource congestion. The resource configuration of the UL physical channels like SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols should be studied.

For example, for PUCCH resource configuration, there could be two options to have PUCCH transmission across SBFD and non-SBFD symbols in different slots. From resource configuration perspective, there could be separate PUCCH resource/set configurations for PUCCH transmission in SBFD and non-SBFD slots, or same PUCCH resource/set configuration for PUCCH transmission across SBFD and non-SBFD slots. The similar principle of the resource configuration (single vs. separate) across SBFD and non-SBFD symbols can be applied to other PHY channels also, like CG-PUSH, SPS-PDSCH, SRS and so on. Different configuration schemes may have impacts on the MAC parameter setting and MAC procedure, like SR, CG, and PHR.

For SBFD-aware UE UL transmission and DL reception across SBFD and non-SBFD symbols, RAN1 is working on the specification for two different configurations. In configuration #1, the transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only. While in configuration 2, the transmissions/receptions can be in SBFD symbols and non-SBFD symbols. Thus, RAN2 should study the MAC impacts on the MAC features with SBFD operations based on RAN1 conclusions.

Proposal 2: RAN2 to study the MAC impacts on the corresponding MAC features like SR and CG with enabling the SBFD operation.

For L1 based UE-to-UE CLI measurement and reporting configuration based on existing CSI framework, the IE CSI-measConfig is used to manages the CSI-RS/CSI-IM resources pool along with the CSI-ReportConfig, and CSI-ResourceConfig defines a group of CSI-RS/CSR-IM resource. Similar to the existing CSI-RS and CSI-IM resource configuration and using the existing CSI framework, at least two new separate CLI resource configurations should be introduced, i.e., SRS-RSRP resource config and CLI RSSI resource config.

Proposal 3: For L1 based UE-to-UE CLI measurement and reporting configuration based on existing CSI framework, two new separate CLI resource configuration IE, i.e., SRS-RSRP resource and CLI-RSSI resource are introduced under CSI-ResourceConfig.

For CLI reporting, several existing CSI reporting configure IEs could be reused for the CLI reporting configuration. RAN1 #118bis made the following agreements.

Agreement In CSI-ReportConfig, reuse resourcesForChannelMeasurement providing the ID of a CSI-ResourceConfig that contains configuration of measurement resources for L1-SRS-RSRP and/or L1-CLI-RSSI.

Proposal 4: For L1 based UE-to-UE CLI reporting configuration, the resourcesForChannelMeasurement is reused for CSI measurement with the associated ID of CLI resource, i.e., SRS-RSRP resource and CLI-RSSI resource.

For L1 based UE-to-UE CLI reporting triggering, RAN1 made following agreements.

Agreement For aperiodic CLI reporting on PUSCH - Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig - Reuse the existing DCI field 'CSI request' to trigger an aperiodic CLI report - Introduce an indication in CSI-AssociatedReportConfigInfo that selects a set from the list of CLI-RSSI measurement resource sets or SRS-RSRP measurement resource sets configured in CSI-ResourceConfig - Note: As in current specifications, the parameter for indication (e.g., resourceSet) is always present, but only used in the case that the CSI-ResourceConfig is configured with multiple sets of aperiodic CLI measurement resources.

According to RAN1 agreements, we have following proposals.

Proposal 5:  For L1 based UE-to-UE CLI reporting configuration, at least AP trigger based CLI reporting is supported.

Proposal 6: For L1 based UE-to-UE CLI reporting configuration, RAN2 waits for RAN1 further conclusion on whether supports periodic based CLI reporting.

For CLI-RSSI resource configuration, Rel-16 has defined CLI-RSSI resource for L3 UE to UE CLI measurement, although the detailed CLI-RSSI resource pattern is discussed in RAN1. But we think reusing the existing Rel-16 contiguous CLI-RSSI pattern, i.e., contiguous frequency resources via start PRB and number of PRBs and time resources via start position and number of symbols can minimize the spec impact and reduce the UE implementation complexity.

Proposal 7: For L1 based UE-to-UE CLI measurement configuration, the configuration of CLI-RSSI resource based on the existing Rel-16 CLI-RSSI resource pattern is supported.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: For UE-specific dedicated RRC configuration on SBFD time/frequency configuration, RAN2 wait RAN1 further decision on whether support.

Proposal 2: RAN2 to study the MAC impacts on the corresponding MAC features like SR and CG with enabling the SBFD operation.

Proposal 3: For L1 based UE-to-UE CLI measurement and reporting configuration based on existing CSI framework, two new separate CLI resource configuration IE, i.e., SRS-RSRP resource and CLI-RSSI resource are introduced under CSI-ResourceConfig.

Proposal 4: For L1 based UE-to-UE CLI reporting configuration, the resourcesForChannelMeasurement is reused for CSI measurement with the associated ID of CLI resource, i.e., SRS-RSRP resource and CLI-RSSI resource.

Proposal 5:  For L1 based UE-to-UE CLI reporting configuration, at least AP trigger based CLI reporting is supported.

Proposal 6: For L1 based UE-to-UE CLI reporting configuration, RAN2 waits for RAN1 further conclusion on whether supports periodic based CLI reporting.

Proposal 7: For L1 based UE-to-UE CLI measurement configuration, the configuration of CLI-RSSI resource based on the existing Rel-16 CLI-RSSI resource pattern is supported.

# References

- RP‑241614, Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei