3GPP TSG-RAN WG2 Meeting #128	R2-2410480

Orlando, USA, 18th – 22nd November, 2024

Agenda Item:	7.3.2

Source:	ZTE Corporation, Sanechips

Title:	Clarification on TN-NTN mobility for IoT NTN

Document for:	Discussion and Decision

Discussion

  - Issue#1: LTE/eMTC TN to NB-IoT NTN mobility

    - Background

In RAN2#127bis meeting, a new requirement about LTE/eMTC TN to NB-IoT NTN mobility has been mentioned in a R18 CR[1]. A new type handheld terminal is mentioned in the CR, e.g., smartphones capable of LTE (i.e., E-UTRA TN) and also NB-IoT NTN, which can work in either legacy LTE network or NB-IoT NTN network. The proponent company also mentions that it is beneficial for such UE to have satellite assistance information to enable seamless switch to NB-IoT NTN from E-UTRA TN and the proposal is that SIB33 (without the -NB suffix) should be allowed to include satellite assistance information for NB-IoT NTN. Therefore, UEs capable of NB-IoT NTN can use the satellite assistance information in SIB33 to search a NB-IoT NTN cell even it currently camps on a LTE network.

The main change in the CR is as following:

– SystemInformationBlockType33 The IE SystemInformationBlockType33 contains satellite assistance information for neighbour cells. When the SystemInformationBlockType33 is signalled in a TN cell, it may contain satellite assistance information for BL UEs, UEs in enhanced coverage, and/or NB-IoT NTN capable UEs.

    - Solutions

Generally, for a certain RAT, e.g., LTE/eMTC, besides the inter-RAT operation related system information (e.g., SIB27), other SIBs are specific to this single RAT. This is also why separate SIB33 and SIB33-NB are defined for eMTC NTN and NB-IoT NTN.

For SIB33, the standard-compliant interpretation should be that this SIB can only contain satellite information related to eMTC NTN, and only UEs capable of eMTC NTN can understand this SIB. Therefore, the simple statement as mentioned in the CR that SIB33 can also include NB-IoT satellite information is not aligned with such principle and cannot provide any real/correct clarification. In other word, either (just based on a simple statement) letting the network to include NB-IoT satellite information in SIB33, or having the UEs interpret the items in SIB33 belonging to NB-IoT NTN, would be just non-standard options.

We think at least there will be some implementation way to fulfil the need of efficient switch to NB-IoT NTN from E-UTRA TN, for example:

- SolutionA: As a dual-mode UE, it anyway can be able to obtain SIB33-NB from the NB-IoT network, which it can store to assist subsequent NB-IoT NTN network search when it camps on LTE/eMTC network.

The proponent companies may think to let UE intermittently acquire SIB33-NB in NB-IoT NTN network is not optimized for the UE. They expect the UE to be always camping in LTE network and able to obtain the NB-IoT satellite information in the camping network, without having to camp on NB-IoT NTN.

With reference to the discussion in WID of LTE TN to NR NTN mobility, some companies think a formal specification change way to fulfil the need is:

- Solution B: To introduce satellite ID to the NB-IoT frequencies in SIB27

However, even a UE capable of this new feature can associate a satellite ID of a NB-IoT satellite with a NB-IoT frequency, the UE camping on LTE TN network still has no way to know the exact NB-IoT satellite information which corresponds to this satellite ID. So it still needs some way to provide NB-IoT satellite information in LTE TN network. One possible thinking is to let network just arbitrarily includes the NB-IoT satellite information in the SIB33 and the new UE only need to pick the satellite information which has the same satellite ID as that of a NB-IoT frequency in SIB27.

However, network CANNOT do like this. Such way would cause problem to legacy LTE/eMTC UEs capable of NTN as those LTE/eMTC UEs may not read SIB27 and have no any clue about a satellite information/satellite ID belonging to a NB-IoT satellite. As a result, these LTE/eMTC UEs will incorrectly handle these NB-IoT satellites as eMTC satellites.

Observation 1: Even satellite ID can be introduced to the NB-IoT frequencies in SIB27, network CANNOT just arbitrarily include the information of pure NB-IoT satellite in the SIB33 for new UE to pick. As the legacy LTE/eMTC UEs may not read SIB27 and have no any clue about a satellite information/satellite ID belonging to a NB-IoT satellite. As a result, the legacy LTE/eMTC UEs will incorrectly handle these NB-IoT satellites as eMTC satellites.

In order to avoid such issue, the following formal ways are figured out for explicitly providing NB-IoT satellite information in LTE TN network. Please note solution C1~C3 can work independently (e.g., UE can measure a NB-IoT neighbour satellite on all the frequencies) or work together with solution B more efficiently (e.g., UE can measure a neighbour NB-IoT satellite on the corresponding frequency indicated in SIB27):

- Solution C1: To schedule the SIB33-NB-like (as SIB33a) in the LTE/eMTC TN network. The possible change is as below:

– SystemInformation The SystemInformation message is used to convey one or more System Information Blocks or Positioning System Information Blocks. All the SIBs or posSIBs included are transmitted with the same periodicity. SystemInformation-BR and SystemInformation-MBMS use the same structure as SystemInformation. Signalling radio bearer: N/A RLC-SAP: TM Logical channels: BCCH and BR-BCCH Direction: E-UTRAN to UE SystemInformation message -- ASN1START SystemInformation-BR-r13 ::= SystemInformation SystemInformation-MBMS-r14 ::= SystemInformation SystemInformation ::= SEQUENCE { criticalExtensions CHOICE { systemInformation-r8 SystemInformation-r8-IEs, criticalExtensionsFuture-r15 CHOICE { posSystemInformation-r15 PosSystemInformation-r15-IEs, criticalExtensionsFuture SEQUENCE {} } } } SystemInformation-r8-IEs ::= SEQUENCE { sib-TypeAndInfo SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { sib2 SystemInformationBlockType2, sib3 SystemInformationBlockType3, sib4 SystemInformationBlockType4, sib5 SystemInformationBlockType5, sib6 SystemInformationBlockType6, sib7 SystemInformationBlockType7, sib8 SystemInformationBlockType8, sib9 SystemInformationBlockType9, sib10 SystemInformationBlockType10, sib11 SystemInformationBlockType11, ..., sib12-v920 SystemInformationBlockType12-r9, sib13-v920 SystemInformationBlockType13-r9, sib14-v1130 SystemInformationBlockType14-r11, sib15-v1130 SystemInformationBlockType15-r11, sib16-v1130 SystemInformationBlockType16-r11, sib17-v1250 SystemInformationBlockType17-r12, sib18-v1250 SystemInformationBlockType18-r12, sib19-v1250 SystemInformationBlockType19-r12, sib20-v1310 SystemInformationBlockType20-r13, sib21-v1430 SystemInformationBlockType21-r14, sib24-v1530 SystemInformationBlockType24-r15, sib25-v1530 SystemInformationBlockType25-r15, sib26-v1530 SystemInformationBlockType26-r15, sib26a-v1610 SystemInformationBlockType26a-r16, sib27-v1610 SystemInformationBlockType27-r16, sib28-v1610 SystemInformationBlockType28-r16, sib29-v1610 SystemInformationBlockType29-r16, sib30-v1700 SystemInformationBlockType30-r17, sib31-v1700 SystemInformationBlockType31-r17, sib32-v1700 SystemInformationBlockType32-r17, sib33-v1800 SystemInformationBlockType33-r18, [[ sib33a-rxx SystemInformationBlockType33-NB-r18 ]] }, nonCriticalExtension SystemInformation-v8a0-IEs OPTIONAL } <skip the unrelated parts> } -- ASN1STOP ---

- Solution C2: To allow SIB33 to include the satellite assistant information of NB-IoT satellite(s). And an indication is introduced for the items in SIB33 to indicate whether a particular satellite assistant information item belongs to LTE/eMTC NTN o NB-IoT NTN. The possible change is as below:

– SystemInformationBlockType33 The IE SystemInformationBlockType33 contains satellite assistance information for neighbour cells. When the SystemInformationBlockType33 is signalled in a TN cell, it may contain satellite assistance information for BL UEs, UEs in enhanced coverage, and/or NB-IoT NTN capable UEs. SystemInformationBlockType33 information element -- ASN1START SystemInformationBlockType33-r18 ::= SEQUENCE { neighSatelliteInfoList-r18 NeighSatelliteInfoList-r18 OPTIONAL, -- Need OR neighValidityDuration-r18 ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40, s45, s50, s55, s60, s120, s180, s240, s900} OPTIONAL, -- Need OP lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ neighSatelliteInfoList-rxx NeighSatelliteInfoList-rxx OPTIONAL, -- Need OR ]] } NeighSatelliteInfoList-r18 ::= SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18 NeighSatelliteInfoList-rxx ::= SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-rxx NeighSatelliteInfo-r18 ::= SEQUENCE { satelliteId-r18 SatelliteId-r18, ephemerisInfo-r18 CHOICE { stateVectors-r18 EphemerisStateVectors-r17, orbitalParameters-r18 EphemerisOrbitalParameters-r17 }, nta-CommonParameters-r18 SEQUENCE { nta-Common-r18 INTEGER (0..8316827) OPTIONAL, -- Need OP nta-CommonDrift-r18 INTEGER (-261935..261935) OPTIONAL, -- Need OP nta-CommonDriftVariation-r18 INTEGER (0..29479) OPTIONAL -- Need OP }, epochTime-r18 SEQUENCE { startSFN-r18 INTEGER (0..1023), startSubFrame-r18 INTEGER (0..9) } OPTIONAL, -- Need OP k-Mac-r18 INTEGER (1..512) OPTIONAL, -- Need OP t-ServiceStartNeigh-r18 TimeOffsetUTC-r17 OPTIONAL -- Need OR } NeighSatelliteInfo-rxx ::= SEQUENCE { satelliteType-rxx ENUMERATED {eMTC, NB-IoT, both, spare1} OPTIONAL, -- Need OR } -- ASN1STOP ---

- Solution C3: To allow SIB33 to include the satellite assistant information of NB-IoT satellite(s). And a new NB-IoT satellites list is introduced to SIB33.

Such way is very similar as that for LTE TN to NR NTN mobility. Although for the case of LTE TN to NB-IoT NTN mobility, there is no such issue that different satellite information structure for eMTC and NB-IoT, we still need to consider the compatibility issues for legacy UEs. And this way seems has minimum impact on legacy UEs. The possible change is as below:

– SystemInformationBlockType33 The IE SystemInformationBlockType33 contains satellite assistance information for neighbour cells. When the SystemInformationBlockType33 is signalled in a TN cell, it may contain satellite assistance information for BL UEs, UEs in enhanced coverage, and/or NB-IoT NTN capable UEs. SystemInformationBlockType33 information element -- ASN1START SystemInformationBlockType33-r18 ::= SEQUENCE { neighSatelliteInfoList-r18 NeighSatelliteInfoList-r18 OPTIONAL, -- Need OR neighValidityDuration-r18 ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40, s45, s50, s55, s60, s120, s180, s240, s900} OPTIONAL, -- Need OP lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ neighSatelliteInfoListNBIoT-rxx NeighSatelliteInfoList-r18 OPTIONAL, -- Need OR ]] } NeighSatelliteInfoList-r18 ::= SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18 NeighSatelliteInfo-r18 ::= SEQUENCE { satelliteId-r18 SatelliteId-r18, ephemerisInfo-r18 CHOICE { stateVectors-r18 EphemerisStateVectors-r17, orbitalParameters-r18 EphemerisOrbitalParameters-r17 }, nta-CommonParameters-r18 SEQUENCE { nta-Common-r18 INTEGER (0..8316827) OPTIONAL, -- Need OP nta-CommonDrift-r18 INTEGER (-261935..261935) OPTIONAL, -- Need OP nta-CommonDriftVariation-r18 INTEGER (0..29479) OPTIONAL -- Need OP }, epochTime-r18 SEQUENCE { startSFN-r18 INTEGER (0..1023), startSubFrame-r18 INTEGER (0..9) } OPTIONAL, -- Need OP k-Mac-r18 INTEGER (1..512) OPTIONAL, -- Need OP t-ServiceStartNeigh-r18 TimeOffsetUTC-r17 OPTIONAL -- Need OR } -- ASN1STOP ---

Comparison:

| Solution | Pros | Cons |
| --- | --- | --- |
| Solution C1 | Seems have no impacts on R18 LTE/eMTC UEs and can allow both SIB33 and SIB33a to be fully configured. | - May have other specification impacts in the procedure text as a new SIB is introduced; - The new SIB will occupy the common channel capacity but can only be used by some (maybe a few) dual-mode UEs in current LTE/eMTC network |
| Solution C2 | Have less specification change | - All the R18 LTE/eMTC UEs need to implement this change. Otherwise, as the R18 LTE/eMTC UEs cannot identify the new indication, they will still incorrectly handle the satellites with “NB-IoT” indication as eMTC satellites. - Only part of SIB33 (some satellites for eMTC NTN while other satellites for NB-IoT NTN) can be used to provide eMTC satellites information |
| Solution C3 | Have less specification change Have no impacts on R18 LTE/eMTC UEs | - If a neighbor satellite is for both eMTC NTN and NB-IoT NTN, its configuration will occur twice in SIB33, which cause signaling inefficiency. - Only part of SIB33 (some satellites for eMTC NTN while other satellites for NB-IoT NTN) can be used to provide eMTC satellites information |

Based on the above analysis, it can be seen the specification impacts are kind of large for supporting this new feature. So the following proposals are given:

Proposal 1a: It’s suggest not to support LTE TN to NB-IoT NTN mobility in R18.

Proposal 1b: If RAN2 can agree to support LTE TN to NB-IoT NTN mobility from R19, RAN2 needs to discuss the following Solution C1~C3, with or without combined with Solution B:

- Solution B: To introduce satellite ID to the NB-IoT frequencies in SIB27

- Solution C1: To schedule the SIB33-NB-like (as SIB33a) in the LTE/eMTC TN network.

- Solution C2: To allow SIB33 to include the satellite assistant information of NB-IoT satellite(s). And an indication is introduced for the items in SIB33 to indicate whether a particular satellite assistant information item belongs to LTE/eMTC NTN o NB-IoT NTN.

- Solution C3: To allow SIB33 to include the satellite assistant information of NB-IoT satellite(s). And a new NB-IoT satellites list is introduced to SIB33.

  - Issue#2: Intra-frequency TN/NTN mobility

    - Discussion

In current spec, there is satellite ID IE in the structure of SIB3. However, it’s unclear whether this satellite ID IE can be configured for intra-frequency neighbor cells. And if yes, what’s the UE behavior when this IE is absent.

According to current RAN4 spec, we understand it’s possible that the operating bands for IoT TN and IoT NTN correspond to a same frequency range. However, the channel numbers are defined completely separated for IoT TN and IoT NTN. That means, IoT TN and IoT NTN would not have overlapped EARFCN. So generally to say, it’s not possible to configure intra-frequency TN/NTN neighbor cells.

So one possible clarification may be to indicate satellite ID IE would not be configured when SIB3 is provided in TN network. However, considering the expandability of specifications, it may be not so necessary to add such restriction. Another way may be to only clarify the UE behavior when this IE is absent. Based on the original meaning of intra-frequency neighbor cell, it can be clarified that without this IE, the cell type of the intra-frequency neighbor cells are same as that of the serving cell.

    - Solutions

Proposal 2: For SIB3, it can be clarified that, when satellite ID is absent, the cell type of the intra-frequency neighbor cells are same as that of the serving cell. The example change is as below:

SystemInformationBlockType3 field descriptions

<skip the unrealted parts>

s-SearchDeltaP Parameter "SSearchDeltaP" in TS 36.304 [4]. This parameter is only applicable for UEs supporting relaxed monitoring as specified in TS 36.306 [5]. Value dB6 corresponds to 6 dB, dB9 corresponds to 9 dB and so on.

satelliteAssistanceInfoList List of satellite ID(s), used to associate with the satellite assistance information for intra-frequency neighbour cell measurements. If the field is not present and SystemInformationBlockType33 is broadcast, the UE considers the intra-frequency neighbour cells to be same as the serving cell, e.g., in an NTN cell, the configured intra-frequency neighbour cells are also NTN cells, and in a TN cell, the configured intra-frequency neighbour cells are also TN cells.

speedStateReselectionPars Speed dependent reselection parameters, see TS 36.304 [4]. If this field is absent, i.e, mobilityStateParameters is also not present, UE behaviour is specified in TS 36.304 [4].

<skip the unrealted parts>

Conclusion

This contribution gives the following proposals:

Proposal 1a: It’s suggest not to support LTE TN to NB-IoT NTN mobility in R18.

Proposal 1b: If RAN2 can agree to support LTE TN to NB-IoT NTN mobility from R19, RAN2 needs to discuss the following Solution C1~C3, with or without combined with Solution B:

- Solution B: To introduce satellite ID to the NB-IoT frequencies in SIB27

- Solution C1: To schedule the SIB33-NB-like (as SIB33a) in the LTE/eMTC TN network.

- Solution C2: To allow SIB33 to include the satellite assistant information of NB-IoT satellite(s). And an indication is introduced for the items in SIB33 to indicate whether a particular satellite assistant information item belongs to LTE/eMTC NTN o NB-IoT NTN.

- Solution C3: To allow SIB33 to include the satellite assistant information of NB-IoT satellite(s). And a new NB-IoT satellites list is introduced to SIB33.

Proposal 2: For SIB3, it can be clarified that, when satellite ID is absent, the cell type of the intra-frequency neighbor cells are same as that of the serving cell. The example change is as below:

SystemInformationBlockType3 field descriptions

<skip the unrealted parts>

s-SearchDeltaP Parameter "SSearchDeltaP" in TS 36.304 [4]. This parameter is only applicable for UEs supporting relaxed monitoring as specified in TS 36.306 [5]. Value dB6 corresponds to 6 dB, dB9 corresponds to 9 dB and so on.

satelliteAssistanceInfoList List of satellite ID(s), used to associate with the satellite assistance information for intra-frequency neighbour cell measurements. If the field is not present and SystemInformationBlockType33 is broadcast, the UE considers the intra-frequency neighbour cells to be same as the serving cell, e.g., in an NTN cell, the configured intra-frequency neighbour cells are also NTN cells, and in a TN cell, the configured intra-frequency neighbour cells are also TN cells.

speedStateReselectionPars Speed dependent reselection parameters, see TS 36.304 [4]. If this field is absent, i.e, mobilityStateParameters is also not present, UE behaviour is specified in TS 36.304 [4].

<skip the unrealted parts>