3GPP TSG-RAN WG2 Meeting #128	R2-2410485

Orlando, Florida, USA, 18th – 22nd November, 2024

Agenda item:	8.8.6

Source:	Samsung

Title:	Further issues on E-UTRAN to NR NTN idle mode mobility

WID/SID:	LTE_TN_NR_NTN_mob-Core, NR_NTN_Ph3

Document for:	Discussion and Decision

# Introduction

In RAN#103 the LTE-part of the WID NR NTN Phase in Rel-19 was agreed [1]:

The work item aims at specifying support for the following objectives: - idle mode mobility based on cell reselection from E-UTRA TN to NR NTN, where E-UTRA TN provides satellite information for NR NTN neighbor cells in a System Information Block. [RAN2]

In RAN#103 the NR-part of the WID NR NTN Phase in Rel-19 was agreed [2]:

4. Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In this contribution, we provide our views on E-UTRAN to NR NTN idle mode mobility as well as some views on regenerative payload for NR NTN.

# Discussion

## Redirection

Re-direction is a key feature of 3GPP cellular networks that allows a network to release a UE to another frequency for the UE to perform idle mode cell selection on that frequency. It is often used to get a UE to connect to another RAT without the need for connected mode handovers, which is often challenging to implement.

In RAN2#127bis, re-direction between TN and NTN was discussed in the context of E-UTRAN to NR NTN mobility. Some companies stated that re-direction cannot be supported without capabilities, while other stated that it shall be considered supported if a UE supports NR NTN. In fact, as far as we understand, re-direction from NR TN to NR NTN was never discussed.

For NTN, connected mode handovers from TN to NTN are supported both for NR NTN and eMTC since Rel-17, although the connected mode measurements does not have any requirements. However, connected mode handovers face difficulties as it relies on either Xn or NG-based handovers. This may be difficult and costly to implement. This means that at least for early mobility and integration between TN and NTN, re-direction is much less costly yet important type of mobility.

Observation 1: Re-direction is a key type of mobility, especially for mobility between RATs.

Observation 2: Re-direction to NTN is an important type of mobility for early TN-NTN networks.

The procedure for re-direction from NR TN to NR NTN as well as E-UTRAN to NR NTN in our view would be as follows for NTN as seen in Figure 1.

Figure 1. Intra-RAT and inter-RAT redirection to NR NTN.

1. eNB/gNB makes a decision to re-direct the UE to NTN. This can for instance be determined based on knowledge of satellite coverage, for instance in a rural network. The network may for instance based on measurement reports determine that both serving and neighbouring TN cells all offer poor coverage, and based upon this the eNB/gNB re-directs the UE.

The network is made aware of the support for re-direction based on basic support of NR NTN, which is indicated capabilities.

2. eNB/gNB sends the RRCRelease/RRCConnectionRelease including the redirectionCarrierInfo along with some assistance information specific to NR.

3. The UE performs cell selection and attempts to find a suitable cell according to the redirectedCarrierInfo. During this process, the UE has to tune to find the NR NTN cells without any NTN assistance information, which involves scanning the band to detect large frequency offsets as seen in NTN.

4. Once the UE has successfully identified a cell and acquired all of the necessary info, the UE camps on the NR NTN cell.

From the above, since the step of performing cell selection on an NTN frequency shall already be supported from Rel-17, the re-direction from both E-UTRAN and NR TN shall also be considered to be supported. The TN network is made aware of the support this by the UE indicating that it supports NTN. This is similar to the Rel-17 assumption that a UE supports connected mode handovers between TN and NTN without any special capabilities. If a UE capability shall be required for this, this should have been made clear in Rel-17.

Proposal 1: E-UTRAN to NR NTN re-direction via RRCConnectionRelease is considered supported if UE supports Rel-17 NTN and no new UE capability is required for this.

Also in RAN2#127bis, it was mentioned by some companies that RRC release and redirection may be enhanced with more assistance information. We believe that RAN2 may discuss providing more assistance information, and if assistance information is introduced, then we certainly believe that a UE capability would be required.

Proposal 2: RAN2 can discuss new assistance information for E-UTRAN to NR NTN redirection, for which a new capability can be considered to be required.

## On polarization information

In RAN2#126, it was agreed that polarization information will be supported to be signalled as part of the NTN assistance information:

4.	To support the idle mode mobility from EUTRA TN to NR NTN, the satellite assistance information for NR NTN neighbor cells is needed and should include the following parameters:

-	Satellite ephemeris information

-	TA common information

-	k-Mac

-	epoch time

-	validity duration

-	ntn-PolarizationDL (FFS if mandatory or optional)

...

6.	The signalling format for ntn-PolarizationDL and TA common related configurations within NTN-Config specified in TS38.331 should be introduced in TS36.331 for NR satellite assistance information.

This follows the design of NR NTN, where the assistance information is signalled per cell. Polarization is often be used in satellite networks to provide Division Multiplexing, similar to FDM and TDM. One popular method is to combine FDM and polarization to achieve cell-resource orthogonality in order to balance to reduce any interference to weak link budgets. For instance with two carrier frequencies and using RHCP and LHCP, 4 orthogonal resources can be created: {freq1, RHCP}, {freq1, LHCP}, {freq2, RHCP} and {freq2, LHCP}.

However, the design in IoT NTN is different where the NTN assistance information is not per cell. The NTN assistance information is valid for multiple cells from a single satellite. And given how It is not likely that all of the cells of a single satellite will use the same polarization mode. The other alternative would be to somehow signal the polarization mode per cell, but SIB24 does not support signalling per-cell information. RAN2 thus needs to discuss how polarization is expected to be used and whether it is needed at all.

Proposal 3: Polarization should not be signalled per satellite in satelliteInfoList-r19.

Proposal 4: RAN2 to discuss how the polarization mode can be signalled per cell or whether it is needed at all.

## Common TA issues related to regenerative payload

Common TA is introduced in Rel-17 as a cell-specific value which covers the RTT between the satellite and the UL synchronization reference point on the feeder link. In regenerative payload scenario which has no feeder link, common TA is not needed, i.e., value 0 should be applied.

The concern raised by some is that UE may over estimate UE specific TA (i.e., RTT of service link), however negative TA value cannot be indicated in RAR, if UE uses common TA value 0, the total TA UE pre-compensated will be larger than the actual TA. In transparent payload scenario, this issue can be solved by configure the common TA with a smaller value than the actual. For regenerative payload, some think negative common TA can be configured.

Regarding the value of common TA in regenerative payload scenario, RAN2 has sent a LS to RAN1 and RAN4. The replies are as follows [4] [5].

RAN1 thanks RAN2 for their questions on common TA in a regenerative payload scenario and would like to provide responses for the question. Question: Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common.. Response: A majority of companies in RAN1 think that it would not be a problem, depending on gNB implementation in a regenerative payload scenario, to stick to 0 as minimum possible value for TA common without introducing the negative value for ta-Common.

RAN4 thanks RAN2 for their enquiry about whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common, and would like to provide the below response to the question asked. [RAN4] No consensus on whether there is an issue due to non-negative value for common TA or whether negative values should be introduced for ta-common. Note: No more discussion of this issue is expected in RAN4.

Majority in RAN1 think there is no issue to have common TA equal to 0, no consensus on whether there is any issue in RAN4. In our view, common TA with value 0 can work without any issue. As specified in 38.211, TA is defined as follows.

Uplink frame number  for transmission from the UE shall start TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc before the start of the corresponding downlink frame at the UE

In TN, UE transmits PRACH without TA pre-compensation. The TA applied on PRACH transmission is based on pre-configured parameter NTA,offset in SIB with NTA is 0, which is not UE specific and cannot be accurate for each UE. In NTN with transparent payload, UE transmits PRACH with TA pre-compensated (including service link RTT NTA,adjUE and common TA NTA,adjcommon). Due to the fast movement of satellite, UE may not apply a precise pre-compensated TA for PRACH transmission. Basically, in both cases TA for PRACH transmission cannot be 100% accurate, and NW receives PRACH with no issue by implementation, for example, certain timing offset and/or time window can be applied by the NW to handle PRACH transmission. Furthermore, NW can adjust the UE specific TA by sending TAC in RAR immediately in Msg2 and TAC MAC CE whenever needed to update NTA. Now in regenerative payload scenario, there should be no issue for NW to receive PRACH similarly. Even if a UE pre-compensates a larger or smaller TA value than the actual value, it does not cause any new issue. PRACH reception and UE specific TA adjustment can work well. Thus, we don’t see any issue to use common TA with value 0.

In contrast, using a negative value of common TA is basically introducing an error in the pre-compensated TA for all UEs in a cell. This will cause more issues, e.g., how to determine the negative value, etc.. What’s worse is that for a UE that can estimate an accurate TA its actual pre-compensated TA will become inaccurate due to this intentionally introduced error. Therefore, we don’t see any benefit of negative common TA.

Proposal 5: In regenerative payload scenario, RAN2 confirms common TA value is 0.

# Conclusion

In this contribution we discussed issues related to regenerative payload for NR NTN.

Proposal 1: E-UTRAN to NR NTN re-direction via RRCConnectionRelease is considered supported if UE supports Rel-17 NTN and no new UE capability is required for this.

Proposal 2: RAN2 can discuss new assistance information for E-UTRAN to NR NTN redirection, for which a new capability can be considered to be required.

Proposal 3: Polarization should not be signalled per satellite in satelliteInfoList-r19.

Proposal 4: RAN2 to discuss how the polarization mode can be signalled per cell or whether it is needed at all.

Proposal 5: In regenerative payload scenario, RAN2 confirms common TA value is 0.

# Reference

RP-240846, New WID: Inter RAT mobility support from E-UTRAN TN to NR-NTN, Thales, CATT, RAN#103, Maastrict, Netherlands, March 2024

RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT, RAN#104, Shanghai, China, June 2024.

R2-2406636, Discussion on regenerative payload, Qualcomm Incorporated, RAN2#127, Maastricht, Netherlands, August 2024.

R2-2409512, Reply LS on common TA in a regenerative payload scenario

R2-2409522, Reply LS on common TA in a regenerative payload scenario