3GPP TSG-RAN WG2 Meeting #128	R2-2410500

# Orlando, FL, USA, 18th – 22nd	November 2024

Agenda Item:	8.7.4.1

Source:	Ericsson

Title:	LCP enhancements

Document for:	Discussion, Decision

# 1	Introduction

At RAN#105 the XR WID [1] for Rel-19 was updated with the following objective on the scheduling enhancements related to LCP:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

In this contribution we continue the discussion on remaining details of the delay-aware LCP solution. Furthermore, we provide a TP for how the enhanced LCP mechanism can be modelled.

# 2	Discussion on LCP enhancements

## 2.1 Delay-aware LCP enhancement

It has been agreed to introduce an independent per-LCH remaining time threshold for applying adjusted priority. It remains to agree on when the priority for an LCH returns to the default priority. At the last meeting it was discussed whether the additional priority is applied in both 1st and 2nd round of resource allocation procedure in LCP. We think the additional LCH priority should only be applied as long as there is LCH adjusted-priority data in an LCH. This would assure that only the LCH adjusted-priority data get the benefit of adjusted priority and results in a fairer LCP procedure. It is also the most logical behaviour without any obvious downsides on either UE complexity or LCP timeline. It was claimed that this would require cross-layer checks during the procedure but that is not needed. The amount of LCH priority-adjusted data is known already when starting the LCP procedure (this is a fast procedure and the initial value can be assumed to be correct during the entire procedure) and thus no complicated operation is needed during the ongoing procedure. The procedure will basically boil down to keeping check of how much of the LCH priority-adjusted data that is left in the LCH (something that anyway needs to be done) and change the priority of the LCH when there is no more of the LCH priority-adjusted data left. This change in priority should consist of a simple operation.

Additional LCH priority is only applied as long as there is LCH priority-adjusted data in the LCH.

To keep track of the transmission of the LCH priority-adjusted data a new counter, Rj, can be applied. This counter can be configured so that only when all of the LCH priority-adjusted data is transmitted from the LCH the resource allocation moves on to another lower priority LCH. This solution assures that the time critical data is being included in the transmission before any other low important data. An example of how this new counter Rj is tracking the data in the LCH is shown in Figure 1.

Figure 1: Examples of how the counter Rj can be calculated for two different LCHs.

We thus propose for the delay-aware LCP enhancements:

A delay-aware LCP solution uses a new counter variable to keep track of the scheduling of the LCH priority-adjusted data.

The described delay-aware LCP solution is simple to implement and understand in the specification. It ensures a fair LCP procedure while at the same time enabling that the LCH priority-adjusted data is transmitted before other data when possible. A text proposal implementing the delay-aware LCP solution using the new counter Rj can be found in Appendix 1.

Implement the TP provided in Appendix 1.

Please note that the text proposal is written as if there are only algorithms supporting delay-aware LCP. When RAN2 has agreed on a solution for the enhanced LCP the text should reflect that there will be two variants of LCP and which one to use will be determined by configuration.

## 2.2 Configuration of enhanced LCP

Each LCH that acts on LCH priority-adjusted data will need to be configured via RRC with an alternative priority value and a threshold for remaining time. This can be done in the same IE that is used for LCP configuration today, LogicalChannelConfig.

For each LCH that contains time critical data, it shall be possible to configure via RRC an alternative priority value and a remaining time threshold in the IE LogicalChannelConfig.

Some data, e.g. pose and signallling should always have higher priority than the LCH priority-adjusted data, even if the time critical data is close to be discarded. This can be solved by the network via configuration of the default priority levels and the alternative priority levels. Since we can’t predict exactly which data that need to be prioritized we shouldn’t specify how these priority levels will be configured, it can be left for network implementation.

How to configure priority values so that e.g. pose and signalling always have higher priority than LCH priority-adjusted data is left for network implementation.

# 4	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Additional LCH priority is only applied as long as there is LCH priority-adjusted data in the LCH.

Proposal 2	A delay-aware LCP solution uses a new counter variable to keep track of the scheduling of the LCH priority-adjusted data.

Proposal 3	Implement the TP provided in Appendix 1.

Proposal 4	For each LCH that contains time critical data, it shall be possible to configure via RRC an alternative priority value and a remaining time threshold in the IE LogicalChannelConfig.

Proposal 5	How to configure priority values so that e.g. pose and signalling always have higher priority than LCH priority-adjusted data is left for network implementation.

# References

RP-241771 Revised WID: XR (eXtended Reality) for NR Phase 3, September 2024

# Appendix 1

Changes to specification TS 38.321, additions highlighted in yellow:

### 5.4.3	Multiplexing and assembly

#### 5.4.3.1	Logical Channel Prioritization

##### 5.4.3.1.1	General

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

-	timeCriticalPriority an alternative priority value.

-	timeCriticalTimeThreshold which set the remainingTimeThreshold (RETT).

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission;

-	allowedCG-List which sets the allowed configured grant(s) for transmission;

-	allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission;

-	allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

-	Rj which is maintained for each logical channel j.

The MAC entity shall initialize Bj and Rj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

1>	if the remaining time until discard for any MAC SDU in logical channel j is less than or equal to RETT:

2>	set Rj to the size in bytes for MAC SDUs with remaining time less or equal to RETT.

2>	adjust priority of logical channel j to timeCriticalPriority

- if the remaining time until discard for all MAC SDU in logical channel j is greater than RETT:

2>	set Rj to zero.

2>	adjust priority of logical channel j to priority.

NOTE:	The exact moment(s) when the UE updates Bj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP.

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

1>	select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to true in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within the same MAC entity (i.e. CA duplication) when CA duplication is deactivated for this DRB in this MAC entity; and

2>	allowedCG-List, if configured, includes the configured grant index associated to the UL grant; and

2>	allowedPHY-PriorityIndex, if configured, includes the priority index (as specified in clause 9 of TS 38.213 [6]) associated to the dynamic UL grant; and

2>	allowedHARQ-mode, if configured, includes the allowed UL HARQ mode for the HARQ process associated to the UL grant.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration, Cell information, and priority index are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 or Rj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2> decrement Rj by the total size of MAC SDUs with remaining time less or equal to RETT served to logical channel j above;

2>	if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE 1:	The value of Bj can be negative.