3GPP TSG-RAN WG2 Meeting #128	R2-2410507

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	8.3.2.1

Source:	InterDigital Inc.

Title:	Simulation results for RRM measurement predictions

Document for:	Discussion

# 1. Introduction

In RAN2-126 and RAN2-127, several agreements were made regarding the simulation assumption for the RRM measurement predictions [1]:

Table 1: Evaluation Scenario Combinations

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In addition, three RRM prediction sub-cases were introduced [2]:

- Sub-case 1: To predict beam level results, then generate cell level results based on the predicted beam results

- Sub-case 2: To directly predict cell level results based on cell level results

- Sub-case 3: To directly predict cell level results based on beam level results

In previous contribution [3], we provided simulation results for the highlighted scenarios in the table above (Case 2, Case 3, and Case 4), focusing on Sub-case 1 and Sub-case 3 as indirect and direct approaches for predicting L3 cell-level measurements. In this contribution, we provide additional simulation result that includes Sub-case 2, and also some additional results at higher speed for Case 4.

# 2. Discussion

### 2.1 Intra-cell intra-frequency FR1 to FR1 temporal domain case B

To generate data, we consider a sampling period of 40 ms for L1 beam-level RSRPs. Then, non-sliding L1 filtering is applied to obtain L1 filtered beam-level RSRPs. With L1 filtering window length that is equal to 5, the sampling period of non-sliding L1 filtered beam-level RSRPs, and consequently L3 cell-level RSRPs is 200 ms. It is worth noting that L1 filtering is a simple averaging process of the input L1 beam-level RSRPs.

Figure 1: Non-sliding L1/L3 Filtering

We study 50% measurement reduction rate. For predicting cell-level or beam-level RSRPs at time instance T, we use input from 3 previous time instances, which are T-5, T-3, and T-1. It is necessary to use input from multiple time instances to enable the AIML model to learn information about UE trajectory. The RSRPs at time instances T-5, T-3, and T-1 are all measured RSRPs.

Figure 2: Temporal-domain Case B Observation Windows (OWs in yellow) and Prediction Windows (PW in green) Pattern with 50% Measurement Reduction Rate

In Case 2, we consider predicting L3 cell-level RSRPs directly and indirectly according to Sub-cases 1 and 3 where:

- Sub-case 1 (Beam-to-cell Indirect Approach): L1 filtered beam-level RSRPs from 3 time instances (T-5, T-3, and T-1) are input to AIML model to predict L1 filtered beam-level RSRPs at time instance T. Then, L3 cell-level RSRP at time instance T is derived using the L1 beam-level RSRPs at time instance T that were obtained from the AIML model.

- Sub-case 2 (Cell-to-cell Direct Approach): L3 cell-level RSRPs from 3 time instances (T-5, T-3, and T-1) are input to AIML model to predict L3 cell-level RSRP at time instance T directly.

- Sub-case 3 (Beam-to-cell Direct Approach): L1 filtered beam-level RSRPs from 3 time instances (T-5, T-3, and T-1) are input to AIML model to predict L3 cell-level RSRP at time instance T directly.

Simulation parameters of Case 2 are listed in Table A.1 [2]-[5]. Evaluation results are shown in Tables 2 and 3.

Table 2: Evaluation results for intra-cell intra-frequency FR1 to FR1 temporal domain case B (UE Speed = 30 Km/h)

| Parameters | Sub-case 1 (Beam-to-cell Indirect) | Sub-case 2 (Cell-to-cell Direct) | Sub-case 3 (Beam-to-cell Direct) |
| --- | --- | --- | --- |
| UE trajectory option [2] | Option 1 | Option 1 | Option 1 |
| UE trajectory boundary processing option [2] | Option 3 | Option 3 | Option 3 |
| UE speed | 30 Km/h | 30 Km/h | 30 Km/h |
| Inter-frequency correlation assumption (yes or no) | N/A | N/A | N/A |
| Measurement reduction rate | 50% | 50% | 50% |
| Observation window | 1000 ms | 1000 ms | 1000 ms |
| Prediction window | 200 ms | 200 ms | 200 ms |
| Any other parameters | N/A | N/A | N/A |
| Number of training examples | 14700 | 14700 | 14700 |
| Number of validation examples | 2100 | 2100 | 2100 |
| Number of testing examples | 4200 | 4200 | 4200 |
| AIML model input | L1 filtered beam-level RSRPs from 3 time instances T-5, T-3, T-1 | L3 cell-level RSRPs from 3 time instances T-5, T-3, T-1 | L1 filtered beam-level RSRPs from 3 time instances T-5, T-3, T-1 |
| AIML model output | L1 filtered beam-level RSRPs at 1 time instance T (then L3 cell-level RSRP is evaluated) | L3 cell-level RSRPs at 1 time instance T | L3 cell-level RSRPs at 1 time instance T |
| AIML model type | ResNet | ResNet | ResNet |
| AIML model number of parameters | 1.51 M | 1.51 M | 1.51 M |
| AIML-based Average L3 cell-level RSRP difference (dB) | 0.0149 dB | 0.0187 dB | 0.0199 dB |
| AIML-based Average L1 beam-level RSRP difference (dB) | 0.0355 dB | N/A | N/A |
| Baseline (Sample-and-Hold) Average L3 cell-level RSRP difference (dB) | 0.1119 dB | 0.1119 dB | 0.1119 dB |

Table 3: Evaluation results for intra-cell intra-frequency FR1 to FR1 temporal domain case B (UE Speed = 60 kmh)

| Parameters | Sub-case 1 (Beam-to-cell Indirect) | Sub-case 2 (Cell-to-cell Direct) | Sub-case 3 (Beam-to-cell Direct) |
| --- | --- | --- | --- |
| UE trajectory option [2] | Option 1 | Option 1 | Option 1 |
| UE trajectory boundary processing option [2] | Option 3 | Option 3 | Option 3 |
| UE speed | 60 Km/h | 60 Km/h | 60 Km/h |
| Inter-frequency correlation assumption (yes or no) | N/A | N/A | N/A |
| Measurement reduction rate | 50% | 50% | 50% |
| Observation window | 1000 ms | 1000 ms | 1000 ms |
| Prediction window | 200 ms | 200 ms | 200 ms |
| Any other parameters | N/A | N/A | N/A |
| Number of training examples | 14669 | 14669 | 14669 |
| Number of validation examples | 2095 | 2095 | 2095 |
| Number of testing examples | 4190 | 4190 | 4190 |
| AIML model input | L1 filtered beam-level RSRPs from 3 time instances T-5, T-3, T-1 | L3 cell-level RSRPs from 3 time instances T-5, T-3, T-1 | L1 filtered beam-level RSRPs from 3 time instances T-5, T-3, T-1 |
| AIML model output | L1 beam-level RSRPs at 1 time instance T (then L3 cell-level RSRP is evaluated) | L3 cell-level RSRPs at 1 time instance T | L3 cell-level RSRPs at 1 time instance T |
| AIML model type | ResNet | ResNet | ResNet |
| AIML model number of parameters | 1.51 M | 1.51 M | 1.51 M |
| AIML model number of FLOPs | 2.05 M | 2.05 M | 2.05 M |
| AIML-based Average L3 cell-level RSRP difference (dB) | 0.0346 dB | 0.0552 dB | 0.0408 dB |
| AIML-based Average L1 beam-level RSRP difference (dB) | 0.0791 dB | N/A | N/A |
| Baseline (Sample-and-Hold) Average L3 cell-level RSRP difference (dB) | 0.2263 dB | 0.2263 dB | 0.2263 dB |

We notice that indirect L3 cell-level RSRP prediction via an AIML model that predicts L1 beam-level RSRPs (i.e., Sub-case 1) achieves a better performance than direct prediction (i.e., Sub-case 2 and Sub-case 3). This is because the direct AIML model is required to predict a filtered L3 cell-level measurement given partial previous history (i.e., information from only 3 previous time instances). The performance of the direct model may be enhanced by providing either L1 beam-level RSRP input from more previous instances or previous measured and/or predicted L3 cell-level measurements.

We also notice that the AIML model achieve a lower prediction error as the UE speed becomes lower. In addition, AIML-based prediction outperforms baseline (sample-and-hold), in which L1-filtered beam-level RSRPs from latest measurement time instance before prediction are maintained as predicted measurements, by achieving less average prediction error.

Observation 1: For temporal domain case B, as UE speed increases, the average L3 cell-level RSRP difference increases, i.e., AIML model prediction accuracy decreases for both direct and indirect prediction.

Observation 2: For temporal domain case B, the indirect prediction approach (Sub-case 1) slightly outperforms the direct prediction approaches (Sub-Cases 2 and 3) when given the same input (i.e., only L1 filtered beam-level RSRPs) since the direct approach tries to predict a value that depends on a long history (due to L3 filtering impact) using partial information on that history (i.e., a few time instances).

Observation 3: For temporal domain case B, AIML model prediction accuracy is higher than baseline (sample-and-hold)

### 2.2 Intra-cell intra-frequency FR2 to FR2 temporal domain case A

To generate data, we consider a sampling period of 20 ms for L1 beam-level RSRPs. To measure RSRPs for 4 Rx beams, a measurement period of 80 ms (i.e., 4 sampling periods) is required. Then, sliding L1 filtering (with window length that is equal to 5) is applied to obtain L1 filtered beam-level RSRPs., the measurement period of sliding L1 filtered beam-level RSRPs, and consequently L3 cell-level RSRPs is 80 ms. It is worth noting that L1 filtering is a simple averaging process of the input L1 beam-level RSRPs.

Figure 3: Sliding L1/L3 Filtering

In Case 4, we consider predicting L3 cell-level RSRPs directly and indirectly according to Sub-cases 1 and 3 where:

- Sub-case 1 (Beam-to-cell Indirect Approach): L1 filtered beam-level RSRPs from OW time instances are input to AIML model to predict L1 filtered beam-level RSRPs of PW time instances. Then, L3 cell-level RSRP of PW time instances are evaluated using the L1 filtered beam-level RSRPs of PW time instances that were obtained from the AIML model.

- Sub-case 2 (Cell-to-cell Direct Approach): L3 cell-level RSRPs from OW time instances are input to AIML model to predict L3 cell-level RSRP of PW time instances directly.

- Sub-case 3 (Beam-to-cell Direct Approach): L1 filtered beam-level RSRPs from OW time instances are input to AIML model to predict L3 cell-level RSRP of PW time instances directly.

Figure 4: Temporal-domain Case A Observation Window (OW) and Prediction Window (PW) Pattern

Simulation parameters of Case 4 are listed in Table A.2 [2]-[5]. Evaluation results are shown in Table 4 and following figures.

Table 4: Evaluation results for intra-cell intra-frequency FR2 to FR2 temporal domain case A

| Parameters | Sub-case 1 (Beam-to-cell Indirect) | Sub-case 2 (Cell-to-cell Direct) | Sub-case 3 (Beam-to-cell Direct) |
| --- | --- | --- | --- |
| UE trajectory option [2] | Option 1 | Option 1 | Option 1 |
| UE trajectory boundary processing option [2] | Option 3 | Option 3 | Option 3 |
| UE speed | 60 Km/h, 120 Km/h | 60 Km/h, 120 Km/h | 60 Km/h, 120 Km/h |
| Inter-frequency correlation assumption (yes or no) | N/A | N/A | N/A |
| Measurement reduction rate | N/A | N/A | N/A |
| Observation window (OW) | Refer to Tables 5 – 9 | Refer to Tables 5 – 9 | Refer to Tables 5 – 9 |
| Prediction window (PW) | Refer to Tables 5 – 9 | Refer to Tables 5 – 9 | Refer to Tables 5 – 9 |
| Any other parameters | N/A | N/A | N/A |
| Number of training examples | 14581, 14077 | 14581, 14077 | 14581, 14077 |
| Number of validation examples | 2083, 2011 | 2083, 2011 | 2083, 2011 |
| Number of testing examples | 4164, 4022 | 4164, 4022 | 4164, 4022 |
| AIML model input | L1 filtered beam-level RSRPs from OW time instances | L3 Cell-level RSRPs of OW time instances | L1 filtered beam-level RSRPs from OW time instances |
| AIML model output | L1 filtered beam-level RSRPs of PW time instances | L3 Cell-level RSRPs of PW time instances | L3 Cell-level RSRPs of PW time instances |
| AIML model type | ResNet | ResNet | ResNet |
| AIML model complexity | Refer to Table 5 | Refer to Table 5 | Refer to Table 5 |
| AIML-based Average L3 cell-level RSRP difference (dB) | Refer to Tables 6, 7 and Figures | Refer to Tables 6, 7 and Figures | Refer to Tables 6, 7 and Figures |
| AIML-based Average L1 beam-level RSRP difference (dB) | Refer to Table 8 | N/A | N/A |
| Baseline (Sample-and-Hold) Average L3 cell-level RSRP difference (dB) | Refer to Table 9 and Figures | Refer to Table 9 and Figures | Refer to Table 9 and Figures |

Table 5: AIML model complexity summary for intra-cell intra-frequency FR2 to FR2 temporal domain case A

| Sub-case | OW, PW (ms) | AIML model number of parameters | AIML model number of FLOPs |
| --- | --- | --- | --- |
| 1 | 400, 400 | 0.413 M | 25.96 M |
| 1 | 400, 800 | 0.415 M | 26.04 M |
| 1 | 800, 400 | 0.416 M | 26.14 M |
| 1 | 800, 800 | 0.417 M | 26.22 M |
| 2 | 400, 400 | 0.575 M | 25.96 M |
| 2 | 400, 800 | 0.586 M | 21.58 M |
| 2 | 800, 400 | 0.658 M | 26.14 M |
| 2 | 800, 800 | 0.598 M | 17.31 M |
| 3 | 400, 400 | 0.575 M | 25.96 M |
| 3 | 400, 800 | 0.656 M | 26.04 M |
| 3 | 800, 400 | 0.658 M | 26.14 M |
| 3 | 800, 800 | 0.669 M | 21.77 M |

Table 6: AIML model prediction accuracy results for intra-cell intra-frequency FR2 to FR2 temporal domain case A (UE Speed = 60 Km/h)

| Sub-case | OW, PW (ms) | OW, PW (Measurement instances) | AIML-based Average L3 cell-level RSRP difference for all PW instances (dB) | AIML-based Average L3 cell-level RSRP difference at the end of PW (dB) |
| --- | --- | --- | --- | --- |
| 1 | 400, 400 | 5, 5 | [0.010, 0.018, 0.035, 0.064, 0.109] | 0.109 |
| 1 | 400, 800 | 5, 10 | [0.035, 0.045, 0.060, 0.082, 0.116, 0.162, 0.223, 0.297, 0.387, 0.492] | 0.492 |
| 1 | 800, 400 | 10, 5 | [0.018, 0.024, 0.041, 0.072, 0.120] | 0.120 |
| 1 | 800, 800 | 10, 10 | [0.023, 0.027, 0.039, 0.062, 0.098, 0.147, 0.208, 0.283, 0.372, 0.470] | 0.470 |
| 2 | 400, 400 | 5, 5 | [0.056, 0.165, 0.305, 0.469, 0.645] | 0.645 |
| 2 | 400, 800 | 5, 10 | [0.058, 0.166, 0.308, 0.472, 0.650, 0.834, 1.012, 1.194, 1.381, 1.573] | 1.573 |
| 2 | 800, 400 | 10, 5 | [0.057, 0.163, 0.296, 0.449, 0.618] | 0.618 |
| 2 | 800, 800 | 10, 10 | [0.061, 0.169, 0.305, 0.459, 0.627, 0.798, 0.971, 1.146, 1.324, 1.505] | 1.505 |
| 3 | 400, 400 | 5, 5 | [0.341, 0.357, 0.387, 0.440, 0.535] | 0.535 |
| 3 | 400, 800 | 5, 10 | [0.364. 0.377, 0.404, 0.440, 0.494, 0.561, 0.638, 0.724, 0.828, 0.953] | 0.953 |
| 3 | 800, 400 | 10, 5 | [0.387, 0.415, 0.459, 0.527, 0.628] | 0.628 |
| 3 | 800, 800 | 10, 10 | [0.379, 0.400, 0.430, 0.476, 0.538, 0.611, 0.696, 0.794, 0.909, 1.041] | 1.041 |

Table 7: AIML model prediction accuracy results for intra-cell intra-frequency FR2 to FR2 temporal domain case A (UE Speed = 120 Km/h)

| Sub-case | OW, PW (ms) | OW, PW (Measurement instances) | AIML-based Average L3 cell-level RSRP difference for all PW instances (dB) | AIML-based Average L3 cell-level RSRP difference at the end of PW (dB) |
| --- | --- | --- | --- | --- |
| 1 | 400, 400 | 5, 5 | [0.035, 0.079, 0.158, 0.271, 0.422] | 0.422 |
| 1 | 400, 800 | 5, 10 | [0.104, 0.149, 0.224, 0.328, 0.467, 0.642, 0.857, 1.109, 1.394, 1.703] | 1.703 |
| 1 | 800, 400 | 10, 5 | [0.043, 0.087, 0.164, 0.282, 0.450] | 0.450 |
| 1 | 800, 800 | 10, 10 | [0.084, 0.098, 0.143, 0.231, 0.360, 0.527, 0.732, 0.970, 1.239, 1.542] | 1.542 |
| 2 | 400, 400 | 5, 5 | [0.158, 0.410, 0.704, 1.018, 1.346] | 1.346 |
| 2 | 400, 800 | 5, 10 | [0.160, 0.411, 0.703, 1.013, 1.336, 1.676, 2.031, 2.399, 2.781, 3.182] | 3.182 |
| 2 | 800, 400 | 10, 5 | [0.143, 0.324, 0.568, 0.874, 1.254] | 1.254 |
| 2 | 800, 800 | 10, 10 | [0.182, 0.320, 0.508, 0.770, 1.085, 1.416, 1.761, 2.122, 2.493, 2.881] | 2.881 |
| 3 | 400, 400 | 5, 5 | [0.391, 0.432, 0.527, 0.663, 0.855] | 0.855 |
| 3 | 400, 800 | 5, 10 | [0.412, 0.454, 0.545, 0.665, 0.821, 1.018, 1.245, 1.494, 1.748, 2.025] | 2.025 |
| 3 | 800, 400 | 10, 5 | [0.475, 0.529, 0.637, 0.786, 0.984] | 0.984 |
| 3 | 800, 800 | 10, 10 | [0.444, 0.490, 0.562, 0.682, 0.843, 1.035, 1.247, 1.494, 1.760, 2.038] | 2.038 |

Table 8: AIML model beam-level prediction accuracy results for intra-cell intra-frequency FR2 to FR2 temporal domain case A, Sub-case 1

| UE Speed (Km/h) | OW, PW (ms) | OW, PW (Measurement periods) | AIML-based Average L1 beam-level RSRP difference for all PW instances (dB) | AIML-based Average L1 beam-level RSRP difference at the end of PW (dB) |
| --- | --- | --- | --- | --- |
| 60 | 400, 400 | 5, 5 | [0.044, 0.102, 0.192, 0.314, 0.476] | 0.476 |
| 60 | 400, 800 | 5, 10 | [0.076, 0.131, 0.217, 0.332, 0.474, 0.634, 0.811, 1.001, 1.204, 1.427] | 1.427 |
| 60 | 800, 400 | 10, 5 | [0.047, 0.105, 0.197, 0.330, 0.504] | 0.504 |
| 60 | 800, 800 | 10, 10 | [0.061, 0.107, 0.182, 0.290, 0.430, 0.588, 0.763, 0.952, 1.155, 1.378] | 1.378 |
| 120 | 400, 400 | 5, 5 | [0.157, 0.341, 0.581, 0.882, 1.252] | 1.252 |
| 120 | 400, 800 | 5, 10 | [0.231, 0.426, 0.662, 0.945, 1.282, 1.635, 2.005, 2.389, 2.781, 3.177] | 3.177 |
| 120 | 800, 400 | 10, 5 | [0.143, 0.324, 0.568, 0.874, 1.254] | 1.254 |
| 120 | 800, 800 | 10, 10 | [0.182, 0.320, 0.508, 0.770, 1.085, 1.416, 1.761, 2.122, 2.493, 2.881] | 2.881 |

Table 9: AIML-based vs. baseline (Sample-and-Hold) prediction accuracy results for intra-cell intra-frequency FR2 to FR2 temporal domain case A

| Sub-case | UE Speed (Km/h) | OW, PW (ms) | OW, PW (Measurement periods) | AIML-based Average L3 cell-level RSRP difference (dB) | Baseline (Sample-and-Hold) Average L3 cell-level RSRP difference (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 60 | 800, 800 | 10, 10 | [0.023, 0.027, 0.039, 0.062, 0.098, 0.147, 0.208, 0.283, 0.372, 0.470] | [0.189, 0.461, 0.765, 1.069, 1.373, 1.672, 1.971, 2.271, 2.578, 2.887] |
| 2 | 60 | 800, 800 | 10, 10 | [0.379, 0.400, 0.430, 0.476, 0.538, 0.611, 0.696, 0.794, 0.909, 1.041] |  |
| 3 | 60 | 800, 800 | 10, 10 | [0.379, 0.400, 0.430, 0.476, 0.538, 0.611, 0.696, 0.794, 0.909, 1.041] |  |
| 1 | 120 | 800, 800 | 10, 10 | [0.084, 0.098, 0.143, 0.231, 0.360, 0.527, 0.732, 0.970, 1.239, 1.542] | [0.650, 1.287, 1.912, 2.526, 3.133, 3.737, 4.345, 4.960, 5.579, 6.212] |
| 2 | 120 | 800, 800 | 10, 10 | [0.182, 0.320, 0.508, 0.770, 1.085, 1.416, 1.761, 2.122, 2.493, 2.881] |  |
| 3 | 120 | 800, 800 | 10, 10 | [0.444, 0.490, 0.562, 0.682, 0.843, 1.035, 1.247, 1.494, 1.760, 2.038] |  |

Figure 5: AIML model prediction accuracy results for intra-cell intra-frequency FR2 to FR2 temporal domain case A (UE Speed = 60 Km/h) [x-axis represents PW instances]

Figure 6: AIML model prediction accuracy results for intra-cell intra-frequency FR2 to FR2 temporal domain case A (UE Speed = 120 Km/h) [x-axis represents PW instances]

We notice that indirect L3 cell-level RSRP prediction via an AIML model that predicts L1 beam-level RSRPs (i.e., Sub-case 1) achieves a better performance than direct prediction (i.e., Sub-cases 2 and 3). This is because the direct AIML model is required to learn both measurement prediction and selection/consolidation to predict a filtered L3 cell-level measurement which may be challenging for AIML regression models. We also notice that sub-case 2 achieves lower prediction error at earlier PW instances than sub-case 3, however, sub-case 3 has a lower prediction error on the long term.

We also notice that the AIML model achieve a lower prediction error as the UE speed becomes lower. In addition, AIML-based direct and indirect prediction outperform baseline (sample-and-hold), in which L1-filtered beam-level RSRPs from latest measurement time instance before prediction are maintained as predicted measurements, by achieving less average prediction error.

To study the impact of changing observation window, we compare the AIML-based prediction for OW and PW pairs (400 ms, 400 ms) and (800 ms, 800ms) since both have 1:1 ratio between OW and PW with different window lengths. Using shorter OW at 60 Km/h, with 1:1 ratio between OW and PW, results in better prediction performance since it seems PW measurements can be better inferred from more recent history at such UE speed, i.e., older OW instances do not help enhance the prediction performance.

We can also compare the AIML-based prediction for OW and PW pairs (400 ms, 800 ms) and (800 ms, 800ms) since both have same PW length, but with 1:2 and 1:1 ratio between OW and PW, respectively. We notice that the performance gap between both case is small (0.22 dB, 0.68 dB and 0.87 dB for Sub-cases 1, 2, and 3, respectively at 60 Km/h and 0.16 dB, 0.3 dB, 0.13 dB for Sub-cases 1, 2, and 3, respectively at 120 Km/h) which indicates that AIML-based prediction did not gain much advantage by having a longer OW for a constant PW.

To study the impact of changing prediction window, we compare the AIML-based prediction for OW and PW pairs (400 ms, 400 ms) and (400 ms, 800ms) since both have same OW length, but with 1:1 and 1:2 ratio between OW and PW, respectively. Increasing the length of PW results in a minor increase in prediction error (0.3 dB – 0.9 dB at 60 Km/h), however, it results in less measurement overhead. It is expected that the prediction error gap shall increase with higher UE speeds which can be noticed at UE speed of 120 Km/h where increasing PW length results in larger prediction errors (> 1 dB).

Observation 4: For temporal domain case A, the indirect prediction approach (Sub-case 1) outperforms the direct prediction approaches (Sub-cases 2 and 3) when given the same input (i.e., only L1 filtered beam-level RSRPs) since the direct approach tries to predict a value that depends on a long history (due to L3 filtering impact) using partial information on that history (i.e., a few time instances).

Observation 5: For temporal domain case A, Sub-case 2 outperforms Sub-case 3 at earlier instances within the PW, but Sub-case 3 outperforms Sub-case 2 at later instances within the PW.

Observation 6: For temporal domain case A, as UE speed increases, the average L3 cell-level RSRP difference increases, i.e., AIML model prediction accuracy decreases for both direct and indirect prediction.

Observation 7: For temporal domain case A, AIML model prediction accuracy is higher than baseline (sample-and-hold)

Observation 8: For temporal domain case A, using shorter OW for a fixed ratio between OW and PW results in better prediction performance.

Observation 9: For temporal domain case A, longer OW may only result in minor prediction performance gain (for a given PW).

Observation 10: For temporal domain case A, at 60 Km/h, increasing the length of PW results in a minor increase in prediction error (0.3 dB – 0.9 dB), however, it results in less measurement overhead.

Observation 11: For temporal domain case A, at 120 Km/h, increasing the length of PW results in a significant increase in prediction error (> 1 dB).

### 2.3 Inter-cell inter-frequency FR1 to FR1 frequency domain

To generate data, we consider a sampling period of 40 ms for L1 beam-level RSRPs. Then, non-sliding L1 filtering is applied to obtain L1 filtered beam-level RSRPs. With L1 filtering window length that is equal to 5, the sampling period of non-sliding L1 filtered beam-level RSRPs, and consequently L3 cell-level RSRPs is 200 ms. It is worth noting that L1 filtering is a simple averaging process of the input L1 beam-level RSRPs.

In Case 3, we consider predicting L3 cell-level RSRPs directly according to Sub-case 3 where:

- Sub-case 2 (Cell-to-cell Direct Approach): L3 cell-level RSRPs of source frequency at OW time instances are input to AIML model to predict L3 cell-level RSRP of target frequency at PW time instances directly.

Simulation parameters of Case 3 are listed in Table A.1 [2]-[5]. Evaluation results are shown in Table 10.

Table 10: Evaluation results for intra-cell intra-frequency FR2 to FR2 temporal domain case A

| Parameters | 4 GHz 2 GHz | 2 GHz 4 GHz |
| --- | --- | --- |
| UE trajectory option [2] | Option 1 | Option 1 |
| UE trajectory boundary processing option [2] | Option 3 | Option 3 |
| UE speed | 30 Km/h | 30 Km/h |
| Inter-frequency correlation assumption (yes or no) | No | No |
| Measurement reduction rate | N/A | N/A |
| Observation window (OW) | 1000 ms | 1000 ms |
| Prediction window (PW) | 1000 ms | 1000 ms |
| Any other parameters | N/A | N/A |
| Number of training examples | 14700 | 14700 |
| Number of validation examples | 2100 | 2100 |
| Number of testing examples | 4200 | 4200 |
| AIML model input | L3 Cell-level RSRPs of source frequency at OW time instances | L3 Cell-level RSRPs of source frequency at OW time instances |
| AIML model output | L3 Cell-level RSRPs of target frequency at PW time instances | L3 Cell-level RSRPs of target frequency at PW time instances |
| AIML model type | ResNet | ResNet |
| AIML model number of parameters | 1.84 M | 1.84 M |
| AIML model number of FLOPs | 2.38 M | 2.38 M |
| AIML-based Average L3 cell-level RSRP difference (dB) at the end of PW | 1.011 dB | 1.001 dB |
| AIML-based Average L3 cell-level RSRP difference (dB) for all PW instances | [0.950, 0.956, 0.968, 0.986, 1.011] dB | [0.946, 0.955, 0.967, 0.983, 1.001] dB |

We notice that using a higher frequency (e.g., 4 GHz) as input, to predict measurements at a lower frequency (e.g., 2 GHz) or vice versa results in a similar prediction performance.

Observation 12: Using measurements from a higher frequency (e.g., 4 GHz) as input, to predict measurements at a lower frequency (e.g., 2 GHz) or vice versa results in a similar performance.

# 3. Conclusion

In this contribution, initial simulation results for the RRM measurement prediction use case were discussed and the following observations and proposals were made:

Observation 1: For temporal domain case B, as UE speed increases, the average L3 cell-level RSRP difference increases, i.e., AIML model prediction accuracy decreases for both direct and indirect prediction.

Observation 2: For temporal domain case B, the indirect prediction approach (Sub-case 1) slightly outperforms the direct prediction approaches (Sub-Cases 2 and 3) when given the same input (i.e., only L1 filtered beam-level RSRPs) since the direct approach tries to predict a value that depends on a long history (due to L3 filtering impact) using partial information on that history (i.e., a few time instances).

Observation 3: For temporal domain case B, AIML model prediction accuracy is higher than baseline (sample-and-hold)

Observation 4: For temporal domain case A, the indirect prediction approach (Sub-case 1) outperforms the direct prediction approaches (Sub-cases 2 and 3) when given the same input (i.e., only L1 filtered beam-level RSRPs) since the direct approach tries to predict a value that depends on a long history (due to L3 filtering impact) using partial information on that history (i.e., a few time instances).

Observation 5: For temporal domain case A, Sub-case 2 outperforms Sub-case 3 at earlier instances within the PW, but Sub-case 3 outperforms Sub-case 2 at later instances within the PW.

Observation 6: For temporal domain case A, as UE speed increases, the average L3 cell-level RSRP difference increases, i.e., AIML model prediction accuracy decreases for both direct and indirect prediction.

Observation 7: For temporal domain case A, AIML model prediction accuracy is higher than baseline (sample-and-hold)

Observation 8: For temporal domain case A, using shorter OW for a fixed ratio between OW and PW results in better prediction performance.

Observation 9: For temporal domain case A, longer OW may only result in minor prediction performance gain (for a given PW).

Observation 10: For temporal domain case A, at 60 Km/h, increasing the length of PW results in a minor increase in prediction error (0.3 dB – 0.9 dB), however, it results in less measurement overhead.

Observation 11: For temporal domain case A, at 120 Km/h, increasing the length of PW results in a significant increase in prediction error (> 1 dB).

Observation 12: Using measurements from a higher frequency (e.g., 4 GHz) as input, to predict measurements at a lower frequency (e.g., 2 GHz) or vice versa results in a similar performance.

# 4. References

[1]	RAN2-126 Chairman Notes, Fukuoka, Japan, May 2024

[2]	R2-2404955: Summary of [POST125bis][021][AIML mobility] Simulation assumptions and methodology (OPPO)

[3]	R2-2408925, Simulation results for RRM measurement predictions, Interdigital Inc., RAN2-127bis, Hefei, China, October 2024

[4]	R2-2406095: Summary of [POST126][031][AIMob] Simulations

[5]	R2-2497849: Summary of [POST127][030][AIMob] RRM Simulation Assumptions

[6]	TR 38.901: Study on channel model for frequencies from 0.5 to 100 GHz (3GPP TR 38.901 version 16.1.0 Release 16)

# Appendix

### A.1 Simulation assumptions of Intra-cell intra-frequency FR1 to FR1 temporal domain case B

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 – 4 GHz |
| Deployment | UMa, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Inter site distance | 500 m |
| Channel model | According to TR 38.901 [6] |
| System BW | 20 MHz |
| Sub-carrier Spacing | 30 kHz |
| UE Speed | 30 Km/h, 60 Km/h |
| UE distribution | 100% outdoor (in vehicle) |
| BS Antenna Configuration | (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern |
| UE Antenna Configuration | (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Antenna radiation pattern | Omni-direction |
| BS Tx Power | 44 dBm |
| UE receiver Noise Figure | 9 dB |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR 38.901 [6] |
| UE trajectory model | Option 1 (Linear trajectory model with random direction change) |
| LOS Update | LOS Soft |
| Number of Beams | 4 Tx Beams (4 Azimuth angles x 1 Elevation angle) 1 Rx Beam (1 Azimuth angle x 1 Elevation angle) |
| L1-Filtering: Method | Non-sliding Averaging |
| L1-Filtering: Filter Window | 5 |
| Beam Consolidation: nrofSS-BlocksToAverage | 1 |
| Beam Consolidation: absThreshSS-BlocksConsolidation | -110 dBm |
| L3-Filtering: Filter Coefficient K | 4 |

### A.2 Simulation assumptions of Intra-cell intra-frequency FR2 to FR2 temporal domain case A

| Parameter | Value |
| --- | --- |
| Frequency Range | FR2 – 30 GHz |
| Deployment | UMa, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Inter site distance | 200 m |
| Channel model | According to TR 38.901 [6] |
| System BW | 80 MHz |
| Sub-carrier Spacing | 120 kHz |
| UE Speed | 60 Km/h |
| UE distribution | 100% outdoor (in vehicle) |
| BS Antenna Configuration | (8,4,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern |
| UE Antenna Configuration | (4,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Antenna radiation pattern | 2 panels (left, right) |
| BS Tx Power | 40 dBm |
| UE receiver Noise Figure | 10 dB |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR 38.901 [6] |
| UE trajectory model | Option 1 (Linear trajectory model with random direction change) |
| LOS Update | LOS Soft |
| Number of Beams | 16 Tx Beams (8 Azimuth angles x 2 Elevation angles) 4 Rx Beams (4 Azimuth angles x 1 Elevation angle) |
| L1-Filtering: Method | Sliding Averaging |
| L1-Filtering: Filter Window | 5 |
| Beam Consolidation: nrofSS-BlocksToAverage | 3 |
| Beam Consolidation: absThreshSS-BlocksConsolidation | -110 dBm |
| L3-Filtering: Filter Coefficient K | 4 |

### A.3 Simulation assumptions of Inter-cell inter-frequency FR1 to FR1 frequency domain

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 – 4 GHz FR1 – 2 GHz |
| Deployment | UMa, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Inter site distance | 500 m |
| Channel model | According to TR 38.901 [6] |
| System BW | 20 MHz |
| Sub-carrier Spacing | 30 kHz |
| UE Speed | 30 Km/h |
| UE distribution | 100% outdoor (in vehicle) |
| BS Antenna Configuration | (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern |
| UE Antenna Configuration | (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Antenna radiation pattern | Omni-direction |
| BS Tx Power | 44 dBm |
| UE receiver Noise Figure | 9 dB |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR 38.901 [6] |
| UE trajectory model | Option 1 (Linear trajectory model with random direction change) |
| LOS Update | LOS Soft |
| Number of Beams | 4 Tx Beams (4 Azimuth angles x 1 Elevation angle) 1 Rx Beam (1 Azimuth angle x 1 Elevation angle) |
| L1-Filtering: Method | Non-sliding Averaging |
| L1-Filtering: Filter Window | 5 |
| Beam Consolidation: nrofSS-BlocksToAverage | 1 |
| Beam Consolidation: absThreshSS-BlocksConsolidation | -110 dBm |
| L3-Filtering: Filter Coefficient K | 4 |