3GPP TSG-RAN WG2 Meeting #128	R2-2410508

Orlando, USA, Nov 18th – 22nd 2024

Agenda Item:	8.3.5

Source:	InterDigital Inc.

Title:	Generalization of AIML models for RRM measurement prediction

Document for:	Discussion

# 1. Introduction

In [1], the study item rapporteur for AIML based mobility proposed one of the items for further (simulation) study was aspects related to model generalization, and this was agreed to be discussed in RAN2-128 [2].

It is not feasible/practical to train a different AIML model for each possible scenario (e.g., UE speed). Thus, it is important to study the capability of RRM measurement prediction AIML models to generalize to scenarios different from the ones they are trained for. This study can be performed by evaluating how an AIML model that is trained for one scenario performs in another scenario. In this contribution, we discuss some aspects of RRM measurements prediction generalization study, namely, UE speed and prediction window length.

# 2. Discussion

## 2.1 UE Speed

If we consider two AIML models (model A and model B) trained for two different UE speed values or ranges (speed A and speed B), we may study the capability of each AIML model to perform inference on the case that it was not trained for. Also, we may study the degradation in performance if we train another AIML model (model C) using a mixed dataset with training examples from both datasets used for training models A and B. In summary, we may consider evaluating the prediction performance in the following cases:

A) Baseline Cases:

- Baseline A: Use AIML model A for inference on examples from dataset of UEs at speed A

- Baseline B: Use AIML model B for inference on examples from dataset of UEs at speed B

B) Direct Generalization Cases:

- Generalization A2B: Use AIML model A for inference on examples from dataset of UEs at speed B

- Generalization B2A: Use AIML model B for inference on examples from dataset of UEs at speed A

C) Mixed Model Cases:

- Mixed A: Use AIML model C (trained with examples from both data sets of speed A and B) for inference on examples from dataset of UEs at speed A

- Mixed B: Use AIML model C (trained with examples from both data sets of speed A and B) for inference on examples from dataset of UEs at speed B

Furthermore, we may study the impact of UE speed difference (difference between speed A and speed B) on the generalization performance. For example, using an RRM measurement prediction model trained for UE speed of 60 Km/h may generalize better than another model that is trained for UE speed of 90 Km/h when it comes to RRM measurement prediction at UE speed of 30 Km/h.

Thus, we propose:

Proposal 1: Study the AIML model generalization performance at different UE speed values or ranges for the RRM measurement prediction use case, specifically:

- Use an AIML model that is trained with examples from a data set of UEs at speed A for inference on examples from dataset of UEs at speed B

- Use an AIML model that is trained with examples from a data set of UEs at speed A and speed B for inference on examples from dataset of UEs at speed A (or UEs at Speed B)

Proposal 2: Study the impact of UE speed difference (between UE speed at which the AIML model is trained and the UE speed for which inference is performed) on the generalization performance of RRM measurement prediction models.

## 2.2 Prediction Window (PW) Length

Consider two AIML models (model A and model B) trained for RRM measurement prediction given the same input size (i.e., same observation window (OW) length), but for different PW lengths (PW A and PW B), where PW B is longer than PW A. We may study the capability of AIML model B to perform inference on the case of PW A by simply ignoring predicted measurements beyond the length of PW A. In summary, we may consider evaluating the prediction performance in the following two cases:

- Baseline A: Use AIML model A for prediction of RRM measurements during PW A

- Generalization B2A: Use AIML model B for prediction of RRM measurements during PW A

If model B generalizes well to case of PW A, i.e., achieves a close prediction performance to the baseline, we may consider training a single model according to the longest PW and configure it to use the subset of the outputs according to a configured PW instead of switching between dedicated AIML models for different PW lengths.

We may also study the impact of PW length difference (difference between PW A and PW B) on generalization performance. For example, using an RRM measurement prediction model trained for PW length of 800 ms may generalize better than another AIML model that is trained for PW length of 1600 ms when it comes to RRM measurement prediction at PW length of 400 ms.

Thus, we propose:

Proposal 3: Study the AIML model generalization for the RRM measurement prediction use case that considers prediction window lengths, specifically:

- Use an AIML model trained for a larger prediction window for inference within a shorter subset of that prediction window

Proposal 4: Study the impact of the difference between PW length that the model is trained for and the PW length that the model is being used for on the generalization performance of RRM measurement prediction models.

# 3. Conclusion

In this contribution, some aspects of RRM measurements prediction generalization study were discussed, and the following proposals were made:

Proposal 1: Study the AIML model generalization performance at different UE speed values or ranges for the RRM measurement prediction use case, specifically:

- Use an AIML model that is trained with examples from a data set of UEs at speed A for inference on examples from dataset of UEs at speed B

- Use an AIML model that is trained with examples from a data set of UEs at speed A and speed B for inference on examples from dataset of UEs at speed A (or UEs at Speed B)

Proposal 2: Study the impact of UE speed difference (between UE speed at which the AIML model is trained and the UE speed for which inference is performed) on the generalization performance of RRM measurement prediction models.

Proposal 3: Study the AIML model generalization for the RRM measurement prediction use case that considers prediction window lengths, specifically:

- Use an AIML model trained for a larger prediction window for inference within a shorter subset of that prediction window

Proposal 4: Study the impact of the difference between PW length that the model is trained for and the PW length that the model is being used for on the generalization performance of RRM measurement prediction models.

# 4. References

[1]	R2-2409402, Discussion on work plan and potential SLS, OPPO, Hefei, China, Oct. 2024

[2]	RAN2-127-bis Chairman Notes, Hefei, China, Oct. 2024