3GPP TSG RAN WG2 #128		R2- 2410529

Orlando, November 18th – 22nd, 2024

Agenda Item:	8.1.5

Source:	BT Plc, Turkcell

Title:	Operators views on the AI_ML model delivery options

Document for:	Discussion/Decision

# Introduction

During Rel-18, the study for AI/ML model transfer/delivery was captured in TR 38.843 [1]. In there, section 7.2.1.4 Model transfer/delivery does the analysis how models for AI/ML can be transferred. The following Table 1 and Table 2 summarizes the conclusions.

Table 1 Need to consider standardization solutions for transferring/delivering AI/ML models

Whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) is unclear from the outcome of the present study

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP). - Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE. Note: The relationships between model transfer/delivery solutions and use cases can be derived from what is captured in clauses 7.3.2, 7.3.3, and 7.3.4.

Table 2 Transfer/deliver model solutioions

Different model transfer/delivery solutions

- A1: Large, no upper limit model/model parameter size, - A2: Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs), - A3: Network controllability on model transfer/delivery (e.g., management decision at gNB), - A4: Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB).

TR 38.843 [1] captures the definition of single-sided and two-sided models as:

- Two-sided (AI/ML) model: A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. - Network-side (AI/ML) model: An AI/ML Model whose inference is performed entirely at the network. - UE-side (AI/ML) model: An AI/ML Model whose inference is performed entirely at the UE.

During RAN2#127bis meeting, companies discussed about the different options and visibility of the data.

In this TDoc, the intetion is to downselect the options based on current deployments and market tractions.

# Discussion

## General requirement

The solutions summarized on Table 1 can be categorized into three different categories.

- RAN

- Core and LMF

- OAM

Since the OTT solution has no impact on 3GPP specifications, we consider there is not need to further discuss.

Proposal 1: Solution 4a, OTT without 3GPP impact, captured on [1] is no further discussed in RAN2

## MNO model structure requirement

First step should be to define the model structure. To facilitate the RAN2 discussion and embrace some progress about how to transfer the model, independently if it is one-side or two-side model, it is important to conclude on the model structure. Otherwise, RAN2 cannot decide how to transfer it.. After last RAN2 meeting, it is clear that different companies have different understandings about the content of that model. In any case, from an operational prespective, it is important to remaind that the solution will be deployed on a multi-vendor environment where collaboration is required.

A few comments made during last RAN2#127bis meeting highglight that companies do not share the same understanding.

Discussions on requirements - Qualcomm explains that the structure and model is UE implementation and it should not be visible. - Samsung thinks that without some understanding of the data structure will be needed before discussing signalling. The focus of RAN1 is z4. - Apple thinks this is different from data collection, but RAN1 will define the format, open or not. Operators should raise this issue in RAN1. We should focus on two sided model. -- Mediatek think that meta data information can be visible and ffs what that data is.

There are companies asking RAN2 to wait until RAN1 concludes even WID indicates RAN2 is responsibe of the model identification “Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1]”. Hence, it seems that RAN2 is the one that has to define the details of how the model is identified and there is no need to wait for RAN1.

Observation 1: RAN2 is in charge to define the details of the model identification based on WID description

Ultimately, the MNO has legal the contracts with customers. Leaving the model definition completely up to vendor implementation is unacceptable for MNOs. The MNOs have legal obligations therefore, they must keep the control over the network. Therefore, RAN2 needs to discuss how MNOs will ensure their legal obligations while the model structure allows innovation.

Observation 2: The MNO is the one that has the legal obligations with their customers

As suggested during the RAN2#127bis online discussion, before RAN2 discuss about the model transfer/deliver over the control plane (CP), the user plane (UP), or a combination of both, it is required to define the AI/ML model structure. Only after that, RAN2 will be capable to design the solution.

Proposal 2: RAN2 to discuss and agree the model structure; including at least the data that will be standardized; before different model transfer/delivery options are discussed

## Visibility and control of the model structure

From an operational point of view, it is important for MNOs to have full visibility of standardized data as it should allow to identify uniquevocally the model. It may include, but not limited to, the model identifier, the model version, the model creation data, the model size, etc. That is the only way MNOs can trace what is in use used and by who it is used.

Observation 3: visibility means the MNO is able to see the information without any restriction

It might be the case where a new model does not behave as expected. That may happen with single and two sided models. That lack of performance on the air interface will be observed at RAN level. Core network has no mechanisms to discover that. It implies, it is necessary to find a solution where RAN (e.g., the gNB) is aware of the UE configured model. That will allow actions like model activation/deactivation/switch. Otherwise, it is not possible for the MNO to properly control the AI/ML models for the air interface.

Observation 4: RAN needs to be aware of the model used by the UE to map performance – model being used

Visibility of the standardizated data and RAN awareness of the model will let the MNO to monitor the model performance. With that information, the MNO will be able to proceed in consequence.

Proposal 3: MNO has visibility of the model standardized data to keep the control over the network for one-sided and two-sided models

There are multiple occasions where MNO may want to delay the model delivery. One example is during a congestion period, the transfer/delivery of new models can be delayed. If the MNO has the performance under control, the transfer/delivery can be postponed. If the MNO knows in advance that a model underperform under certain network configuration, the model transfer/delivery can be simply rejected. If multiple models are delivered/transferred in one shot, the MNO should be the entity responsible to activate/deactivate them.

Only the MNO is in a position to control the end-to-end network. For that reason, it is the MNO who should control the model delivery/transfer.

Proposal 4: Only the MNO has control over the model delivery/transfer and control over the AI/ML model activation/deactivation

MNO must preserve customers security. It also has NDAs with providers. Then, the data used to train the model needs to be anonymazied and visible to MNO to ensure that data does not violate any legal obligation.

Proposal 5: Data used to train models needs to be visible to MNO

## AI/ML model transfer/delivery by gNB, CN – LMF, OAM

After the study [1], and recent agreements in RAN1 [2], there are several entities identified as candidates to deliver the model as captured in Table 1 and Table 3.

Table 3: CSI feedback enhancements from [1]

### 7.2.2 CSI feedback enhancement The following set of objectives have been identified for the two-sided CSI compression use case. […] For data collection, model transfer/delivery, and function-to-entity mapping analysis, various scenarios unfold for both the two-sided CSI compression use case, as well as for the UE-side CSI prediction use case, when the data generation and termination entities differ. For instance, for: - Model Training: o For the two-sided CSI compression use case, training data can be generated by either the UE or the gNB, depending on specific requirements, while the termination point for training data may include the gNB, OAM, Over-The-Top (OTT) server or UE.  Note: RAN2 identified the case in which Core Network may be used for model training. However, no study was conducted since this is beyond the scope of this Working Group.

Conclusions from Table 3 indicates gNB and OAM may be included while RAN1 clearly stated that the core network is beyond their scope, therefore, they are not in a position to include it. It is understood that RAN1 has not performed any study or analysis on the core network hence, it cannot be excluded.

Observation 5: Based on Note of 2.2.1 [1], the core network for the two-sided use case was skipped from RAN1 study

To deliver the model, Table 1 captures the different options, gNB, Core/LMF and OAM. As it stated before, OTT is outside 3GPP.

The gNB can transfer the model but on real deployments, all gNBs in the network need to be updated with additional storage and SW. MNOs cannot ensure that all gNBs can be updated in time. On specific scenarios like RAN sharing, perhaps the gNB cannot be updated due to lack of business agreements.

Once the gNB is responsible to deliver the model, it will require to consider how the model is transfered to its neighbours where not all neighbours might be updated. In cases like CHO where different gNB might be involved before the HO is executed, the model will be required to be transferred to several gNBs and keep them updated to ensure the model can continue its transference successfully.

But as we stated before, gNB needs to be aware of the model used by UE to keep the performance under control. The fact that gNB does not transfer the model does not mean it is totally excluded from the delivery process.

Observation 6: The gNB solution adds challenges to real deployments to widely deploy the feature, specially when different pre-Rel-19 features start to be deployed but it is beneficial if gNB is aware of the model being used by the UE

From 3GPP side, OAM can support multivendor but that feature is not implemented. Current deployments require a OAM per RAN vendor. Then, it is not clear how one OAM belonging to one RAN vendor will transfer the model to a gNB belonging to a different vendor. For standardization, this issue can be handled but it is important to remaind the lack of interest to support inter-RAT OAM features. Taking into account that in a real deployment, each MNO may have several RAN vendors providers, the OAM is not a realistic option.

Observation 7: Current implementations lacks for support of inter-RAN OAM capabilities and market shows no interest in support them

The core network can handle above problems since data is stored in a single location and all RAN vendors must interact with it. Hence, the core network cannot be excluded at this stage.

Observation 8: Core Network may have the best balance between pros and cons for model transfer

Since it is not clear which is the best entity to deliver the model and core network was not considered during the study, RAN2 should not exclude the core network. Instead,

Proposal 6: RAN2 is asked to create a pros/cons list based for the Core Network, gNB and OAM to transfer/deliver the AI/ML model considering real deployment scenarios (e.g., mobility, inter-vendor compatibility, costs, security, etc.) taking into account the model structure before any downselection

## AI/ML model transfer/delivery over CP and/or UP

Table 2 points to different aspects to be considered. A1 – “Large, no upper limit model/model parameter size” makes reference to container size. The CP has a limitation as defined in [1]. The UP does not have such limitation but without any limitation, it may result in unnecessary large container.

Proposal 7: The maximum size of the container used to transfer/deliver the AI/ML model needs to be standardized

RAN2 needs to decide between the CP or the UP to transfer/deliver the model. With CP solution, segmentation may be required but RAN2 will not know that until the model data structure is defined. Segmentation of CP paquets will increase the load and the overhead and that may have a severe impact on all UEs. During the design of the delivery mechanisms, it is imperative to avoid any impact on other UEs. By any mean, this new mechanism can create CP congestion.

On live deployments, MNOs uses DSS. That limit the number of symbols the PDCCH. Market has a lack of traction of UE capabilities to support additional NR symbols when DSS like pdcch-MonitoringSingleOccasion or pdsch-MappingTypeB-Alt-r16. That implies, DSS bands are in fact limited to one symbol for the PDCCH.

Observation 9: MNOs have different 5G SA approaches that leads to different challenges on the CP

The UP does not have size restrictions to transfer the model but as it is stated before, MNO requires visibility and control over the model tranfer and for that, the gNB needs to be aware of the model being used by UEs. This is not possible if the model transfer mechanism only includes UP. It is for that reason that we propose to postpone the CP or UP discussion.

Observation 10: The final solution to transfer/deliver the model may be a hybrid solution where data required to identify and to control the model is transferred via CP and the model itself via UP

Privacy of each MNO customer is a requirement for a 3GPP solution. Ultimately, the MNO is responsible to guarantee the privacy of end customers. In addition, each MNO has its own network configuration optimized for their use cases. Sending that information outside the MNO control may have legal consequences. Sending MNOs own data outside MNO’s network transparently may break NDAs with legal consequences for MNOs.

Observation 11: For security reasons, specific UE data cannot be sent outside the MNO network without MNO knowledge either MNO proprietary data

Proposal 8: RAN2 to postpone the UP and/or CP donwselection to transfer/deliver the AI/ML model until it is decided the entity(ies) to transfer the model

# Conclusions

During the TDoc, we have made the following observations

Observation 1: RAN2 is in charge to define the details of the model identification based on WID description

Observation 2: The MNO is the one that has the legal obligations with their customers

Observation 3: visibility means the MNO is able to see the information without any restriction

Observation 4: RAN needs to be aware of the model used by the UE to map performance – model being used

Observation 5: Based on Note of 2.2.1 [1], the core network for the two-sided use case was skipped from RAN1 study

Observation 6: The gNB solution adds challenges to real deployments to widely deploy the feature, specially when different pre-Rel-19 features start to be deployed but it is beneficial if gNB is aware of the model being used by the UE

Observation 7: Current implementations lacks for support of inter-RAN OAM capabilities and market shows no interest in support them

Observation 8: Core Network may have the best balance between pros and cons for model transfer

Observation 9: MNOs have different 5G SA approaches that leads to different challenges on the CP

Observation 10: The final solution to transfer/deliver the model may be a hybrid solution where data required to identify and to control the model is transferred via CP and the model itself via UP

Observation 11: For security reasons, specific UE data cannot be sent outside the MNO network without MNO knowledge either MNO proprietary data

For those reasons, it is proposed

Proposal 1: Solution 4a, OTT without 3GPP impact, captured on [1] is no further discussed in RAN2

Proposal 2: RAN2 to discuss and agree the model structure; including at least the data that will be standardized; before different model transfer/delivery options are discussed

Proposal 3: MNO has visibility of the model standardized data to keep the control over the network for one-sided and two-sided models

Proposal 4: Only the MNO has control over the model delivery/transfer and control over the AI/ML model activation/deactivation

Proposal 5: Data used to train models needs to be visible to MNO

Proposal 6: RAN2 is asked to create a pros/cons list based for the Core Network, gNB and OAM to transfer/deliver the AI/ML model considering real deployment scenarios (e.g., mobility, inter-vendor compatibility, costs, security, etc.) taking into account the model structure before any downselection

Proposal 7: The maximum size of the container used to transfer/deliver the AI/ML model needs to be standardized

Proposal 8: RAN2 to postpone the UP and/or CP donwselection to transfer/deliver the AI/ML model until it is decided the entity(ies) to transfer the model

# References

- TR 38.843 - Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- Report of 3GPP TSG RAN WG1 #118 v0.3.0