3GPP TSG-RAN WG2 Meeting #128					           R2-2410539

Orlando, US, November 18 – 22, 2024

Agenda Item:		8.3.2

Source:			Huawei, HiSilicon

Title:			Simulation results for RRM measurement prediction

Document for:		Discussion and Decision

# 1   Introduction

In RAN2#126 meeting, the agreement on the priority of each use cases of RRM measurement prediction were reached as follows:

Agreements Case number; Prioritization; Evaluation scenario combination; target study goal; Methodology. 1; Low; FR1 to FR1 intra-frequency temporal domain case A; 2nd goal; TBD. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. 5; Low; FR2 to FR2 intra-frequency temporal domain case B; 1st goal; TBD. 6; Middle; FR2 to FR2 intra-frequency spatial domain; 1st goal; Intra-cell.

According to the agreement, case 2/3/4 are considered as high priority. In this paper, we will provide some simulation results for RRM measurement prediction, which will cover cases 2/3/4. In addition, we will also provide the detailed simulation assumptions and model description. The simulation assumptions that we used are provided in Annex A.

# 2   Simulation results for RSRP prediction accuracy

## 2.1 General

To facilitate the discussion of the prediction results, a unified report template within excel file were agreed. The simulation results in that form are attached to this contribution in separate files for easy merging with results from other companies. In this paper, we list our RRM measurement prediction results in the tabular report format which was agreed earlier and is captured in Annex A.1 of [1].

## 2.2 Case 2 (FR1 intra-frequency temporal domain case B)

Table 3: Simulation assumptions, settings and model information for Case 2

| Report parameters |  |  |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 1 (wrap-around model) |
|  | UE speed (30,60,90,120 Km/h) | 30, 60, 90, 120 km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | no |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | no |
|  | Whether LOSsoft is modeled or not | no |
|  | spatial consistency option (A or B) | B |
|  | Number of TX beams | 4 |
|  | Number of RX beams | 1 |
|  | Measurement reduction rate(50%,…Note2) | 50%, 66% |
|  | Observation window (Note3) | See below |
|  | Prediction window (Note3) | See below |
|  | Any other parameters (Note 4) | N/A |
| Data Size (Number of Samples) | Training/validity | 430000 |
|  | Testing | 3000 |
| AIML model input/output | Model input (Note 5) | Best cell RSRP, RRM sub-use case 1 and 2 |
|  | Model output(Note 6) | Best cell RSRP, RRM sub-use case 1 and 2 |
| AIML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | 0.08 M |
|  | Model complexity in model size (e.g. Mbyte) | 0.35 Mbytes |
|  | Computational complexity [FLOPs] | 1.03 M |
| Metrics | Average L3 cell-level RSRP difference (dBm) | See below for results for different speeds and OW/PW values. |

L1 filtering with non-sliding window was used for all simulations. We simulated MRRT of 50% and 66%, according to the following patterns:

| Measurement | Prediction |
| --- | --- |

|  |  |  |  | MRRT=50% |
| --- | --- | --- | --- | --- |

|  |  |  |  |  |  | MRRT=66% |
| --- | --- | --- | --- | --- | --- | --- |

The results are presented in tables below.

Table 4: Simulation results for Case 2 (in red, results with RMSE<3dBm)

| MRRT: 50% RRM sub-use case 1 |  |  |  | MRRT: 50% RRM sub-use case 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| OW [ms] | PW [ms] | RMSE [dBm] for UE speed: 30/60/90/120 km/h |  | OW [ms] | PW [ms] | RMSE [dBm] for UE speed: 30/60/90/120 km/h |
| 600 | 200 | 0.46/1.23/1.61/1.80 |  | 600 | 200 | 0.44/1.21/1.61/1.79 |
|  | 600 | 1.46/2.35/2.63/2.66 |  |  | 600 | 1.45/2.32/2.62/2.66 |
| 1000 | 200 | 0.47/1.17/1.58/1.80 |  | 1000 | 200 | 0.46/1.18/1.57/1.81 |
|  | 600 | 1.36/2.40/2.60/2.65 |  |  | 600 | 1.36/2.39/2.59/2.65 |
| MRRT: 66% RRM sub-use case 1 |  |  |  | MRRT: 66% RRM sub-use case 2 |  |  |
| OW [ms] | PW [ms] | RMSE [dBm] for UE speed: 30/60/90/120 km/h |  | OW [ms] | PW [ms] | RMSE [dBm] for UE speed: 30/60/90/120 km/h |
| 800 | 400 | 1.09/2.00/2.82/3.96 |  | 800 | 400 | 1.08/1.99/2.82/3.40 |
|  | 1000 | 2.37/3.27/3.84/4.25 |  |  | 1000 | 2.37/3.28/3.83/4.24 |
| 1400 | 400 | 1.04/2.00/2.78/3.31 |  | 1400 | 400 | 1.00/2.01/2.81/3.30 |
|  | 1000 | 2.50/3.48/3.94/4.13 |  |  | 1000 | 2.44/3.52/3.95/4.16 |

In the results above, we also highlighted the predictions where RMSE was below a 3dBm threshold. This threshold is used only to facilitate an analysis of the prediction accuracy and it does not mean that the prediction accuracy is necessarily good or bad. In order to better understand what kind of prediction accuracy may be acceptable for the system, it is necessary to consider other KPIs, such as system level KPIs.

In general, based on the results it can be observed that the prediction accuracy is dependent on UE speed, prediction window length, MRRT as well as combination of these factors. For MRRT of 50%, it was possible to obtain RMSE<3dBm even for very high speed scenarios (120 km/h) for all simulated OW and PW lengths. For MRRT of 66% such accuracy was only achievable either for lower UE speeds when PW of 1000 ms was used or for higher speeds, but only in case of shorter PW=400ms.

The following observations can be made based on the above results:

Observation 1 (for case 2): With the MRRT increasing, the prediction accuracy is declining.

Observation 2 (for case 2): With the UE speed increasing, the prediction accuracy is declining.

Observation 3 (for case 2): With the increase of PW length, the accuracy is declining.

Observation 4 (for case 2): The eventual accuracy of the prediction depends on the combination of factors such as UE speed, length of the prediction window and MRRT.

## 2.3 Case 3 (FR1 to FR1 inter-frequency (frequency domain))

For Case 3, simulation assumptions and results can be found in Table X and Y below.

Table 5: Simulation assumptions, settings and model information for Case 3

| Report parameters |  |  |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 1 (wrap-around model) |
|  | UE speed (30,60,90,120 Km/h) | 30, 60, 90, 120 km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | no |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | no |
|  | Whether LOSsoft is modeled or not | no |
|  | spatial consistency option (A or B) | B |
|  | Number of TX beams | 4 |
|  | Number of RX beams | 1 |
|  | Measurement reduction rate(50%,…Note2) | N/A |
|  | Observation window(Note3) | See below |
|  | Prediction window (Note3) | N/A |
|  | Any other parameters (Note 4) | High to low frequency prediction was performed |
| Data Size (Number of Samples) | Training/validity | 30000 |
|  | Testing | 500 |
| AIML model input/output | Model input (Note 5) | Best cell RSRP, RRM sub-case 1 and 2 |
|  | Model output(Note 6) | Best cell RSRP, RRM sub-case 1 and 2 |
| AIML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | 0.08 M |
|  | Model complexity in model size (e.g. Mbyte) | 0.35 Mbytes |
|  | Computational complexity [FLOPs] | 1.03 M |
| Metrics | Average L3 cell-level RSRP difference (dBm) | See below |

Table 6: Simulation results for Case 3

| PW=0ms, RRM sub-use case 1 4GHz predicted based on 2GHz |  |  |  | PW=0ms, RRM sub-use case 1 2GHz predicted based on 4GHz |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OW [ms] | RMSE [dBm] for UE speed: 30/60/90/120 km/h |  |  | OW [ms] |  |  |  | RMSE [dBm] for UE speed: 30/60/90/120 km/h |  |  |
| 200 | 0.24/0.24/0.65/0.81 |  |  | 200 |  |  |  | 0.23/0.23/0.70/0.76 |  |  |
| 400 | 0.31/0.32/0.71/0.82 |  |  | 400 |  |  |  | 0.30/0.32/0.70/0.78 |  |  |
| 600 | 0.35/0.39/0.74/0.84 |  |  | 600 |  |  |  | 0.33/0.38/0.73/0.81 |  |  |
| 800 | 0.48/0.50/0.80/0.89 |  |  | 800 |  |  |  | 0.48/0.50/0.80/0.89 |  |  |
| 1000 | 0.68/0.68/0.83/0.95 |  |  | 1000 |  |  |  | 0.67/0.68/0.81/0.91 |  |  |
| PW=0ms, RRM sub-use case 2 4GHz predicted based on 2GHz |  |  |  | PW=0ms, RRM sub-use case 2 2GHz predicted based on 4GHz |  |  |  |  |  |  |
| OW [ms] | RMSE [dBm] for UE speed: 30/60/90/120 km/h |  |  | OW [ms] |  |  |  | RMSE [dBm] for UE speed: 30/60/90/120 km/h |  |  |
| 200 | 0.52/0.56/0.79/0.82 |  |  | 200 |  |  |  | 0.46/0.55/0.78/0.80 |  |  |
| 400 | 0.53/0.58/0.79/0.85 |  |  | 400 |  |  |  | 0.51/0.56/0.79/0.82 |  |  |
| 600 | 0.54/0.58/0.81/0.86 |  |  | 600 |  |  |  | 0.52/0.57/0.82/0.84 |  |  |
| 800 | 0.59/0.59/0.83/0.89 |  |  | 800 |  |  |  | 0.57/0.59/0.82/0.84 |  |  |
| 1000 | 0.69/0.70/0.86/0.97 |  |  | 1000 |  |  |  | 0.68/0.69/0.83/0.95 |  |  |

Based on the results it can be seen that the accuracy of inter-frequency measurement prediction is very good (below 1dBm) for all the simulated UE speeds and observation window lengths, and for both 2->4Ghz and 4->2Ghz directions. In this scenario, RRM sub-use case 1 performs slightly better, in particular using shorter OW lengths and for lower UE speeds. It should be also noted that extending OW length beyond the baseline value of 200ms only deteriorates the accuracy. This may be justified by the fact that measured results closer to the sampling timestamp are more correlated with the RSRP of the predicted cell than the samples more distant in time.

Observation 5 (for Case 3): The accuracy of inter-frequency measurement prediction is very good (below 1dBm) for all the simulated UE speeds (30-120km/h) and observation window lengths (200-1000ms), and for both 2->4Ghz and 4->2Ghz directions.

Observation 6 (for Case 3): With the increase of OW, the prediction accuracy declines. This may be justified by the fact that the current RSRP of the inter-frequency cell is best correlated with the latest measurements on the serving frequency cell.

Observation 7 (for Case 3): RRM sub use case 1 performs slightly better for inter-frequency prediction than RRM sub use case 2, in particular for lower OW lengths and UE speeds (the observed difference is up to 0.32dBm).

Observation 8 (for Case 3): The accuracy of prediction in the 2->4GHz direction is comparable to this of 4->2 GHz direction, with a prediction in 4->2Ghz (low frequency predicted based on higher frequency) performing slightly better (the maximum difference=0.06 dBM)

## 2.4 Case 4 (FR2 intra-frequency temporal domain case A)

Table 7: Simulation assumptions, settings and model information for Case 4

| Report parameters |  |  |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 1 (wrap-around model) |
|  | UE speed (30,60,90,120 Km/h) | 30, 60, 90, 120 km/h |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | no |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | no |
|  | Whether LOSsoft is modeled or not | no |
|  | spatial consistency option (A or B) | B |
|  | Number of TX beams | 64 |
|  | Number of RX beams | 8 |
|  | Measurement reduction rate(50%,…Note2) | N/A |
|  | Observation window(Note3) | See below |
|  | Prediction window (Note3) | See below |
|  | Any other parameters (Note 4) | N/A |
| Data Size (Number of Samples) | Training/validity | 60000 |
|  | Testing | 1000 |
| AIML model input/output | Model input (Note 5) | Best cell RSRP, RRM sub-case 1 and 2 |
|  | Model output(Note 6) | Best cell RSRP, RRM sub-case 1 and 2 |
| AIML model description | Model type (e.g., LSTM, CNN, transformer …) | LSTM |
|  | Model complexity in a number of parameters(M) | 0.08 M |
|  | Model complexity in model size (e.g. Mbyte) | 0.35 Mbytes |
|  | Computational complexity [FLOPs] | 1.04 M |
| Metrics | Average L3 cell-level RSRP difference (dBm) | See below |

Table 8: Simulation results for Case 4

| RRM sub use case 1 PW=400ms |  |  |  | RRM sub use case 2 PW=400ms |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OW [ms] | RMSE [dBm] for UE speed: 30/60/90/120 km/h |  |  | OW [ms] |  |  | RMSE [dBm] for UE speed: 30/60/90/120 km/h |  |  |  |
| 400 | 1.37/2.23/3.16/3.55 |  |  | 400 |  |  | 1.35/2.07/2.98/3.45 |  |  |  |
| 800 | 1.14/1.80/2.72/3.05 |  |  | 800 |  |  | 0.92/1.74/2.37/2.81 |  |  |  |
| 1200 | 1.18/2.02/2.64/3.22 |  |  | 1200 |  |  | 0.93/1.75/2.36/2.79 |  |  |  |
| RRM sub use case 1 PW=800ms |  |  |  | RRM sub use case 2 PW=800ms |  |  |  |  |  |  |
| OW [ms] | RMSE [dBm] for UE speed: 30/60/90/120 km/h |  |  | OW [ms] |  |  | RMSE [dBm] for UE speed: 30/60/90/120 km/h |  |  |  |
| 800 | 2.31/3.67/4.89/5.75 |  |  | 800 |  |  | 2.02/3.21/4.55/5.23 |  |  |  |
| 1600 | 2.42/3.75/4.92/5.82 |  |  | 1600 |  |  | 2.01/3.34/4.43/5.24 |  |  |  |
| 2400 | 2.44/3.87/5.34/5.54 |  |  | 2400 |  |  | 2.02/3.30/4.34/5.16 |  |  |  |
| RRM sub use case 1 PW=1200ms |  |  |  | RRM sub use case 2 PW=1200ms |  |  |  |  |  |  |
| OW [ms] | RMSE [dBm] for UE speed: 30/60/90/120 km/h |  |  | OW [ms] |  |  | RMSE [dBm] for UE speed: 30/60/90/120 km/h |  |  |  |
| 1200 | 3.37/4.77/6.79/7.40 |  |  | 1200 |  |  | 3.00/4.47/5.96/6.75 |  |  |  |
| 2400 | 3.38/4.73/6.62/7.45 |  |  | 2400 |  |  | 3.04/4.41/5.80/6.48 |  |  |  |
| 3600 | 3.44/4.67/6.50/7.23 |  |  | 3600 |  |  | 3.02/4.31/5.88/6.33 |  |  |  |

Based on the results it can be seen that the prediction accuracy for FR2 intra-frequency temporal domain case A depends on several factors, in particular: prediction window length, and whether L1 measurements or L3 measurements were used as an input/output (i.e. RRM sub use case 1 versus RRM sub use case 2). The following observations can be made:

Observation 9 (for Case 4): Prediction using RRM sub use case 2 performs better than using RRM sub use case 1, e.g. for PW=400ms:

- Error range for RRM sub-use case 1: 1.37~3.55 dBm

- Error range for RRM sub-use case 2: 1.35~3.45 dBm

Observation 10 (for Case 4): With longer prediction window and higher speed, accuracy declines.

Observation 11 (for Case 4): For prediction windows of 400ms it is usually possible to obtain accuracy error < 3dB. Higher prediction windows result in bigger errors, especially in higher speeds.

# 3   Summary

Based on the simulations presented in this paper, the following observations are made:

Observation 1 (for case 2): With the MRRT increasing, the prediction accuracy is declining.

Observation 2 (for case 2): With the UE speed increasing, the prediction accuracy is declining.

Observation 3 (for case 2): With the increase of PW length, the accuracy is declining.

Observation 4 (for case 2): The eventual accuracy of the prediction depends on the combination of factors such as UE speed, length of the prediction window and MRRT.

Observation 5 (for Case 3): The accuracy of inter-frequency measurement prediction is very good (below 1dBm) for all the simulated UE speeds (30-120km/h) and observation window lengths (200-1000ms), and for both 2->4Ghz and 4->2Ghz directions.

Observation 6 (for Case 3): With the increase of OW, the prediction accuracy declines. This may be justified by the fact that the current RSRP of the inter-frequency cell is best correlated with the latest measurements on the serving frequency cell.

Observation 7 (for Case 3): RRM sub use case 1 performs slightly better for inter-frequency prediction than RRM sub use case 2, in particular for lower OW lengths and UE speeds (the observed difference is up to 0.32dBm).

Observation 8 (for Case 3): The accuracy of prediction in the 2->4GHz direction is comparable to this of 4->2 GHz direction, with a prediction in 4->2Ghz (low frequency predicted based on higher frequency) performing slightly better (the maximum difference=0.06 dBM)

Observation 9 (for Case 4): Prediction using RRM sub use case 2 performs better than using RRM sub use case 1, e.g. for PW=400ms:

- Error range for RRM sub-use case 1: 1.37~3.55 dBm

- Error range for RRM sub-use case 2: 1.35~3.45 dBm

Observation 10 (for Case 4): With longer prediction window and higher speed, accuracy declines.

Observation 11 (for Case 4): For prediction windows of 400ms it is usually possible to obtain accuracy error < 3dB. Higher prediction windows result in bigger errors, especially in higher speeds.

# References

- R2-2406309	Text proposal of 38.744	OPPO	draft TR

# Annex A – simulation assumptions

Below are the company selectable assumptions chosen by us.

| Configurable/selectable simulation assumption |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spatial consistency | Whether LOSsoft is modeled or not | UE trajectory model | UE trajectory boundary processing | L1/L3 filtering (sliding/non-sliding) | Number of Tx beams | Number of Rx beams |
| Procedure B in TR38.901 | Not modelled | Option 1 in 38.843 section 6.3.1 | Wrap-around model | Both non-filtered and non-sliding L1 filtering cases were simulated | FR1: 4 FR2: 64 | FR1: 1 FR2: 8 |

All the simulation assumptions are listed in the table below with the selectable ones highlighted in yellow.

Table 1: Summarized simulation assumptions

| Parameter | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz, 15/30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft and inter-frequency correlation; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading without LOSsoft; without Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage |
| System BW | 20MHz | 80MHz |
| UE speed | 30,60,90 km/h for study targeting measurement reduction | 60,90,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm | 40 dBm |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | Procedure B in TR38.901 | Procedure B in TR38.901 |
| UE trajectory model | Option 1 in 38.843 section 6.3.1 | Option 1 in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | wrap round model | wrap round model |

In addition, other simulation configurations used to generate the dataset are listed in Table 2, including the number of UEs, UE placement and dataset splitting method.

Table 2: Other simulation configurations

| Parameter | Value for FR1 and FR2 |
| --- | --- |
| UE number | 20 in total |
| UE placement | Uniform by cell |
| Dataset splitting | Data of 19 UEs used for training, data of the remaining UE used for testing |
| Layer 1 filtering | Non-sliding window |