3GPP TSG-RAN WG2 #128	R2-2410546

# Orlando, US, 18th – 22nd November 2024

Agenda Item:	8.6.4

Source:	Ericsson

Title:	Further considerations for conditional LTM

Document for:	Discussion

# 1	Introduction

After the RAN plenary in September, the following objective has been agreed in the WID in [1] for conditional LTM:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this contribution we will address several aspects which are necessary to discuss after the first agreements during RAN2#127bis.

# 2	Discussion

## 2.1	CLTM configuration

Conditional LTM can be seen as an enhancement of the LTM feature specified in Rel-18 and in line with this it would be good to discuss how the legacy LTM candidate configurations relate to Conditional LTM candidate configuration. We see at least two options:

- Option 1:  A maximum of 8 conditional and non-conditional LTM candidate configurations can be configured by the network. This means that the 8 LTM candidate configurations which have been specified in Rel-18 is the total number of both non-conditional (Rel-18 LTM) and conditional LTM candidate configurations.

- Option 2: A maximum of 8 conditional LTM candidate configurations can be configured by the network, which are independent from the 8 non-conditional LTM candidate configurations.

Either option work but of course both have pro and cons. For instance, with Option 1 there is less flexibility in configuring LTM and CLTM as the number of candidate configuration will be limited. On the other side, Option 2 give more flexibility in configuring LTM and CLTM, but it would require extensive effort in adapting the existing procedure that have been specified for LTM since Rel-18. Since in Rel-19 the time is limited to finish the objective, in order to limit the standardization efforts, it would be nice to re-use what has been done for LTM in Rel-18. According to this, we propose:

A maximum number of 8 candidate configurations (LTM plus CLTM candidate configurations) can be configured.

The LTM-Candidate IE is re-used to model an CLTM candidate cell configuration.

## 2.2	Early synchronization for CLTM

### 2.2.1	Early UL synchronization

Given the agreements made in the last RAN2#127bis meeting, it was agreed that the PDCCH order solution would be re-used as baseline to initiate the early UL synchronization procedure. However, an open question is how to deliver the TA to the UE, since in CLTM the LTM cell switch MAC CE is not used. According to this, in order to mimic the behavior that exists in Rel-18, a straightforward solution would be for the TA to be delivered also for conditional LTM by the serving DU. Nevertheless, since there is no LTM cell switch MAC CE in case of conditional LTM, the TA could be delivered in a new MAC CE. This will also make the work much easier for RAN3, since the signaling over F1 that has been specified in Rel-18 can be completely re-used. According to this, we propose:

For CLTM, the TA value for the early UL synchronization procedure is delivered by the serving DU.

For CLTM, the serving DU delivers the TA value to the UE via a MAC CE (FFS whether a new one or an existing one).

Another issue that needs to be also addressed (and which is different from LTM) is how to maintain the validity of the received TA value. In LTM, since it is the serving DU which delivers the TA within the LTM cell switch MAC CE, RAN2 has agreed that is also the serving DU which check the validity of the TA value before sending it to the UE. However, since in CLTM the UE receives the TA value before a CLTM cell switch procedure may be triggered, in this case it would make sense that the UE checks and maintain the validity of the received TA. Therefore, we propose:

For CLTM, the validity of the TA value received from the serving DU is maintained by the UE with the following principles:

  - When a TA value is received, the UE starts a Time Alignment Timer (TAT) which is specific for the CLTM candidate configuration to which the received TA value belongs. How the TAT value is signaled is FFS.

  - If the TAT has expired before the execution of a CLTM cell switch, the UE executes a RACH-based CLTM cell switch.

  - if the TAT is still running upon the execution of a CLTM cell switch, the UE executes a RACH-less CLTM cell switch and the TAT is not re-started.

  - If a new TA value is received before the execution of a CLTM cell switch, the UE restarts the TAT for the CLTM candidate configuration to which the TA value belongs (if the TAT is running).

An additional aspect to be considered is, once the TAT expires before a CLTM cell switch procedure is initiated, how a new TA value could be provided to the UE. In few words, this means how the early UL synchronization procedure can be restarted. According to this, there are basically two options for the network to know that a new TA value should be provided to the UE:

    - When to trigger again the early UL synchronization procedure (in case TA value at the UE is not valid anymore) is up to the network implementation.

    - In case TA value at the UE is not valid anymore, UE will inform the network that the TAT has expired, and it is up to the network whether to trigger a new early UL synchronization procedure.

For simplicity, for CLTM we can adopt solution 1 as is the simplest and basically with no standardization impact. Thus, we propose:

When to re-trigger the early UL synchronization procedure (in case TA value at the UE is not valid anymore) is up to network implementation.

### 2.2.2	Early DL synchronization

In the last RAN2 meeting it was also agreed that the Rel-18 procedure for the early DL synchronization will also be re-used for CLTM. According to this, the only open issue remaining about this aspect is how the TCI states for a CLTM candidate cell are activated/deactivated at the UE. For this, it would be straightforward to completely re-use the Candidate Cell TCI States Activation/Deactivation MAC CE that has been specified in Rel-18.

For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

### 2.2.3	Unified UL/DL early synchronization signalling

If a MAC CE is specified to deliver the TA value to the UE for CLTM, a possible enhancement to reduce the signalling over the air would be to unify the UL/DL early synchronization signaling. This means that the new MAC CE which we could use to send the TA value to the UE could also be used for activating/deactivating the candidate TCI state (s) for a CLTM candidate cell configuration. Thus, we propose:

If a new MAC CE is specified to deliver the TA value to the UE, RAN2 to discuss whether this MAC CE can be used to unify the UL/DL early synchronization signaling (one MAC CE used to deliver the TA and activate/deactivate the candidate TCI state (s) for a CLTM candidate cell configuration).

## 2.3	CLTM cell switch execution

One FFS from the last meeting is whether the DG can be used for CLTM:

10. For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

About this issue, we see no problem in allowing the UE to use a DG grant when a CLTM cell switch procedure is initiated. Relying on the dynamic grant, will allow the network to lighten the big burden of reserving resource in advance for a time that could be particularly long and for which the network has no control. Pre-reserving radio resource for a candidate cell has been an issue since when CHO was specified, and it is something that was also discussed during LTM in Rel-18.

Reserving radio resource for an LTM/CLTM candidate cell is challenging for the network.

Therefore, allowing the network to rely on the dynamic grant will bring benefits both on the network (which does not need to keep resource reserved for long time) and UEs (which can use some of those resources that would be otherwise reserved for CLTM). It is worth noticing that the use of a dynamic grant implies also that the BSR/SR mechanism can also be used by the UE. Therefore, we propose:

For RACH-less CLTM, DG-based (i.e., BSR/SR mechanism) for first UL transmission on target CLTM candidate cell is supported.

## 2.4	Using L3 conditions for CLTM

One of the topics that was discussed in the last RAN2 meeting was whether CLTM can use (or re-use) L3 measurement and evaluation conditions. While it would be possible to use existing L3 evaluation condition specified for CHO as baseline, a concern is related about the workload that this would bring to RAN2 if the LTM execution conditions would be based on both L1 and L3 measurements. Nevertheless, since the use of L3 measurements is already supported for LTM in Rel-18, it would be good to keep CLTM and LTM aligned in this sense since would make also implementation of CLTM and LTM a bit easier (since signalling for LTM can be re-used for CLTM). However, RAN2 should strive to support LTM evaluation condition with L3 measurements with minimum specification impact. According to this, we propose:

LTM evaluation conditions based on L3 measurements are supported but RAN2 should strive to support this with minimum standardization impact.

## 2.4	Conditional LTM and DC scenarios

In the last RAN plenary, it was discussed whether to support conditional LTM in DC scenarios and, if this was the case, in which particular scenarios. If we go back to Rel-18, when LTM was specified RAN2 supported the coexistence of LTM with DC but only according to some restrictions. In fact, LTM with DC was supported only according to the following:

- Only intra-CU LTM is supported at the MN and SN

- LTM is configured independently at the MN and SN

- In case, of LTM at the MN, the SN can either be kept or released

- PCell change with PScell change with LTM is not possible

- No security change is supported with LTM

These are the principles that we took into account in Rel-18 and, since the scenarios which are going to be supported for conditional LTM are basically the same what we have done for the normal LTM, a straightforward way to support conditional LTM in DC scenarios would be to re-use the following principles. According to this, we propose:

Conditional LTM is supported with DC according to the following principles (same as in Rel-18 for intra-CU LTM):

  - Only intra-CU conditional LTM is supported at the MN and SN

  - Conditional LTM is configured independently at the MN and SN

  - In case of a conditional LTM cell switch at the MN, the SN can either be kept or released

  - PCell change with PSCell change is not supported with conditional LTM

  - No security change is supported with conditional LTM

FFS whether NR-DC enhancements for inter-CU LTM which are applicable for intra-CU LTM can also be considered.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Reserving radio resource for an LTM/CLTM candidate cell is challenging for the network.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A maximum number of 8 candidate configurations (LTM plus CLTM candidate configurations) can be configured.

Proposal 2	The LTM-Candidate IE is re-used to model an CLTM candidate cell configuration.

Proposal 3	For CLTM, the TA value for the early UL synchronization procedure is delivered by the serving DU.

Proposal 4	For CLTM, the serving DU delivers the TA value to the UE via a MAC CE (FFS whether a new one or an existing one).

Proposal 5	For CLTM, the validity of the TA value received from the serving DU is maintained by the UE with the following principles:

a.	When a TA value is received, the UE starts a Time Alignment Timer (TAT) which is specific for the CLTM candidate configuration to which the received TA value belongs. How the TAT value is signaled is FFS.

b.	If the TAT has expired before the execution of a CLTM cell switch, the UE executes a RACH-based CLTM cell switch.

c.	if the TAT is still running upon the execution of a CLTM cell switch, the UE executes a RACH-less CLTM cell switch and the TAT is not re-started.

d.	If a new TA value is received before the execution of a CLTM cell switch, the UE restarts the TAT for the CLTM candidate configuration to which the TA value belongs (if the TAT is running).

Proposal 6	When to re-trigger the early UL synchronization procedure (in case TA value at the UE is not valid anymore) is up to network implementation.

Proposal 7	For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

Proposal 8	If a new MAC CE is specified to deliver the TA value to the UE, RAN2 to discuss whether this MAC CE can be used to unify the UL/DL early synchronization signaling (one MAC CE used to deliver the TA and activate/deactivate the candidate TCI state (s) for a CLTM candidate cell configuration).

Proposal 9	For RACH-less CLTM, DG-based (i.e., BSR/SR mechanism) for first UL transmission on target CLTM candidate cell is supported.

Proposal 10	LTM evaluation conditions based on L3 measurements are supported but RAN2 should strive to support this with minimum standardization impact.

Proposal 11	Conditional LTM is supported with DC according to the following principles (same as in Rel-18 for intra-CU LTM):

a.	Only intra-CU conditional LTM is supported at the MN and SN

b.	Conditional LTM is configured independently at the MN and SN

c.	In case of a conditional LTM cell switch at the MN, the SN can either be kept or released

d.	PCell change with PSCell change is not supported with conditional LTM

e.	No security change is supported with conditional LTM

Proposal 12	FFS whether NR-DC enhancements for inter-CU LTM which are applicable for intra-CU LTM can also be considered.

# 4	References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #105, Melbourne, Australia, Sep 9-12, 2024.