3GPP TSG-RAN WG2 Meeting #128		R2-2410571

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item	: 8.6.3

Source	: Fraunhofer HHI, Fraunhofer IIS

Title	: Open issues for L1 event triggered measurement reporting

Document for	: Discussion & Decision

# Introduction

RAN2#127bis agreed the following [1]:

- MR can be sent when the leaving condition is met, based on NW configuration. - Event triggered periodic MR can be supported, based on NW configuration. - For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3. - For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed. - For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed. - The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1. - TTT operates only based on a timer (like TTT used in L3 event triggered MR). - Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). - Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) - MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). - N is configurable by NW. - Additional information included in MR MAC CE: - The information and quantity of current beam, based on NW configuration. - The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. - NW can configure a dedicated SR configuration for MR MAC CE transmission.

RAN2#127 agreed the following [2]:

- Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. - R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. - MAC layer handles the event evaluation and measurement report triggering. - Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

RAN2#126 agreed the following [3]:

1. Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. 2. For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. 3. Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. 4. Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. 5. RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. 6. For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

This paper outlines the open issues and proposes solutions to address them.

# Discussion

## Measurement reference signal type alignment

3GPP has agreed to support both SSB and CSI-RS for Layer 1 measurement resource configuration in LTM configurations. These reference signals serve different purposes and have distinct characteristics. SSB is primarily used for evaluating cell quality over wider beams, typically suitable for initial cell search and broad coverage assessments. On the other hand, CSI-RS provides higher resolution measurements due to its use of narrower beams, which makes it more effective for precise channel state information. Given these differences, the NW should configure the UE with both SSB and CSI-RS to enable effective LTM event evaluation.

Proposal 1: UE is configured with both SSB and CSI-RS for event evaluation purposes.

Therefore, the UE needs to compare measurements taken from different types of reference signals, such as CSI-RS and SSB, especially in dynamically changing beam quality scenarios. To address this, we propose introducing an offset that enables the UE to evaluate one reference signal type in relation to another. This will ensure consistency in the measurement process and accurate comparison of serving and candidate beams. By using this offset, the UE can make more informed decisions.

Proposal 2: UE is configured with an offset which enables it to assess one reference signal type in relation to another.

If the event conditions are met during the TTT period, the UE can report the offset used for the evaluation to the network. This ensures that the network has visibility into the offset applied for the comparison, allowing for more accurate handover decision-making based on the measurement results.

Proposal 3: Offset is used for event evaluation is reported to the network.

## TTT management

In RAN2#127, it was agreed that the current beam, corresponding to the TCI state, is used for event evaluation in L1 measurement reporting for the serving cell. In the L3 Event Triggered mechanism, the TTT timer is stopped once the leaving condition is satisfied, and the UE can report this information to the network. However, the quality of the L1 beam may fluctuate more dynamically than that of L3 measurements. The current beam of the serving cell can change via the indicated TCI state during the TTT duration, which poses a risk of the TTT timer being reset too frequently, potentially leading to delayed handovers or radio link failure. Therefore, it is crucial to prevent TTT reset when the current beam changes, to avoid the negative impact of resetting the timer too often and to reduce the likelihood of late handover.

Proposal 4: Do not reset TTT when the current beam changes.

Additionally, the candidate beam could also change dynamically during the measurement process since the candidate beam could be configured with CSI-RS, which uses narrower beams compared to SSB. Therefore, there is no need for different TTT reset conditions for the current beam and candidate beam. Both should be handled with the same approach to avoid unnecessary complexity.

Proposal 5: The same TTT reset conditions are applied to both the current and candidate beams.

Since the legacy L3 TTT mechanism was designed for L3 measurements, it may not be directly applicable to L1 measurements, which are subject to more dynamic fluctuations. Therefore, the TTT mechanism must be adapted to better handle these fluctuations in L1 measurements by adding appropriate reset conditions or exceptions.

Proposal 6: The legacy TTT mechanism requires modification.

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: UE is configured with both SSB and CSI-RS for event evaluation purposes.

Proposal 2: UE is configured with an offset which enables it to assess one reference signal type in relation to another.

Proposal 3: Offset is used for event evaluation is reported to the network.

Proposal 4: Do not reset TTT when the current beam changes.

Proposal 5: The same TTT reset conditions are applied to both the current and candidate beams.

Proposal 6: The legacy TTT mechanism requires modification.

# References

[1] Chair notes, 3GPP RAN1#127bis, Hefei, China, Oct 14th – 18th, 2024.

[2] Chair notes, 3GPP RAN2#127, Maastricht, NL, August 19th – 23rd, 2024.

[3] Chair notes, 3GPP RAN2#126, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.