3GPP TSG-RAN WG2 Meeting #128	   R2-2410574

Orlando, USA, Nov.  18th – 22nd , 2024

Agenda Item	:	8.11.2

Title	:	Random Access in Sub-Band Full Duplex

Source	:	Google

Document for	:	Discussion and Decision

# Introduction

In RAN#104 [1], the following Rel-19 objective has been captured:

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

In this Tdoc, we share our views on the SBFD operation to support random access in SBFD symbols by UEs in RRC _CONNECTED mode and RRC_IDLE/INACTIVE mode.

# Random Access in Sub-band full duplex

- Support of 2-step RACH in SBFD operation

In RAN1#116, the following agreement has been made regarding the support of Type-1 and Type-2 random access procedure for SBFD aware UEs:

Agreement

For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols.

- FFS Type-2 random access procedure (2-step RACH)

In RAN2#127, the following agreement has been made:

RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

Given the objective of SBFD operation to improve uplink capacity and coverage while minimizing latency, the integration of 2-step RACH within SBFD symbols is a natural progression. Both mechanisms contribute synergistically to latency reduction.

- SBFD aware UEs in RRC CONNECTED and RRC IDLE/INACTIVE state support Type-2 random access procedure (2-step RACH) in SBFD symbols.

- Power control:

The existing RACH power control mechanisms might not be optimal for SBFD RACH operation. The interference levels in the SBFD symbol, where the UE transmits the RACH preamble, can be significantly different from those in regular uplink symbols. This can lead to either insufficient power for reliable detection at the gNB or excessive power causing interference to the downlink reception in the same sub-band.

Therefore, introducing separate power control parameters for SBFD ROs to account for the varying interference levels in SBFD symbols compared to regular UL symbols is essential. This would allow the network to configure the appropriate transmit power for SBFD RACH preambles, ensuring reliable detection at the gNB while minimizing interference to downlink reception.

For RACH configuration Option 2 using two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration, two separate power control parameters for each RACH configuration are needed (preambleReceivedTargetPower, powerRampingStep, and preambleTransMax in RACH-ConfigGeneric in AdditionalRACH-Config).

- For Option 2, with two separate RACH configurations, support two separate power control parameters for legacy RACH configuration and the additional SBFD RACH configuration (e.g., preambleReceivedTargetPower, powerRampingStep, and preambleTransMax in RACH-ConfigGeneric).

The UE can determine the target receive power for a PRACH transmission occasion for the legacy PRACH configuration or the additional SBFD PRACH configuration as PREAMBLE_RECEIVED_TARGET_POWER = preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA + POWER-OFFSET-SBFD, where the preambleReceivedTargetPower is configured for both PRACH configurations or for the legacy PRACH configuration, the PREAMBLE_POWER_RAMPING_STEP is configured for both PRACH configurations or the legacy PRACH configuration.

The UE may determine the power-offset-SBFD as 0 if the PRACH is based on the first PRACH configuration and determine it as power-offset-SBFD if the PRACH is based on the additional SBFD PRACH configuration. The power-offset-SBFD can be specified or can be configured as part of the additional SBFD PRACH configuration.

- An SBFD power offset can be introduced for the UE to determine a target receive power for additional ROs vs. legacy ROs.

- Switching from using additional and legacy ROs to legacy ROs only

After RACH failures on SBFD slots/symbols due to high level of CLI, the UE could be instructed to switch from using additional ROs on SBFD slots/symbols to legacy ROs on non-SBFD slots/symbols. The UE could instruct to switch from additional ROs to legacy ROs or to stop using additional ROs.

Alternatively, the use of additional ROs could be dynamically activated/deactivated by the base station.

Some other rules could be defined for the UE to select the additional ROs or the legacy ROs, for example the number of RACH failures, the channel conditions, the level of CLI, the level of pathloss (e.g., be defining pathloss threshold(s) , …), UE speed/doppler, handover types, …

- The base station can instruct the UE or rules can be defined for the UE to switch from using additional ROs and legacy ROs to using legacy ROs only due to high level of CLI, number of RACH failures, …

In RAN2#127b, the following agreement has been reached:

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

Latency is the main motivation for supporting SBFD and we think latency should be the main criterion for the selection between legacy-ROs and additional-ROs.

- Support timing-based selection - The UE can prioritize ROs based on their timing, selecting the earliest starting or ending RO to minimize latency.

# 3		Conclusions

In conclusion, we have the following proposals:

- SBFD aware UEs in RRC CONNECTED and RRC IDLE/INACTIVE state support Type-2 random access procedure (2-step RACH) in SBFD symbols.

- For Option 2, with two separate RACH configurations, support two separate power control parameters for legacy RACH configuration and the additional SBFD RACH configuration (e.g., preambleReceivedTargetPower, powerRampingStep, and preambleTransMax in RACH-ConfigGeneric).

- An SBFD power offset can be introduced for the UE to determine a target receive power for additional ROs vs. legacy ROs.

- The base station can instruct the UE or rules can be defined for the UE to switch from using additional ROs and legacy ROs to using legacy ROs only due to high level of CLI, number of RACH failures, …

- Support timing-based selection - The UE can prioritize ROs based on their timing, selecting the earliest starting or ending RO to minimize latency.

# 4		References

- RP-241614 ‘Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)’	Huawei, RAN#104,  June 17-20, 2024 , Shanghai, China.