3GPP TSG-RAN WG2 Meeting #128	R2-2410589

Orlando, USA, 18th – 22th November, 2024

Agenda Item:	8.13.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on service continuity for Multi-hop Relay

Document for:	Discussion and Decision

# Introduction

In RP-242349, we have the following description for service continuity scenarios for multi-hop U2N relay.

Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this paper, we discuss the two scenarios in the first priority and provide the signalling flows for these two scenarios.

# Discussion

## 2.1 Service continuity procedures

In the following subsection, we describe the stage-2 procedures for:

- Intra-gNB multi-hop indirect to direct path switching, and

- Intra-gNB multi-hop indirect to single-hop indirect path switching,

## 2.1.1 Intra-gNB multi-hop indirect to direct path switching

Figure 1 shows the procedure for the intra-gNB multi-hop indirect to direct path switching case which is extended from single hop intra-gNB  indirect to direct path switching procedure as defined in clause 16.12.6.1 in TS 38.300.

Figure 1.  Intra-gNB multi-hop indirect to direct path switching procedure

Step 1.	Remote UE triggers the measurements report when the sidelink measurement quantity between First Relay UE and Remote UE indicates deteriorated link quality. The triggering Event X1 and Event X2 can be reused in the measurement configuration. The Uu measurements of neighbour cells can be included in the measurements report.

- Event X1: Serving L2 U2N Relay UE becomes worse than threshold1 and NR Cell becomes better than threshold2

- Event X2: Serving L2 U2N Relay UE becomes worse than threshold

Proposal 1: The triggering Event X1 and Event X2 can be adopted and reused in the measurement configuration for the multi-hop U2N relay.

Step 2.	The gNB decides to switch the L2 U2N Remote UE onto direct Uu path.

Step 3.     The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The L2 U2N Remote UE stops User Plane and Control Plane transmission via the First Relay UE after reception of the RRCReconfiguration message with the path switch configuration.

Step 4.	The Remote UE performs Random Access procedure to gNB.

Step 5.	The UE (i.e., L2 U2N Remote UE in previous steps) sends the RRCReconfigurationComplete message to the gNB via the direct path, using the configuration provided in the RRCReconfiguration message. From this step, the UE (i.e., L2 U2N Remote UE in previous steps) uses the RRC connection via the direct path to the gNB .

Step 6.	The gNB sends the RRCReconfiguration message to the Relay UE(s) (last relay, intermediate relay and the first relay UE) to reconfigure the connection between the Relay UEs and the gNB. The RRCReconfiguration message to the L2 U2N Multi-hop Relay UEs can be sent any time after step 3 based on gNB implementation (e.g., to release Uu Relay RLC Channel and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE ).

Observation 1: First relay UE, intermediate  relay UE and last relay UE of this remote UE may serve other remote UE. If not, the release of the link between the relay UEs will be triggered.

Step 7.	Either L2 U2N First Relay UE or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation .

Step 8.	The data path is switched from multi-hop indirect path to direct path between the UE (i.e., previous L2 U2N Remote UE) and the gNB. The PDCP re-establishment or PDCP data recovery in uplink is performed by the UE (i.e., previous L2 U2N Remote UE) for lossless delivery during path switch if gNB configures it .

NOTE 1:	Step 8 can be executed any time after step 4. Step 8 is independent of step 6 (includes 6-1、6-2 and 6-3) and step 7.

Proposal 2: Agree the above procedure in Figure 1 as a baseline for intra-gNB multi-hop indirect to direct path switching procedure to be captured in TS 38.300.

## 2.1.2 Intra-gNB multi-hop indirect to single-hop indirect path switching

Figure 2 shows the intra-gNB multi-hop indirect to single-hop indirect path switching procedure, which is extended from  switching indirect to indirect path as defined in clause 16.12.6.3 in TS 38.300.

Figure 2.  Intra-gNB multi-hop indirect to single-hop indirect path switching procedure

Step 1.	 Remote UE reports one or multiple candidate Relay UE(s), when the sidelink measurement quantity between First Relay UE and Remote UE or Relay UE and next/last-hop Relay UE shows deteriorated link quality. The triggering Event Z1 and Event Z2 (if agreed) can be used in the measurement configuration. The reporting results includes at least the Relay UE ID (source Layer-2 ID of Relay UE), Relay UE’s serving cell ID, and sidelink measurement quantity (SD-RSRP derived by measuring the discovery message broadcasted by the Relay UE).

- Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2

- Event Z2 (if agreed): Candidate L2 U2N Relay UE becomes an offset better than the serving L2 U2N Relay UE

Proposal 3: The triggering Event Z1 and Event Z2 (if agreed) can be adopted and reused in the measurement configuration for the multi-hop U2N relay.

Step 2.	The gNB decides to switch the L2 U2N Remote UE to a target L2 U2N Relay UE under the same gNB.

Step 3.    The gNB sends an RRCReconfiguration message to the target L2 U2N Relay UE, which includes at least the L2 U2N Remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

Step 4.   The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the target L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic, and the associated end-to-end radio bearer(s). The L2 U2N Remote UE stops UP and CP transmission over the multi-hop indirect path via the source L2 U2N multi-hop relay after the reception of the RRCReconfiguration message from the gNB.

Step 5.     The L2 U2N Remote UE establishes PC5-RRC connection with the target L2 U2N Relay UE.

Step 6.	The L2 U2N Remote UE sends the RRCReconfigurationComplete message to the gNB via the target L2 U2N Relay UE, using the configuration provided in the RRCReconfiguration message. From this step, the L2 U2N Remote UE uses the RRC connection via the target L2 U2N Relay UE to the gNB .

Step 7.  The gNB sends the RRCReconfiguration message to the source L2 U2N multi-hop relay to reconfigure the connection between the source L2 U2N multi-hop relay UEs and the gNB. The RRCReconfiguration message to the source L2 U2N multi-hop relay UEs can be sent any time after step 4 based on gNB implementation (e.g., to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

Observation 2: First relay UE, intermediate  relay UE and last relay UE of this remote UE may serve other remote UE in other links. If not, the release of the link between the relay UEs will be triggered.

Step 8.	Either source First L2 U2N Relay UE's AS layer or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation after step 4 or step7.

Step 9.	The data path is switched from the source L2 U2N Relay UE to the target L2 U2N Relay UE between the L2 U2N Remote UE and the gNB. This step can be any time after step 7.

Proposal 4: Agree the above procedure in Figure 2 as a baseline for intra-gNB multi-hop indirect to single-hop indirect path switching to be captured in TS 38.300.

# Conclusion

In this paper, we discuss the two scenarios in the first priority and provide the signalling flows for these two scenarios. We have the following observation and proposals

.

Service Continuity Procedures

Intra-gNB multi-hop indirect to direct path switching

Proposal 1: The triggering Event X1 and Event X2 can be adopted and reused in the measurement configuration for the multi-hop U2N relay.

Observation 1: First relay UE, intermediate  relay UE and last relay UE of this remote UE may serve other remote UE. If not, the release of the link between the relay UEs will be triggered.

Proposal 2: Agree the above procedure in Figure 1 as a baseline for intra-gNB multi-hop indirect to direct path switching procedure to be captured in TS 38.300.

Intra-gNB multi-hop indirect to single-hop indirect path switching

Proposal 3: The triggering Event Z1 and Event Z2 (if agreed) can be adopted and reused in the measurement configuration for the multi-hop U2N relay.

Observation 2: First relay UE, intermediate relay UE and last relay UE of this remote UE may serve other remote UE in other links. If not, the release of the link between the relay UEs will be triggered.

Proposal 4: Agree the above procedure in Figure 2 as a baseline for intra-gNB multi-hop indirect to single-hop indirect path switching to be captured in TS 38.300.

# Reference

[1] 3GPP TS 38.300, V18.3.0, NR; NR; NR and NG-RAN Overall Description

[2] RP-242349 Revised WID SL_relay_multihop