3GPP TSG-RAN WG2 #128	R2-2410593

Orlando, USA, Nov. 18th– 22nd, 2024

Agenda Item:	8.2.4

Source:	KT Corp.

Title:	Discussion on Ambient IoT random access

Document for:	Discussion

WID/SID:	FS_Ambient_IoT_solutions

1	Introduction

The RAN2 objectives for Ambient IoT was approved as in SID [1].

- RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential.

In RAN2#126 meeting, the following agreements related to contents in paging message and “4-step, 2-step CBRA” were made [2]:

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

RAN2#126 Agreements on “4 step” RA 1 A-IoT Msg1: the device sends an ID to the reader. ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID). FFS on ID size. This doesn’t preclude any other RAN1 agreed information 2 A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements 3 A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request) 4 The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose. 5 “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed. RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA 1 A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed. This doesn’t preclude any other RAN1 agreed information 2 A-IoT Msg2: the reader may echo some information from Msg1. FFS what some information is. “Msg2” usage/presence can be further discussed.

Agreement - From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

In RAN2#127, the following agreements related to resource allocation indication aspects and contents per each msg2, msg3 were made briefly [2], [3]:

RAN2#127 Agreements 1 RAN2 assumes that Msg3 transmission resource can explicitly be indicated in Msg2 (based on RAN1 agreements/design). Wait for RAN1 for further details. 2 For 3-step CBRA Support fixed random ID size is 16 bit. The ID is randomly generated. 3 Failure/success indication of D2R will be studied. FFS if it would be implicit or explicit and for which use case it is needed. FFS whether it is applied only to some cases. 4 For 2step CBRA, RAN2 design will support msg2. Whether it is needed it is up to the reader. FFS when it is needed. For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2. FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices. 5 In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2). FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes. 6 FFS if reader assigns the AS ID for scheduling purposes.

In previous RAN2#127-bis, the following agreements related to energy status indication and device information in paging message, and re-access in CBRA were made briefly [2], [3]:

Agreements on energy bit indication Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can. 1 bit indication can be captured in the TR. We will capture the use case in the TR. FFS whether it is reader controlled.

Agreements - The A-IoT paging message can include information to avoid duplicate response from the device to a reader. This information should be short and simple. FFS how to indicate. Wait for further information and requirements from other WGs and make this decision in normative phase. - Based on this information the device determines whether to skip sending the response to paging. - From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs. - If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

Agreements for Random Access - In case of D2R data transmission failure, device follows the reader instruction:  it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and  reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R). 2 Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose. FFS for following D2R data. 3 For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access. The re-access is always controlled by reader. 4 For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure. 5 R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message. FFS additional indications to differentiate if needed in paging message. FFS if other message can be used. 6 From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA. FFS if this is applicable to FDMA. Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

In this contribution, we would like to share our view on A-IoT device’s random access decision and resource allocation briefly.

2	Discussion

As described above, RAN2 has been discussing the A-IoT RA (Random Access) procedure so far. The 3-step, 2-step CBRA (Contention Based Random Access) and CFRA (Contention Free Random Access) procedures have been defined, but the detailed specification is in progress. Below figures are the A-IoT RA procedure drawn based on the agreements reached so far.

Figure 1 shows 3-step CBRA procedure and Msg4 will be transferred if needed. Figure 2 shows 2-step CBRA procedure and Msg2 will be also transmitted when it is needed. Finally, figure 3 shows CFRA procedure. In this paper, we discuss the random-access aspects for Ambient IoT and would like to provide our opinion based on the below RA procedure figures.

Figure 1. 3-step A-IoT CBRA (or 4-step if Msg4 is present)

Figure 2. 2-step A-IoT CBRA

Figure 3. 2-step A-IoT CBRA

2.1	Random Access type decision

According to the current RAN2 meeting agreement, A-IoT can support three RA types (3-step CBRA, 2-step CBRA, CFRA). If A-IoT devices have capability for all three RA types, there is no agreement yet on which RA type the devices select. In this case, the reader can explicitly inform the device of the RA type. Also, the device can implicitly select the RA type based on the information in the paging message and other information.

Device can determine one RA type among three (or more) RA types explicitly or implicitly based on the information in A-IoT paging message.

If reader include the RA type information in the A-IoT paging message explicitly, A-IoT device can determine the RA type as is. However, if the RA type is not specified in A-IoT paging message, the RA type can be implicitly selected based on the information contained in the paging message.

As shown in below agreements and above figures, A-IoT paging message may contain A-IoT Device ID information with four options as follows:

1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

The RA type can be set based on the A-IoT device ID information included in the A-IoT paging message. For example, when receiving a paging message containing a single A-IoT device ID or multiple A-IoT device IDs, the A-IoT devices can select CFRA. In this case, there is no agreement yet on whether dedicated or shared resources will be included. If the paging message does not contain device ID information or contains a group ID, the devices can select CBRA and transmit Msg1 with a random ID.

Additionally, in previous meeting, it was agreed that the device may include 1 bit energy status indication in D2R messages (e.g. MSG1, MSG3 and Command Response message), which can be captured in the TR. But, if it device can transmit current energy status or level by using bits more than 1 bit, it would be more useful for taking care of them. And device can implicitly select RA type depending on the situation.

for example, the device can select the RA type by considering its own residual energy level. Based on its own residual energy awareness, the device can select 2-step if the current residual energy is lower than the threshold value. If the A-IoT device can transmit its energy level or energy status information in msg1 or msg3, the reader can send relevant information for energy harvesting through msg2 or msg4.

Device can implicitly select RA type among three (or more) RA types based on the types of A-IoT device ID and/or current energy level.

RAN2 to study for including energy level/status (more than 1 bit) information to msg1 or msg3.

2.2	Radio resource allocation

In case of CFRA, shared or dedicated radio resource information for Msg1(First D2R) may be included A-IoT paging message. If shared resource information is included in paging message, device implicitly select radio resources based on the A-IoT device ID information in paging message. For example, A-IoT devices can select time/frequency resource information in order of the size of the device ID in the paging message. RAN1 is also considering supporting CDM(A), so A-IoT devices can select code sequences based on the above method.

Device can select explicit or implicit radio resource based on the A-IoT device ID information and any other information in paging

3	Conclusion

We made the following observations:

Proposal 1	Device can determine one RA type among three (or more) RA types explicitly or implicitly based on the information in A-IoT paging message.

Proposal 2	Device can implicitly select RA type among three (or more) RA types based on the types of A-IoT device ID and/or current energy level.

Proposal 3	RAN2 to study for including energy level/status (more than 1 bit) information to msg1 or msg3.

Proposal 4	Device can select explicit or implicit radio resource based on the A-IoT device ID information and any other information in paging

4	References

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR,” RAN #103, March 18-21, 2024.

RP-242405, “Status Report for New SID: Study on solutions for Ambient IoT (Internet of Things) in NR,” Sept 9-12, 2024.

R2-24079xx, “Draft Report of 3GPP TSG RAN WG2 meeting #127,” RAN WG2#127, 19 - 23 August, 2024.

TR 38.848, “Study on Ambient IoT (Internet of Things) in RAN (Release 18)” V18.0.0 (2023-09)

Chair’s note RAN2 #127-bis