3GPP TSG-RAN WG2 Meeting #128	R2-2410595

# Orlando, USA, Nov. 18th – 22nd, 2024

Agenda item:		8.2.5 (FS_Ambient_IoT_solutions)

Source:	LG Electronics Inc.

Title: 	Discussion on topology 2 considerations of ambient IoT

Document for:	Discussion and Decision

1.	Introduction

This document discusses topology 2 considerations of ambient IoT (A-IoT). In this contribution, we discuss validity of resources used in A-IoT air interface between UE reader and A-IoT devices in topology 2. The related agreements in R2-127bis meeting are as follows.

Agreements 1 Dedicated resource configuration is only given to the UE reader via dedicated signalling. Mechanisms for shared resource pool amongst readers are not considered in this release. 2 The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control. FFS how valid resource is determined – we will consider temporary out of connection scenarios. FFS on resource validity across multiple cells. 3 The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

2.	Discussion

According to the agreements in R2-127bis meeting a UE reader (i.e., intermediate UE) is configured with the dedicated resources used in A-IoT air interface between the UE reader and the A-IoT devices, and the UE reader performs the A-IoT procedure on the A-IoT interface only if the resource configuration is valid. However, it has not been concluded yet how valid resource is determined and whether the resource may be valid across multiple cells or not. In this document, we would like to discuss validity of resources used in UE reader’s A-IoT air interface.

Firstly, we think, the RRC state of a UE reader needs to be taken into account to determine validity of the A-IoT radio resource for the UE reader. If the UE reader is in RRC_CONNECTED or RRC_INACTIVE, the resource is considered to be valid. On the other hand, if the UE reader is in RRC_IDLE, the resource should be invalid or released.

The network is in control of resources to be used by A-IoT air interface, which is agreed in R2-127 meeting. When an A-IoT radio resource for a UE reader is valid, the network or gNB has to be able to control it. If the UE reader is in RRC_CONNECTED or RRC_INACTIVE, the network or gNB is aware of which radio resources are configured to the UE reader for A-IoT air interface and there are active or suspended radio bearers between the UE reader and the gNB. When network control is required, the resources can be reconfigured by network via dedicated signaling immediately or after a connection resume. On the other hand, if the UE reader is in RRC_IDLE, gNB is not aware of which radio resources are configured and whether they are valid or not for the UE reader in RRC_IDLE because gNB has no information about the UE reader. It is noted that the UE context is released when a UE goes to RRC_IDLE. Furthermore, there is no connection between the UE reader and the gNB to send dedicated signaling for network control.

Proposal 1. A-IoT radio resource used by a UE reader is released or considered to be invalid when the UE reader is in RRC_IDLE.

Secondly, it is further discussed how the UE reader determines this A-IoT radio resource validity in its temporary out of connection scenarios (e.g., RLF and handover cases). In a temporary out of connection scenario, the UE reader is still in RRC_CONNECTED and gNB knows the A-IoT radio resource which it configures to the UE reader. In addition, the position of the UE reader may not be much different with the position where it was located just before the temporary out of connection scenario. Therefore, we think that the A-IoT radio resource can be used and considered as valid for the on-going A-IoT procedure considering that the A-IoT resource is a dedicated resource. The dedicated A-IoT resource can be used and valid nearby area without interference with neighboring UE readers because the interference is already taken into account when dedicated resources are configured to the UE reader and neighboring UE readers.

Proposal 2. A-IoT radio resource is valid and used for the on-going A-IoT procedure when the UE-reader is in a temporary out of connection scenario (e.g., RLF and handover cases).

By the way, the Uu interface of the UE reader is not properly working in a temporary out of connection scenario. It means that the UE reader does not receive new A-IoT messages including A-IoT service requests from the network. Therefore, it seems a straightforward conclusion that a new A-IoT procedure cannot be initiated in a temporary out of connection scenario.

Proposal 3. Confirm that a UE reader is not expected to receive a new A-IoT service request and not expected to initiate a new A-IoT procedure when the UE reader is in a temporary out of connection scenario (e.g., RLF and handover cases).

Thirdly, it is discussed on the resource validity across multiple cells. A dedicated A-IoT resource is configured to the UE reader via dedicated signalling and it can be (re)configured during RRC reconfiguration. Assuming that the dedicated A-IoT resource configuration is valid in a single cell only (e.g., the cell in which the configuration is received), no critical issue is found compared to the case that the resource is valid across multiple cells. One difference is that the number of signalling message may be reduced in a certain case, for example, the resource can be used for a long duration and the UE reader mobility is relatively high. On the other hand, network needs to reserve the A-IoT radio resource in multiple cells. It may cause resource wastes. Based on the above-mentioned analysis, it is preferred that an A-IoT radio resource configuration is valid in a single cell only (e.g., the cell in which the configuration is received).

Observation 1. If an A-IoT radio resource is considered to be valid across multiple cells, network may need to reserve the A-IoT radio resource in multiple cells. It may cause resource wastes.

Proposal 4. An A-IoT radio resource configuration is valid in a single cell (e.g., the cell in which the configuration is received). i.e., A-IoT radio resource validity across multiple cells is not considered.

3.	Conclusion

This document discusses topology 2 considerations of ambient IoT (A-IoT). We discuss validity of resources used in A-IoT air interface between UE reader and A-IoT devices in topology 2.

Proposal 1. A-IoT radio resource used by a UE reader is released or considered to be invalid when the UE reader is in RRC_IDLE.

Proposal 2. A-IoT radio resource is valid and used for the on-going A-IoT procedure when the UE-reader is in a temporary out of connection scenario (e.g., RLF and handover cases).

Proposal 3. Confirm that a UE reader is not expected to receive a new A-IoT service request and not expected to initiate a new A-IoT procedure when the UE reader is in a temporary out of connection scenario (e.g., RLF and handover cases).

Observation 1. If an A-IoT radio resource is considered to be valid across multiple cells, network may need to reserve the A-IoT radio resource in multiple cells. It may cause resource wastes.

Proposal 4. An A-IoT radio resource configuration is valid in a single cell (e.g., the cell in which the configuration is received). i.e., A-IoT radio resource validity across multiple cells is not considered.