3GPP TSG-RAN WG2 Meeting #128	R2-2410617

Orlando, USA, 18th – 22nd November, 2024

Agenda Item:	7.0.2.9

Source:	Huawei, HiSilicon

Title:	Discussion on LS from RAN1 on the time period for R18 preamble repetition

Document for:  Discussion and Decision

1	Introduction

RAN1 sent a LS to ask for configuration clarification for time period introduced in Rel-18 coverage enhancement in [1]. In this contribution, we discuss the RAN2 understandings in respond to the question mentioned in the LS.

2	Discussion

RAN1 introduced a time period parameter which is used to determine the available RO set for the Msg1 repetition and made the following agreements. Note that currently this parameter is configured per FeatureCombinationPreambles, i.e. per repetition number. However, it is not clear in RAN1 discussions that how to interpret “all the configured number of Msg1 repetitions” in case multiple repetition numbers are indicated by RRC and corresponding RACH resource and parameters are configured.

RAN1#114 Agreement For the number of SSB-to-RO association pattern periods K within the time period X, - For multiple PRACH transmissions with different numbers, support One common K is implicitly determined as a minimum integer for all the configured number of multiple PRACH transmissions such that for each of NTxSSB SSBs, there is at least one RO group per each configured number of multiple PRACH transmissions consisting of ROs associated with the SSB.

In the RAN1 LS, the following cases are raised for RAN2 to clarify the ambiguity, which are applied to the determination of the common time period, including Case#1, Case#1-rev and Case#2.

Case#1: Repetition number n2 and n4 are configured in one RACH-ConfigCommon, while n8 is configured in another RACH-ConfigCommon. All these repetition numbers are associated to one feature combination(i.e. msg1-repetition-r18). Figure 1 - Case#1-rev: the same as the case#1 except that the feature combination msg1-repetition-r18 in the second RACH-ConfigCommon is replaced with msg1-repetition-r18+redCap-r17. Case#2: Only single RACH-ConfigCommon is configured with multiple feature combinations. As shown in Figure 2, repetition number n2 and n4 are associated to one feature combination(i.e. msg1-repetition-r18), n8 is associated to another feature combination(i.e. msg1-repetition-r18+redCap-r17). Figure 2

In the following subclauses, we will discuss RAN1 questions one by one with respect to above cases mentioned in LS.

2.1 Q0

Q0: Has the case#1 above been precluded by the existing RAN2 signaling and procedures? If yes, please share the relevant RAN2 specification.

Note that Q0 is related to case 1. RACH configuration framework has been intensively discussed in RAN2 and we have made the following agreements in RAN2#123bis:

Adopt Alt 2.3 for Msg1 repetition framework Separate RO for different number is supported; - For sharedRO and separateRO case, different repetition numbers are configured via separate featureCombinationPreamble IEs only for CE. - RACH resources of RACH partitions that are configured with the same “featureCombination” are considered to be within the same set of RACH resources; - Fallback from lower number to higher number is performed within the selected set of RACH resources. - Alt1: Fallback is only supported for sharedRO case

Therefore, RACH resources for different repetition numbers can be configured in separate RO, i.e. different RACH-ConfigCommon, or shared RO, i.e. same RACH-ConfigCommon, which is reflected in the current RACH configuration framework in RRC and procedures in MAC. More specifically, Case#1 has been supported in both RRC and MAC specifications as follows:

- For RRC spec, repetition number n2, n4, n8 are indicated in three IE FeatureCombinationPreambles, while the first and second IE FeatureCombinationPreambles corresponding to n2 and n4 are associated with one RACH-ConfigCommon, and the third IE FeatureCombinationPreambles corresponding to n8 is associated with another RACH-ConfigCommon, which is configured in additional-RACH-ConfigList.

- For MAC spec, fallback from n2 to n4 can be supported given that corresponding IE FeatureCombinationPreambles are associated with one RACH-ConfigCommon, i.e. shared RO, which can be illustrated in the highlighted text.

2> if the Random Access procedure is not completed: 3> if the RA_TYPE is set to 4-stepRA: 4> if the Random Access Preamble is transmitted with repetitions and contention-free Random Access Resources have not been provided for this Random Access procedure: 5> if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or 5> if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1: 6> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available: 7> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure; 7> initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

Based on the above, RAN2 can respond with Case#1 is allowed for Q0.

Proposal 1: For Q0, RAN2 confirms that Case#1 is not precluded, i.e. Repetition number n2 and n4 are configured in one RACH-ConfigCommon, while n8 is configured in another RACH-ConfigCommon. All these repetition numbers are associated to one feature combination (i.e. msg1-repetition-r18).

2.1 Q1/Q2

Q1: Has the case#2 above been precluded by the existing RAN2 signaling and procedures? If yes, please share the relevant RAN2 specification. Q2: Has the following case#1-rev (a combination of the case#1 and case#2) been precluded by the existing RAN2 signaling and procedures? If yes, please share the relevant RAN2 specification. - Case#1-rev: the same as the case#1 except that the feature combination msg1-repetition-r18 in the second RACH-ConfigCommon is replaced with msg1-repetition-r18+redCap-r17.

On Case#2 and Case#1-Rev, the difference from Case 1 is that, n8 is associated with a different feature combination. In RAN2, we agreed CE can be applicable to RedCap and did not preclude any specific feature combinations, which is up to network implementation as long as the number of total RACH configurations is not beyond 16. Therefore, in our view, RAN2 can confirm Case#2 and Case#1-rev are both allowed.

Proposal 2: For Q1, RAN2 confirms that Case#2 is not precluded, i.e. Only single RACH-ConfigCommon is configured with multiple feature combinations. Repetition number n2 and n4 are associated to one feature combination(i.e. msg1-repetition-r18), n8 is associated to another feature combination(i.e. msg1-repetition-r18+redCap-r17).

Proposal 3: For Q2, RAN2 confirms that Case#1-Rev is not precluded, i.e. Repetition number n2 and n4 are configured in one RACH-ConfigCommon, while n8 is configured in another RACH-ConfigCommon, n8 is associated to another feature combination(i.e. msg1-repetition-r18+redCap-r17).

2.2 Q3

Q3: If the answer to Q1 is “not precluded”, according to the following specification excerpt of TS 38.331, a UE capable of msg1-repetition but incapable of RedCap ignores the RACH resource indicated by the third FeatureCombinationPreambles in the case#2. In addition to the ignored RACH resource, does it mean that the corresponding parameter repetition number n8 is also ignored, i.e. considered as “not configured” to the UE? featureCombination Indicates which combination of features that the preambles indicated by this IE are associated with. The UE ignores a RACH resource defined by this FeatureCombinationPreambles if any feature within the featureCombination is not supported by the UE or if any of the spare fields within the featureCombination is set to true..

In addition to the above RRC description, according to the current RACH procedures in MAC specification, in case the UE capable of Msg1 repetition but incapable of RedCap intends to use RACH resource sets for a RACH procedure, the UE will check the available RACH resource sets (subclause 5.1.1c) by only considering the RACH resource sets indicating feature combination msg1-repetition-r18 only as RedCap is not applicable for the RACH procedure, which is reflected in the following highlighted MAC text. Thus the UE will not consider the RACH resource corresponding to n8 when determining the RACH resource and the corresponding RACH parameters can be understood as “not configured” as mentioned in the RAN1 LS. In a word, nothing is broken for the current MAC specification.

5.1.1c Availability of the set of Random Access resources The MAC entity shall for each set of configured Random Access resources: 1> if redCap is set to true for a set of Random Access resources: 2> consider the set of Random Access resources as not available for a Random Access procedure for which RedCap is not applicable.

Proposal 4: For Q3, RAN2 confirms that in Case #2, a UE capable of msg1-repetition but incapable of RedCap will not consider the RACH resource and parameters indicated by feature combination msg1-repetition-r18+redCap-r17 according to the current RAN2 specifications.

2.3 Q4

Q4: If the answer to Q1 is “not precluded”, according to the following RAN2 agreement, a UE capable of RedCap must select the RACH resource indicated by the third FeatureCombinationPreambles in the case#2. For the RedCap UE, does it mean that the corresponding parameter repetition number n2 and n4 indicated by the first and second FeatureCombinationPreambles in the case#2 are considered as “not configured” to the UE? RAN2#116bis Proposal 1’: [Easy] In MAC perspective, a RedCap UE uses Msg1 early identification whenever transmitting preamble for CBRA, as long as the Msg1 early identification is configured for RedCap by NW. - Agreed.

In the current MAC specification, for RedCap UE initiating RACH procedure with Msg1 repetition, the UE will select the RACH resource set indicating all the features trigging the RACH procedure, i.e. msg1-repetition-r18+redCap-r17. As a result, the UE will only select the RACH resource set corresponding to n8 since only this RACH resource is assocatied with feature combination msg1-repetition-r18+redCap-r17. Similar to our anlysis to Q3, the UE will not consider RACH reource sets corresponding to n2 and n4, and thus the RACH parameters will not be considered, i.e. not configured to the UE.

5.1.1b Selection of the set of Random Access resources for the Random Access procedure The MAC entity shall: […] 1> if neither contention-free Random Access Resources nor Random Access Resources for SI request have been provided for this Random Access procedure and one or more of the features including (e)RedCap and/or Slicing and/or SDT and/or MSG3 repetition and/or MSG1 repetition is applicable for this Random Access procedure: NOTE 2: The applicability of SDT is determined by MAC entity according to clause 5.27. The applicability of NSAG-ID is determined by upper layers when the Random Access procedure is initiated. The applicability of (e)RedCap is also determined by upper layers when Random Access procedure is initiated and it is applicable to the Random Access procedures initiated by PDCCH orders and any Random Access procedure initiated by the MAC entity. NOTE 3: SDT is not applicable for the Random Access procedure initiated by upper layers for MT-SDT. 2> if none of the sets of Random Access resources are available for any feature applicable to the current Random Access procedure (as specified in clause 5.1.1c): 3> select the set(s) of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure. 2> else if there is one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure: 3> select this set of Random Access resources for this Random Access procedure. 2> else if there are more than one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure and Msg1 repetition is applicable for this Random Access procedure: 3> select the set of Random Access resources that associated with highest repetition number among the sets of Random Access resources. 2> else (i.e. there are one or more sets of Random Access resources available that are configured with indication(s) for a subset of all features triggering this Random Access procedure): 3> select a set of Random Access resources from the available set(s) of Random Access resources based on the priority order indicated by upper layers as specified in clause 5.1.1d for this Random Access Procedure.

Proposal 5: For Q4, RAN2 confirms that in Case #2, a UE capable of msg1-repetition and RedCap will not consider the RACH resource and parameters indicated by feature combination msg1-repetition-r18 according to the current RAN2 specifications.

2.5 Q5

Q5: If the answer to Q1 is “not precluded”, in a case where a UE selects a set of PRACH resources according to featurePriorities and S5.1.1d of TS 38.321, whether or not only parameter repetition numbers indicated for the sets of PRACH resources selected by the UE are considered as “configured” to the UE while all parameter repetition numbers indicated for all the other sets of PRACH resources as “not configured”?

Similar discussion as in proposal 3 and 4, we think that only the RACH resource set and parameter corresponding to the repetition numbers indicated for the sets of PRACH resources selected by the UE are considered as “configured” to the UE while all parameter repetition numbers indicated for all the other sets of PRACH resources as “not configured”.

Proposal 6: For Q5, RAN2 confirms that only the RACH resource and parameters corresponding to repetition numbers indicated for the sets of PRACH resources selected by the UE are considered as “configured” to the UE while all parameter repetition numbers indicated for all the other sets of PRACH resources as “not configured”.

# 4	Conclusion

In this paper, there are the following proposals, and the draft LS reply is attached in Annex.

Proposal 1: For Q0, RAN2 confirms that Case#1 is not precluded, i.e. Repetition number n2 and n4 are configured in one RACH-ConfigCommon, while n8 is configured in another RACH-ConfigCommon. All these repetition numbers are associated to one feature combination (i.e. msg1-repetition-r18).

Proposal 2: For Q1, RAN2 confirms that Case#2 is not precluded, i.e. Only single RACH-ConfigCommon is configured with multiple feature combinations. Repetition number n2 and n4 are associated to one feature combination(i.e. msg1-repetition-r18), n8 is associated to another feature combination(i.e. msg1-repetition-r18+redCap-r17).

Proposal 3: For Q2, RAN2 confirms that Case#1-Rev is not precluded, i.e. Repetition number n2 and n4 are configured in one RACH-ConfigCommon, while n8 is configured in another RACH-ConfigCommon, n8 is associated to another feature combination(i.e. msg1-repetition-r18+redCap-r17).

Proposal 4: For Q3, RAN2 confirms that in Case #2, a UE capable of msg1-repetition but incapable of RedCap will not consider the RACH resource and parameters indicated by feature combination msg1-repetition-r18+redCap-r17 according to the current RAN2 specifications.

Proposal 5: For Q4, RAN2 confirms that in Case #2, a UE capable of msg1-repetition and RedCap will not consider the RACH resource and parameters indicated by feature combination msg1-repetition-r18 according to the current RAN2 specifications.

Proposal 6: For Q5, RAN2 confirms that only the RACH resource and parameters corresponding to repetition numbers indicated for the sets of PRACH resources selected by the UE are considered as “configured” to the UE while all parameter repetition numbers indicated for all the other sets of PRACH resources as “not configured”.

# 5	Reference

- R1-2409262 LS on the time period for R18 preamble repetition;

# 6	Annex

3GPP TSG RAN WG2 #128			R2-24xxxxx

Orlando, USA, Nov. 18th – 22nd, 2024

Title:	(draft) Reply LS on on the time period for R18 preamble repetition

Response to:	LS on the time period for R18 preamble repetition (R1-2409262)

Release:	Rel-18

Work Items:	NR_cov_enh2-Core

Source:	RAN WG2

To:	RAN WG1

CC:	-

Contact Person:

Name:	  TBD

E-mail Address:	TBD

Attachment:	               -

1. Overall Description:

RAN2 discussed the RAN1 LS and has the following agreement.

- For Q0, RAN2 confirms that Case#1 is not precluded, i.e. Repetition number n2 and n4 are configured in one RACH-ConfigCommon, while n8 is configured in another RACH-ConfigCommon. All these repetition numbers are associated to one feature combination (i.e. msg1-repetition-r18). - For Q1, RAN2 confirms that Case#2 is not precluded, i.e. Only single RACH-ConfigCommon is configured with multiple feature combinations. Repetition number n2 and n4 are associated to one feature combination(i.e. msg1-repetition-r18), n8 is associated to another feature combination(i.e. msg1-repetition-r18+redCap-r17). - For Q2, RAN2 confirms that Case#1-Rev is not precluded, i.e. Repetition number n2 and n4 are configured in one RACH-ConfigCommon, while n8 is configured in another RACH-ConfigCommon, n8 is associated to another feature combination(i.e. msg1-repetition-r18+redCap-r17). - For Q3, RAN2 confirms that in Case #2, a UE capable of msg1-repetition but incapable of RedCap will not consider the RACH resource and parameters indicated by feature combination msg1-repetition-r18+redCap-r17 according to the current RAN2 specifications. - For Q4, RAN2 confirms that in Case #2, a UE capable of msg1-repetition and RedCap will not consider the RACH resource and parameters indicated by feature combination msg1-repetition-r18 according to the current RAN2 specifications. - For Q5, RAN2 confirms that only the RACH resource and parameters corresponding to repetition numbers indicated for the sets of PRACH resources selected by the UE are considered as “configured” to the UE while all parameter repetition numbers indicated for all the other sets of PRACH resources as “not configured”.

2. Actions:

To RAN WG1

ACTION: RAN2 respectfully asks RAN1 to consider above information.

3. Date of Next RAN WG2 Meetings:

TSG-RAN WG1 Meeting #129				February 17 to 21, 2024		Athens, Greece

TSG-RAN WG1 Meeting #129-bis			April 07 to 11, 2024		   China, CN