3GPP TSG-RAN WG2 Meeting #128	R2- 2410631

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.13.3

Source:	Nokia

Title:	On approach 1

WID/SID:	NR_SL_relay_multihop - Release 19

Document for:	Discussion and Decision

# 1	Introduction

During the e-mail discussion [402][Relay] on multi-hop relay control plane, two approaches have been discussed:

- Approach 1: The network needs to directly control each of the intermediate relay UEs via Uu RRC.

- Approach 2: Only the last relay UE requires control by the network via Uu RRC.

In this contribution, we continue the discussion on approach 1.

# 2	Discussion

For the approach 1, the basic assumption seems to be that all the relay UEs (including first/ intermediate/ last) should be in RRC_CONNECTED state to function as relay UEs. Thus, the procedural steps outlined for the approach 1 in [406] indicates that each relay UE initiates its own RRC Connection establishment procedure if it is not already in RRC_CONNECTED state upon receiving a request from its child UE, e.g., SidelinkUEInformationNR message. This is based on the legacy U2N relay procedure, which involves only a single relay UE, i.e., 1-hop relay. However, with multi-hop configuration like 2 or 3 hops, the sequential initiation of RRC connection establishment procedures at each relay UE may not be advantageous, as it increases the overall latency from the Remote UE perspective.

Observation: Initiation of RRC connection establishment procedure upon reception of SidelinkUEInformationNR may increase the total latency in making the end-to-end connection from the Remote UE to the gNB.

During the email discussion, the latency introduced by the cascaded establishment of RRC connection at each relay UE was identified as a drawback of approach 1. To address this issue, one option could be that the relay UE initiates the RRC connection establishment prior to receiving a message from its child UE via the PC5 Relay RLC channel. This would imply that the relay UE may need to initiate the RRC connection establishment based on its intent to function as a relay UE, even if there is no traffic for itself or for relay purposes at that moment.

Proposal: RAN2 discuss whether to reduce the total latency in establishing the RRC Connection of each relay UE.

# 3	Conclusion

In this contribution, we address one issue identified during the email discussion [406], and provide the following observation/proposal:

Observation: Initiation of RRC connection establishment procedure upon reception of SidelinkUEInformationNR may increase the total latency in making the end-to-end connection from the Remote UE to the gNB.

Proposal: RAN2 discuss whether to reduce the total latency in establishing the RRC Connection of each relay UE.