3GPP TSG-RAN WG2 Meeting #128                                                               R2-2410642

Orlando, USA, November 18th– 22nd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0904 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RACH-less HO in NR-NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Samsung, Nokia, Nokia Shanghai Bell, THALES, Huawei, HiSilicon, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN2#121bis, RAN2 agreed to support the RACH-less handover for inter-gNB and intra-gNB cases in NTN discussion. 1. NTN RACH-less HO is supported for Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB; 2. NTN RACH-less HO can be supported for intra-satellite handover with different feeder links, i.e., with gateway/gNB switch, inter-satellite handover with gateway/gNB switch, and inter-satellite handover with same gateway/gNB.. However, according to the latest stage 2 spec, only the intra-gNB RACH-less handover case is supported regardless it is in TN or NTN scenario. The RAN2 NTN agreements are not well captured. We should capture all the applicable cases as agreed for NTN in the stage-2 spec |
| Summary of change: |  | Capture that RACH-less handover can be supported for intra/inter-satellite handover with same gateway/gNB and intra/inter-satellite handover with gateway/gNB switch and remove the existing intra-gNB restriction. Impact analysis Impacted 5G architecture options: Standalone Inter-operability: 1. if the network supports the change and the UE does not, there is no inter-operability issue foreseen. 2. if the UE supports the change and the network does not, there is no inter-operability issue foreseen. Impacted functionality: RACH-less handover in NTN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In NTN, RACH-less HO applicable cases are not complete in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R2-2408013 |  |  |  |  |  |  |  |  |

Start of change

9.2.3.6	RACH-less handover

The RACH-less handover procedure applies the following functionality:

-	The UE uses the same timing advance value at the target cell as in the source cell or timing advance value of 0.

-	The handover command for the UE may contain a beam identifier for the beam to be used by the UE at the target cell. The beam may be determined based on a UE measurement report and/or left up to gNB implementation, e.g., using the target cell's knowledge about the beam(s) used by the UE at the co-located source cell.

-	The handover command may include a configured UL grant. UE can fallback to RACH when there is no valid configured uplink grant. Alternatively, an UL grant is dynamically signalled by the target cell.

-	The UE transmits the RRCReconfigurationComplete message using the configured or dynamically signalled UL grant. Successful UL data reception on the target cell terminates the RACH-less handover execution.

In NTN, RACH-less handover can be supported for intra/inter-satellite handover with same gateway/gNB and intra/inter-satellite handover with gateway/gNB switch.

End of change