3GPP TSG-RAN WG2 Meeting #128	R2-2410657

Orlando, USA, 18 – 22 November, 2024

Title: 	Discussion on support of the Rel-18 leftovers

Source: 	Huawei, HiSilicon

Agenda item:	8.10.5

Document for:	Discussion/Decision

1   Introduction

In this paper, we will discuss the leftover issues from Rel-18 SON/MDT.

2   Discussion

2.1 RACH optimization for SDT

The RAN2#127bis meeting has achieved the following agreements:

Agreements - For failed SDT case, UE includes the DL RSRP and UL data volume at the time of SDT evaluation in SON report. For successful SDT procedure, the UE does not log. - RAN2 understands for SON/MDT R19 the SDT enhancements only relate to the RA-SDT procedure. This does not rule out the case when the UE falls back from RA-SDT.

RA report for SDT has been standardized in Rel-18, which is one means to detect the SDT related RACH resource issues. The Rel-19 WID allows the further development of RACH optimization for SDT.

RAN2 has agreed to introduce the downlink RSRP and buffered data volume at the time of deciding whether to perform SDT or not in order to optimize the condition parameters for triggering the SDT. Some company were also interested in the RSRP and data volume at the time of SDT failure. Between the time of SDT initiation and SDT failure, the RSRP and buffered data volume may change a lot, which is dynamic random variation. In our understanding, the RSRP value and data volume when SDT failure happens is not helpful for network to adjust the condition parameter for triggering SDT or not.

Proposal 1: It is proposed RAN2 to investigate the necessity of including RSRP threshold and data volume threshold when SDT failure happens.

An FFS from the RAN2#127 meeting is the down selection of the failure cause for SDT operation failure. Per current specification TS 38.331, it could be considered as SDT failure under the following circumstances:

- T319a expires; or

- random access problem indication received from MCG MAC while SDT procedure is ongoing; or

- indication received from MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing; or

- indication from the lower layers that the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing; or

- integrity check failure indication received from lower layers while SDT procedure is ongoing; or

From our point of view, it is important to detect the random access issue, the RLC re-transmission issue and T319a setting issue, because we think they should be the main reasons for leading SDT failures. Other failure type could be de-prioritized, as they are relatively less likely to happen. Therefore, it is suggested to indicate the failure causes for random access, RLC re-transmission and T319a in RA report.

Proposal 2: Enhance the following failure cause to RA report: random access, RLC re-transmission and T319a.

Furthermore, since RAN2 does not rule out the case when UE fall backs from the CG-SDT to RA-SDT, we analyse the necessity of other SDT related information as below:

- CG-SDT related information if the UE is configured with CG-SDT but fails to initiate the CG-SDT

- indication regarding that the UE failed to initiate RA-SDT or CG-SDT

When UL data arrives in the UE in RRC inactive, the UE could first check whether the CG-SDT is configured and thus the conditions for CG-SDT are fulfilled, and then check if the RA-SDT is fulfilled if CG-SDT is not configured or the conditions for CG-SDT are not fulfilled. In our point of view, the network may need to know why CG-SDT is not initiated when the CG is reserved and configured for the UE. The UE also compares the DL RSRP and buffered data volume to CG-SDT condition parameter to derive the fulfilment status. Regarding the fallback case from CG-SDT to RA-SDT, it may help the network to optimize the CG-SDT related parameters and improve the utilization probability of CG resources. Since the DL RSRP and buffered data are already agreed to be reported by UE and the UE-specific CG-SDT config is known from the UE context, we understand that an indication of whether CG-SDT is configured in RA report, when RA-SDT is initiated, enables the network to know that it fails to initiate CG-SDT.

Proposal 3: The UE indicates in the RA-Report if the CG-SDT initiation was evaluated.

2.2 MRO for MR-DC SCG failure

Rel-17 SON/MDT WI developed the solution for MRO for NR-DC SCG failures according to TS 38.331 specification [2]. In Rel-17, SON/MDT WI considered primarily two SCG failure scenarios as follows:

- SCG RLF, and

- Legacy PSCell change or addition failure.

In Rel-19, we further discuss the MRO for SCG failure in other DC options.

EN-DC

For EN-DC scenario, the UE report the NR SCG failure information to LTE MN node, and then the LTM MN node may further forward the NR SCG failure information to NR SN node.

At RAN2#127bis meeting, the following agreement was made:

Agreements - Add reporting of the following parameters for SCG failure report in EN-DC scenario: ⁻ For failedPSCellId and previousPSCellId: frequency and the PCI of the PSCell; ⁻ For timeSCGFailure: value range 0-1023; ⁻ For failureType: Reuse the legacy field. - perRA-InfoList

In addition, RAN2#127bis endorsed the following two CRs for Correction on IE perRAInfoList for SCGFailureInformation: R2-2409298 [1] and R2-2409299 [2]. They are about RA info correction for NR-DC scenario. We could consider to apply this for EN-DC scenario, e.g. if beam failure recovery happens on SCG, the UE could indicate perRAInfoList-v1660 related to the CSI-RS resource associating with NZP-CSI-RS-Resource. As below, we copied the relevant information elements in TS 38.331.

PerRAInfoList-r16 ::= SEQUENCE (SIZE (1..200)) OF PerRAInfo-r16 PerRAInfoList-v1660 ::= SEQUENCE (SIZE (1..200)) OF PerRACSI-RSInfo-v1660 PerRAInfo-r16 ::= CHOICE { perRASSBInfoList-r16 PerRASSBInfo-r16, perRACSI-RSInfoList-r16 PerRACSI-RSInfo-r16 } PerRACSI-RSInfo-r16 ::= SEQUENCE { csi-RS-Index-r16 CSI-RS-Index, numberOfPreamblesSentOnCSI-RS-r16 INTEGER (1..200) } CSI-RS-Index ::= INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1) maxNrofCSI-RS-ResourcesRRM-1 INTEGER ::= 95 -- Maximum number of CSI-RS resources per cell for an RRM measurement object -- minus 1. PerRACSI-RSInfo-v1660 ::= SEQUENCE { csi-RS-Index-v1660 INTEGER (1..96) OPTIONAL }

Proposal 4: Add the perRAInfoList-v1660 as part of perRA-InfoList for SCG failure report in EN-DC scenario.

NE-DC

For NE-DC scenario, the UE report the LTE SCG failure information to NR MN node, and then the NR MN node may further forward the LTE SCG failure information to LTE SN node. Therefore, the SCGFailureInformationEUTRA message in TS 38.331 needs to be enhanced accordingly.

NGEN-DC

Similar as EN-DC case, the SCGFailureInformationNR message defined in TS 36.331 needs to be enhanced accordingly, as above discussed.

Proposal 5: The enhancement of SCGFailureInformationNR message for EN-DC applies also to NGEN-DC.

Proposal 6: For NE-DC, the enhancement to SCGFailureInformationEUTRA message in TS 38.331 is introduced, i.e., failedPSCellId, previousPSCellId, timeSCGFailure and perRAInfoList.

3   Conclusion

In this paper, we provide the following proposals:

RACH optimization for SDT:

Proposal 1: It is proposed RAN2 to investigate the necessity of including RSRP threshold and data volume threshold when SDT failure happens.

Proposal 2: Enhance the following failure cause to RA report: random access, RLC re-transmission and T319a.

Proposal 3: The UE indicates in the RA-Report if the CG-SDT initiation was evaluated.

MRO for MR-DC SCG failure:

Proposal 4: Add the perRAInfoList-v1660 as part of perRA-InfoList for SCG failure report in EN-DC scenario.

Proposal 5: The enhancement of SCGFailureInformationNR message for EN-DC applies also to NGEN-DC.

Proposal 6: For NE-DC, the enhancement to SCGFailureInformationEUTRA message in TS 38.331 is introduced, i.e., failedPSCellId, previousPSCellId, timeSCGFailure and perRAInfoList.

4   Reference

[1] R2-2409298, Correction on IE perRAInfoList for SCGFailureInformation (Rel-17), Huawei, HiSilicon (In principle agreed CR)

[2] R2-2409299, Correction on IE perRAInfoList for SCGFailureInformation (Rel-18), Huawei, HiSilicon (In principle agreed CR)