3GPP TSG RAN WG3 Meeting #128	 R2-2410662

Orlando, USA, 18 - 22 November, 2024

Agenda Item:	8.8.4

Source:    TCL.

Title:          Further Discussion on Support of MBS Broadcasting over NTN

Document for:	Discussion and Decision

# Introduction

Multicast/broadcast service (MBS) feature provides an important add-value for NR NTN system, leveraging the large coverage area of NTN compared to TN for MBS deployment. During 3GPP RAN#102, a new Rel-19 satellite access related WID was approved [1] targeting enabling MBS features over NTN in the 5G specifications. The major focus this WID, is to enhancements NTN/MBS to address the scenario when intended service area of MBS broadcast is smaller than the coverage of an NTN cell or a satellite beam. The following objectives were agreed as part of] this WID:

- Specify signaling of the intended service area of a broadcast service (e.g., MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signalling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signalling between CN and NG-RAN. [RAN3]

During last meetings the following agreements has been made on this regard:

Agreements:

- The intended service area can cover the area of more than one NTN cells (or portions thereof)

- Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area)

- At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered):

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

- RAN2 understands that the expected UE behavior, is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions.

- For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

- For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

This report discusses the enhancements for supporting of MBS broadcasting over NTN focusing on the intended MBS broadcasting area definition, the signalling design for controlling MBS broadcasting over NTN and the service continuity issues.

# Discussion

## 2.1 Definition of the MBS intended area on the ground

During last meetings RAN2 has made the following agreement regarding the intended area for MBS service broadcasting over NTN access:

Agreements:

- The intended service area can cover the area of more than one NTN cells (or portions thereof)

- At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered):

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

- The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

And in RAN3 the following agreements have been achieved on this regard

Agreements - RAN3 starts work on MBS broadcast service. - Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN.

Based on our understanding of the above RAN2 and RAN2 agreements

The geolocation information such as TN circles or other geographical area information like a polygon shape that are used to describe the geographical area formats in RAN2 can correspond to /associate with to RAN3 Mapped Cell ID(s). According to NTN design in TS 38.300, it is stated that “a specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate different geographical areas (e.g. overlapping and/or with different dimensions). From this statement, it can be deduced that a mapping between the geolocations on ground and a network identities are used to describe the intended MBS service area for a UE Table 1.

Table 1: mapping between geolocations on ground and a network identities

| Actual geolocations on ground | Network identities |
| --- | --- |
| geolocation 1, geolocation 2 | mapped cell id 1 |
| geolocation 3 | mapped cell id 2 |
| geolocation 4 | mapped cell id 3 |
| ………………………………. | …………………. |

In case of MBS broadcasting over NTN access, the satellite coverage on the ground level may span to cover a large geographical area whereas the intended area for MBS broadcasting is only a small region within the satellite coverage.  For example in case of GEO/MEO satellite,  the coverage may span to cover a set of multiple countries or region whereas, the intention could be to limit the MBS broadcasting to only subset of these countries or regions or to restrict the broadcasting not to go beyond a boundary of a specific country or region (Figure 1). Given the fact that, any specific country or region on the ground is associated with TN network identities such as TN PLMNs within a country and/or TN tracking areas within a region which are known for the UE (Figure 1). It could be more appropriate to discuss using such information along with the agreed geographical area formats to describe the intended service area for the UE on the ground Table 2. Therefore, we have the following observation.

Observation : In case of GEO/MEO satellite,  the coverage may span to cover a set of multiple countries or region whereas, the intention could be to limit the MBS broadcasting to only a subsets of these countries or regions or to restrict it not to go beyond a boundary of a specific country or region. Given the fact that, any specific country or region in the ground is technically associated with TN network identities such as TN PLMN ids within a country and/or TN tracking areas ids within a region which are known for the UE, such information could be more appropriate for describing the intended service area for the UE on the ground for GEO/MEO satellite.

Figure 1: MBS intended service area

Table 2: mapping between geolocations on ground and a network identities

| Actual geolocations on ground | Network identities |
| --- | --- |
| geolocation 1, geolocation 2 | PLMN/Track area/ mapped cell id 1 |
| geolocation 3 | PLMN/Track area/ mapped cell id 2 |
| geolocation 4 | PLMN/Track area/ mapped cell id 3 |
| ………………………………. | …………………. |

And we propose:

  - Proposal 1: The geographical format for the intended MBS service area could be also modelled by a TN tracking area and/or an area covered by TN PLMN identity.

## 2.2 Contents & Signalling Format of intended service area

Regarding expected UE behavior based on MBS intended area, the following agreements has been made during latest meeting:

Agreements:

- RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

- For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

- For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3).

- For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

The above agreements are assuming that a UE can be aware of the geolocation boundary of the intended service area, so that it can initiate the actions of release or establishment of an MBS broadcast session based on that. In practical conditions, a UE initiates a session release or establishment actions based on its knowledge of network identities (e.g., PLMN, tracking areas and/or mapped cell identities) rather than a knowledge of a location information. For example during connected or inactive state mobility, the locations are defined for UE as set of network identities (e.g. cells or tracking areas), and based on such identities the UE may perform the actions of release or an establishment of a radio bearer. Without the knowledge of the identities that define the boundary of intended area for a UE, it is hard for a UE to know whether it is within or outside the intended service area so that to (re)acquire mcch or initiate the broadcast MRB establishment procedure when it enter or not (re)acquire mcch and/or MRB release when it leaves the intended area. In addition, if no association between the geographical area formats describing the entire region under the coverage of the satellite and the network identities (e.g., PLMN IDs, tracking areas or mapped cell ids) is provided to UE in advance by the network, it would be unclear how the UE will identify whether a specific region within the satellite coverage is an intended service area or not.

On the other hand, it was agreed during last meeting that MBS intended service area IDs can be included into MCCH.  While for the mapping between broadcast service session and intended service area, it was argued whether if it shall also be included in MCCH or provided in a new SIB or in an existing SIB. TCL strongly supports the option of including the mapping between broadcast service session and intended service area in a SIB. Regarding this option, some companies have showed a concern on whether the size of the MBS SIB can fit for such mapping information if an existing SIB is used. Others argued that there will be a signaling duplication and delay because the UE may need to decode more than one SIB, one for intended service area IDs another for the mapping, if we consider introducing a new SIB. In our view, these concerns are addressable, for example by reusing NTN SIB, such as SIB 19; for providing the mapping between broadcast service session and intended service area. By this way, the signaling duplication can be avoided because technically the UE have to acquire NTN SIB if MBS is configured over satellite access. If the mapping information is provided there along with other NTN configuring, UE do not need to acquire two SIBs. This give a free option on whether to include the intended service ID in MCCH or MBS SIB. Additionally, NTN SIB has enough capacity to fit for such mapping information and has impact on legacy UEs. Furthermore, this option is more favorable in terms of skipping MCCH re-acquisition in the sense that, an association between TN network identities (e.g., tracking areas and/or mapped cell identities) and the geographical area formats can be provided along with the mapping information to assist UE on skipping re-acquisition of MCCH when UE is not within intended service area of any interested broadcast service.

Based on the above discussion we made the following observation. Observation 2: UE initiate action based on network identity, without the knowledge of the identities defining the boundary of intended area for a UE, it is hard for a UE to know whether it is within or outside the intended service area so that to (re)acquire mcch or initiate the broadcast MRB establishment procedure when it enter or not (re)acquire mcch and/or MRB release when it leaves the intended area

Observation 3: Without a prior knowledge of the association between the geographical area formats describing the entire region under the coverage of the satellite (including both the intended and unintended areas for MBS broadcasting on the ground) and the network identities known by UE, it would be unclear how the UE will identify whether a specific region within the satellite coverage is an intended service area or not (unintended service area from MBS).

Observation 4: The concerns such as whether the size of SIB can fit for the mapping information, signaling duplication and delay because UE need to decode two SIBs are all addressable if we consider providing the mapping between broadcast service session and intended service area in NTN SIB.

Observation 5: The option of providing the mapping within the SIB option more favorable in terms of skipping MCCH re-acquisition. In the sense that, an association between TN network identities (e.g., tracking areas and/or mapped cell identities) and the geographical area formats can be provided along with the mapping information to assist UE on skipping re-acquisition of MCCH when UE is not within intended service area of any interested broadcast service.

Based on these observations, we propose the following proposals

  - Proposal 2: To better describe the MBS intended area for the UE; an association between network identities (e.g., PLMNs, tracking areas and/or mapped cell identities) and the geographical area formats used to describe the intended region on the ground is needed to be signalled to the UE.

  - Proposal 3: An association between the geographical area formats describing the entire region under the coverage of the satellite (including both the intended and unintended areas for MBS broadcasting on the ground) and the network identities shall be also signaled  to UE, to help UE to identifying whether it is within an intended service area or not .

  - Proposal 4: Once UE know the mapping between the network identities and geographical formats describing the intended/unintended area for MBS broadcasting under the satellite coverage, the intended service area can signaled as a subset of these network identities to UE.

  - Proposal 5: The mapping between broadcast service session and intended service area are provided to UE in a SIB.

  - Proposal 6: If proposal 5 is agreeable, discuss reusing the existing NTN SIBs such as SIB 19 for providing the mapping information between broadcast service session and intended service area.

  - Proposal 7: Discuss whether additional information such TN networks identities used to describe the intended area on the ground can be also included along within mapping information the NTN SIB to assist UE to skip re-acquisition of MCCH when UE is not within intended service area.

## 2.2 Service Continuity Outside of intended Area

During the last meeting there was a discussion on enhancing MBS service continuity considering intended service area. In our understating, for both moving earth and quasi-fixed-earth scenarios the definition of the intended area on the ground will never change, from UE point of view. However, according to the definition of geographical formats used for describing the MBS intended service area on the ground, which could be linked with cell identity (e.g., mapped cells ids). In case of satellite movement, the definition of the cells defining the intended service area could be changed. Thus,  UE may need reselect to a new cell to continue MBS reception. Therefore, in our opinion, in case of in case of satellite movement, neighbouring cell information can be provided for an MBS broadcast service session to assist on addressing a cell reselection rather than a service continuity issue .

  - Proposal 8: In case of satellite movement the neighbouring cell ids can be provided for an MBS broadcast service session to assist on to assist on addressing a cell reselection rather than a service continuity issue.

# Conclusion

In this contribution, we have discussed the enhancements for supporting MBS broadcasting over NTN focusing on the intended MBS broadcasting area, the signalling design for controlling MBS broadcasting over NTN and the service continuity issues. Based on the discussion we made following observations:

Observation 2: UE initiate action based on network identity, without the knowledge of the identities defining the boundary of intended area for a UE, it is hard for a UE to know whether it is within or outside the intended service area so that to (re)acquire mcch or initiate the broadcast MRB establishment procedure when it enter or not (re)acquire mcch and/or MRB release when it leaves the intended area

Observation 3: Without a prior knowledge of the association between the geographical area formats describing the entire region under the coverage of the satellite (including both the intended and unintended areas for MBS broadcasting on the ground) and the network identities known by UE, it would be unclear how the UE will identify whether a specific region within the satellite coverage is an intended service area or not (unintended service area from MBS).

Observation 4: The concerns such as whether the size of SIB can fit for the mapping information, signaling duplication and delay because UE need to decode two SIBs are all addressable if we consider providing the mapping between broadcast service session and intended service area in NTN SIB.

Observation 5: The option of providing the mapping within the SIB option more favorable in terms of skipping MCCH re-acquisition. In the sense that, an association between TN network identities (e.g., tracking areas and/or mapped cell identities) and the geographical area formats can be provided along with the mapping information to assist UE on skipping re-acquisition of MCCH when UE is not within intended service area of any interested broadcast service.

And propose the following proposals

  - Proposal 1: The geographical format for the intended MBS service area could be also modelled by a TN tracking area and/or an area covered by TN PLMN identity.

  - Proposal 2: To better describe the MBS intended area for the UE; an association between network identities (e.g., PLMNs, tracking areas and/or mapped cell identities) and the geographical area formats used to describe the intended region on the ground is needed to be signalled to the UE.

  - Proposal 3: An association between the geographical area formats describing the entire region under the coverage of the satellite (including both the intended and unintended areas for MBS broadcasting on the ground) and the network identities shall be also signaled  to UE, to help UE to identifying whether it is within an intended service area or not .

  - Proposal 4: Once UE know the mapping between the network identities and geographical formats describing the intended/unintended area for MBS broadcasting under the satellite coverage, the intended service area can signaled as a subset of these network identities to UE.

  - Proposal 5: The mapping between broadcast service session and intended service area are provided to UE in a SIB.

  - Proposal 6: If proposal 5 is agreeable, discuss reusing the existing NTN SIBs such as SIB 19 for providing the mapping information between broadcast service session and intended service area.

  - Proposal 7: Discuss whether additional information such TN networks identities used to describe the intended area on the ground can be also included along within mapping information the NTN SIB to assist UE to skip re-acquisition of MCCH when UE is not within intended service area.

  - Proposal 8: In case of satellite movement the neighbouring cell ids can be provided for an MBS broadcast service session to assist on to assist on addressing a cell reselection rather than a service continuity issue.

# References

- RP-240775, “Revised WID: Rel-19 WI Non-Terrestrial Networks (NTN) for NR Phase 3”, THALES, CATT, RAN#103, Netherlands, March 18-22, 2024.