3GPP TSG-RAN WG2 Meeting #128		R2-2410663

Orlando, US, Nov 18th – 22th, 2024

Source:	Baicells

Title:		Discussion on LTM measurement reporting configuration

Agenda Item:		8.6.3

Document for:	Discussion and Decision

- Introduction

As is defined in the WID for R19 LTM mobility enhancements at RAN#102 meeting, the event triggered L1 measurement reporting is needed to be supported in LTM.

On the objective of LTM event triggered L1 measurement enhancement in R19 mobility WI, RAN2 made the following progress in the RAN2#125bis[1], RAN2#126[2], RAN2#127[3] and RAN2#127bis[3]:

RAN2#125bis Agreements on measurements: 1. L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

RAN2#126 Agreements on measurements: 1. Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. 2. For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. 3. Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. 4. Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighboring cell for event LTM3 and event LTM5. 5. RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. 6. For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

RAN2#127 Agreements on measurements: 1. Event LTM1 is not defined. 2. Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. 3. Any beam in candidate RS configuration can be used for LTM event evaluation. 4. Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. 5. R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. 6. MAC layer handles the event evaluation and measurement report triggering. 7. Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

RAN2#127bis Agreements on L1 event triggered MR: 5. MR can be sent when the leaving condition is met, based on NW configuration. 6. Event triggered periodic MR can be supported, based on NW configuration. 7. For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3. 8. For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed. 9. For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed. 10. The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1. 11. TTT operates only based on a timer (like TTT used in L3 event triggered MR). 12. Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). 13. Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW. 14. Additional information included in MR MAC CE: - The information and quantity of current beam, based on NW configuration. 11. The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. 12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

In this contribution, we will discuss the remaining open issues and further details of event triggered L1 measurement reporting configuration, including:

-	How to configure the event triggered L1 measurement reporting for LTM.

- Discussion

  - Event triggered L1 measurement reporting configuration

We understand that one measurement resource may be configured with multiple events e.g. Event LTM2, Event LTM3 and Event LTM5 are configured to the same measurement resource. If we reuse LTM-CSI-ReportConfig for the event triggered measurement report configuration, there are two options to configure the measurement report configurations. The two options are as follow:

Option1: One report configuration (LTM-CSI-ReportConfig) includes one event configuration.

Option2: One report configuration (LTM-CSI-ReportConfig) includes multiple event configurations.

NW needs to configure the LTM-report-quantity, LTM-report-content and LMT-CSI-ResourceConfigID for each event. For the above two options, Option1 needs multiple report configurations to configure multiple events for one measurement resource. If the LTM-report-quantity, LTM-report-content and LMT-CSI-ResourceConfigID configurations are the same for each event, then these IEs are transmitted multiple times in multiple report configurations causing signaling overhead. Even if the configuration of LTM-report-quantity and LTM-report-content can be different for different events, at least the LMT-CSI-ResourceConfigID is retransmitted in different events for the same measurement resource. Option2 only needs one measurement report configuration, and the above IEs may need to be configured once in the report configuration for one measurement resource. The following table compares the two options.

Table I   The comparison between the two options

|  | Option1：Multiple report configurations for multiple events (each configuration for one event) | Option2：One report configuration containing multiple events |
| --- | --- | --- |
| Signaling overhead for Measurement report configuration | The following things are configured by gNB to UE in the LTM-CSI-ReportConfig: LMT-CSI-ReportConfigId1, LMT-CSI-ResourceConfigId, LTM-EVENT::={Event LTM2}, LTM-report-quantity ::= SEQUENCE { RSRP, RSRQ, SINR, }, LTM-report-content ::= SEQUENCE { nrOfReportedCells ENUMERATED {n1,n2,n3,n4}, nrOfReportedRS-PerCell ENUMERATED {n1,n2,n3,n4}, spCellInclusion ENUMERATED {true} OPTIONAL -- Need R }, LMT-CSI-ReportConfigId2, LMT-CSI-ResourceConfigId, LTM-EVENT::={LTM3}, LTM-report-quantity ::= SEQUENCE { RSRP, RSRQ, SINR, }, LTM-report-content ::= SEQUENCE { nrOfReportedCells ENUMERATED {n1,n2,n3,n4}, nrOfReportedRS-PerCell ENUMERATED {n1,n2,n3,n4}, spCellInclusion ENUMERATED {true} OPTIONAL -- Need R }, | The following things are configured by gNB to UE in the LTM-CSI-ReportConfig: LMT-CSI-ReportConfigId1, LMT-CSI-ResourceConfigId, LTM-EVENT::= SEQUENCE {LTM2, LTM3}, LTM-report-quantity ::= SEQUENCE { RSRP , RSRQ , SINR , }, LTM-report-content ::= SEQUENCE { nrOfReportedCells ENUMERATED {n1,n2,n3,n4}, nrOfReportedRS-PerCell ENUMERATED {n1,n2,n3,n4}, spCellInclusion ENUMERATED {true} OPTIONAL -- Need R }, |
|  | The following things are configured by gNB to UE in the LTM-CSI-ReportConfig: LMT-CSI-ReportConfigId1, LMT-CSI-ResourceConfigId, LTM-EVENT::={LTM2}, LTM-report-quantity ::= SEQUENCE { RSRP, RSRQ, SINR, }, LTM-report-content ::= SEQUENCE { nrOfReportedCells ENUMERATED {n1,n2,n3,n4}, nrOfReportedRS-PerCell ENUMERATED {n1,n2,n3,n4}, spCellInclusion ENUMERATED {true} OPTIONAL -- Need R }, LMT-CSI-ReportConfigId2, LMT-CSI-ResourceConfigId, LTM-EVENT::={LTM3}, LTM-report-quantity ::= SEQUENCE { RSRP, RSRQ, }, LTM-report-content ::= SEQUENCE { nrOfReportedCells ENUMERATED { n1,n2,n3, }, nrOfReportedRS-PerCell ENUMERATED {n1,n2,n3, }, spCellInclusion ENUMERATED {false} OPTIONAL -- Need R }, | The following things are configured by gNB to UE in the LTM-CSI-ReportConfig: LMT-CSI-ReportConfigId, LMT-CSI-ResourceConfigId, LTM-EVENT::= SEQUENCE {LTM2, LTM-report-quantity ::= SEQUENCE { RSRP , RSRQ , SINR , }, LTM-report-content ::= SEQUENCE { nrOfReportedCells ENUMERATED {n1,n2,n3,n4}, nrOfReportedRS-PerCell ENUMERATED {n1,n2,n3,n4}, spCellInclusion ENUMERATED {true} OPTIONAL -- Need R } }, LTM-EVENT ::= SEQUENCE {LTM3, LTM-report-quantity ::= SEQUENCE { RSRP , RSRQ , }, LTM-report-content ::= SEQUENCE { nrOfReportedCells ENUMERATED {n1,n2,n3,}, nrOfReportedRS-PerCell ENUMERATED {n1,n2,n3,}, spCellInclusion ENUMERATED {false} OPTIONAL -- Need R } }, |

Observation 1: When multiple events are configured for one measurement resource simultaneously, there are two options to indicate the triggered event, corresponding to two measurement report configurations.

Option1: One report configuration (LTM-CSI-ReportConfig) includes one event configuration.

Option2: One report configuration (LTM-CSI-ReportConfig) includes multiple event configurations.

Observation 2: The Option1 retransmits at least the LMT-CSI-ResourceConfigID for multiple events for one measurement resource. The LTM-report-quantity and LTM-report-content are retransmitted when they are configured same for the multiple events for the same measurement resource.

Observation 3: The Option2 uses one report configuration and at least the LMT-CSI-ResourceConfigID only need to be configured once in the report configuration for one measurement resource.

From the above observations, Option2 reduces the signaling overhead in the measurement reporting configuration from gNB to UE and we prefer Option2.

Therefore, we propose:

Proposal 1: One LTM CSI reporting configuration contains multiple events for the LTM CSI reporting.

- Conclusion

In this contribution, we discussed the issues related to LTM measurement event evaluation. We have the following Observations and Proposals:

Observation 1: When multiple events are configured for one measurement resource simultaneously, there are two options to indicate the triggered event, corresponding to two measurement report configurations.

Option1: One report configuration (LTM-CSI-ReportConfig) includes one event configuration.

Option2: One report configuration (LTM-CSI-ReportConfig) includes multiple event configurations.

Observation 2: The Option1 retransmits at least the LMT-CSI-ResourceConfigID for multiple events for one measurement resource. The LTM-report-quantity and LTM-report-content are retransmitted when they are configured same for the multiple events for the same measurement resource.

Observation 3: The Option2 uses one report configuration and at least the LMT-CSI-ResourceConfigID only need to be configured once in the report configuration for one measurement resource.

Proposal 1: One LTM CSI reporting configuration contains multiple events for the LTM CSI reporting.

- References

- RAN2 #125bis chairman notes;

- RAN2 #126 chairman notes;

- RAN2 #127 chairman notes;

- RAN2 #127bis chairman notes;