3GPP TSG-RAN WG2 Meeting #128	 R2-2410708

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	8.6.3

Source:	KDDI Corporation

Title:	Discussion on L1 event triggered measurement reporting

WID:       	NR_Mob_Ph4-Core

Tdoc Type:	Discussion

Document for: 	Discussion and decision

- Introduction

In RAN#104 meeting, a revised Rel-19 WID of ‘NR mobility enhancements Phase 4’ was approved [1].

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

In this contribution, we will discuss and share our views on event triggered L1 measurement reporting for “Specify necessary components to support event triggered L1 measurement reporting” described in the second bullet of the WID objective above.

- Discussion

TTT operation per beam or per cell

In RAN2#127bis meeting, proposals were discussed but not reached to make decision below [2].

7. TTT operation per beam or per cell (for a candidate cell)? - Option1: Per beam (P1 in R2-2408260, MediaTek) - Option2: Per cell (P2 in R2-2408525, ZTE) We will make decision next meeting.

TTT per cell can trigger L1 MR earlier than TTT per beam. Also for LTM switch purpose, NW doesn’t care what beam. TTT per CSI-RS resource set can be considered.

We analysed pros and cons of two options below.

| Option | Pros | Cons |
| --- | --- | --- |
| Option1: Per beam | - TTT per beam allows for finer control - Optimal settings can be made according to the characteristics of each beam - Provides optimal response for specific beams during LTM | - Management can become complex if there are many beams - Potential increased load on UE |
| Option2: Per cell | - Earlier L1 MR triggering - TTT per cell results in simpler and more efficient management - Consistent operation across the entire network - Allows for a simpler configuration even when using multiple beams | - Relies on overall cell characteristics, so individual beam characteristics may not be considered - Response to environmental changes may be slower |

Proposal 1: RAN2 should support option2. For the LTM event evaluation, TTT is operated per cell, i.e., not per beam.

Measurement RS type alignment

In RAN2#127bis meeting, proposals were discussed but not reached to make decision below [2].

11. Measurement RS type alignment (P7 in R2-2409096, Samsung, P1 in R2-2408525, ZTE) - Option 1: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam when UE evaluate the L1 event measurement. - Option 2: Only one type of RS is configured for the new beam as a NW restriction, and UE perform the L1 measurement when RS type of the current beam is same with the new beam for event L1 measurement. - Option 3: NW provides the offset which can be applied one type of RS to evaluate the different type of RS. For example, If the current beam is SSB and the measured beam for the candidate cell is CSI-RS, the UE applies the configured the offset to the measured CSI-RS value and then compare it with the current beam quality i.e. SSB measurement. - Option 4: Rely on network configuration, i.e. same RS type is configured. - Option 5 (new option proposed by CATT): Which type to be used in measurement will be configured by NW. Then the UE only consider the configured RS type in the measurement. - Option6 (copied from P1, ZTE): The RS for current beam of serving cell is same as or associated with QCL RS provided in the indicated TCI-state, i.e. using the same type of RS as the candidate beam. Proposal 7: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam when UE evaluate the L1 event measurement. Proposal 1: RAN2 confirms the working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. The RS for current beam of serving cell is same as or associated with QCL RS provided in the indicated TCI-state, i.e. using the same type of RS as the candidate beam. ⇒ Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). We will decide which option to go next meeting.

At the last RAN1 meeting, the blow was agreed.[3]

Agreement@RAN1#118bis The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs - FFS: Details on determination of QCL RS or SSB QCLed with QCL RS Note: This does not imply the support of mTRP scenarios

We analysed pros and cons of six options below.

| Option | Pros | Cons |
| --- | --- | --- |
| 1 | - The NW configures both types of RS (SSB and CSI-RS), offering flexibility in beam selection for UE - UE can choose the most suitable beam type for L1 event measurement | - Increased complexity in network configuration and management - Potential for confusion if multiple beam types are available |
| 2 | - Simplicity in network configuration by restricting to one type of RS - Easier management and lower overhead for the network | - Limits UE's ability to select the best beam type for measurement - May not adapt well to varying network conditions |
| 3 | - Allows for comparisons between different RS types using offsets - Provides flexibility in scenarios where different RS types are present | - Requires careful calibration of offsets, which can be challenging - Complexity in implementation and potential for inaccuracies |
| 4 | - Simplifies the process by relying solely on network configuration | - Reduces flexibility in beam selection and may not optimize performance |
| 5 | - Network determines which RS type to use, simplifying UE's decision-making | - Limits options for UE and may not account for varying conditions and preferences |
| 6 | - Ensures that the RS for the current serving cell is consistent with the candidate beam | - May require additional configuration and oversight to ensure alignment |

Based on the agreement of RAN1 #118bis and the analysis of the options, it is appropriate to select which proposal to support according to operational environment and specific needs.

Option 6 appears to be the most favourable choice.

1. Consistency Across Beams:

- Option 6 ensures that the RS for the current beam of the serving cell is the same as or associated with the QCL RS provided in the indicated TCI state. This consistency is crucial for maintaining signal quality and ensuring seamless communication during LTM.

2. Simplified Network Management:

- By using the same type of RS as the candidate beam, Option 6 simplifies network configuration and management. This can reduce the complexity involved in managing multiple RS types, leading to easier implementation and maintenance of the network.

3. Reduced UE Complexity:

- Option 6 minimizes the decision-making burden on the UE regarding which RS type to use. This simplification can lead to faster and more efficient measurements and evaluations, ultimately enhancing user experience.

Proposal 2: RAN2 should support option 6 (the RS of the serving cell's current beam is the same as or related to the QCL RS provided in the specified TCI state, i.e. uses the same type of RS as the candidate beam).

- Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: RAN2 should support option2. For the LTM event evaluation, TTT is operated per cell, i.e., not per beam.

Proposal 2: RAN2 should support option 6 (the RS of the serving cell's current beam is the same as or related to the QCL RS provided in the specified TCI state, i.e. uses the same type of RS as the candidate beam).

References

[1] RP-241515 Revised Work Item: NR mobility enhancements Phase 4 (Apple Inc, China Telecom)

[2] RAN2 #127bis Report from session on LTE V2X and NR SL (Vice Chairman: Samsung)

[3] RAN1 #118bis Chair notes