3GPP TSG-RAN WG2 Meeting #128	R2-2410716

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.7.6

Source:	Samsung

Title:	Discussion on UL rate control for Rel-19 XR

Document for:	Discussion & Decision

Introduction

In the Rel-19 XR WID [1], it is specified that

Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

Also, in the last RAN2 meeting (127bis), we agreed that:

Agreements on XR rate control - FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc. - RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - Extend the Bit Rate field - Define a new bit rate table to provide sufficient granularity for XR traffic - Introduce new values for the bitRateMultiplier - Send LS to SA4 asking about range/granularity which is required

In this paper, we provide our view on the UL congestion/rate control signalling per QoS flow/DRB.

Discussion

QoS Flow-Level Rate Indication

Following the discussion of the last meeting on whether we need to introduce QoS flow-level rate indication, we can first conclude that there are mainly two options worth considering in realizing the QoS flow-level rate adaptation for UL traffic, i.e.,

- Option 1) gNB transmits a new MAC CE featured with QoS flow-level rate indication to UE, and UE follows gNB instruction.

- Option 2) gNB transmits legacy recommended bit rate MAC CE to UE, indicating the bit rate per LCH/DRB, and let UE itself determine the QoS flow(s) suffering rate adaptation, among the QoS flows belonging to the corresponding LCH/DRB indicated by gNB.

Although we understand Option 2 entails less spec change, we think Option 1 is more desirable. The reason can be explained from the perspectives of responsibility and feasibility.

Responsibility: From the responsibility point of view, it is gNB who is supposed to be responsible for enforcing the QoS requirement for each QoS flow, based on the corresponding QoS profile provided by CN. Also, in the case of congestion, when QoS requirements of all the QoS flows cannot be fulfilled, it is gNB who should be responsible for how to optimize the application performance and network capacity, e.g., by deciding which QoS flows to be prioritized/deprioritized/pre-empted (see Excerption 1). Therefore, the decision on which QoS flows to be throttled when congestion happens should be decided by gNB and indicated to UE.

Observation 1: From the responsibility point of view, it is gNB who is supposed to be responsible for enforcing the QoS requirement for each QoS flow, based on the corresponding QoS profile provided by CN.

Excerption 1:

TS 23.501 V19.1.0 5.7.2.2 ARP The QoS parameter ARP contains information about the priority level, the pre-emption capability and the pre-emption vulnerability. This allows deciding whether a QoS Flow establishment/modification/handover may be accepted or needs to be rejected in the case of resource limitations (typically used for admission control of GBR traffic). It may also be used to decide which existing QoS Flow to pre-empt during resource limitations, i.e., which QoS Flow to release to free up resources. … 5.7.3.3 Priority Level … In the case of congestion, when all QoS requirements cannot be fulfilled for one or more QoS Flows, the Priority Level shall be used to select for which QoS Flows the QoS requirements are prioritised such that a QoS Flow with Priority Level value N is prioritized over QoS Flows with higher Priority Level values (i.e. N+1, N+2, etc). In the case of no congestion, the Priority Level should be used to define the resource distribution between QoS Flows. In addition, the scheduler may prioritize QoS Flows based on other parameters (e.g. resource type, radio condition) in order to optimize application performance and network capacity. …

Feasibility: From the technical point of view, it is feasible for gNB to control the rate per QoS flow. Although how to decide which QoS flow to be throttled should belong to the gNB implementation issue, there is quite a lot relevant information for gNB to take into account for proper decision. For example, ARP priority/pre-emption capability/pre-emption vulnerability, and priority level in 5G QoS characteristics are all reflecting the relative importance among different QoS flows, and hence, gNB with such information can, for example, firstly consider the relatively less important/more vulnerable QoS flow for the rate throttling.  Also, gNB can make use of the GFBR and MFBR of a certain QoS flow in determining whether a QoS flow is capable of throttling or not, as well as the target rate (see Excerption 2).

Observation 2: From the technical point of view, it is feasible for gNB to decide which QoS flow to be rate adapted, and the corresponding rate, by making use of the information in QoS profile, e.g., priority level, ARP, GFBR, and MFBR, per QoS flow.

Excerption 2:

TS 23.501 V19.1.0 5.7.2.5 Flow Bit Rates For GBR QoS Flows only, the following additional QoS parameters exist: - Guaranteed Flow Bit Rate (GFBR) - UL and DL; - Maximum Flow Bit Rate (MFBR) -- UL and DL. The GFBR denotes the bit rate that is guaranteed to be provided by the network to the QoS Flow over the Averaging Time Window. The MFBR limits the bit rate to the highest bit rate that is expected by the QoS Flow (e.g. excess traffic may get discarded or delayed by a rate shaping or policing function at the UE, RAN, UPF). Bit rates above the GFBR value and up to the MFBR value, may be provided with relative priority determined by the Priority Level of the QoS Flows (see clause 5.7.3.3). GFBR and MFBR are signalled to the (R)AN in the QoS Profile and signalled to the UE as QoS Flow level QoS parameter (as specified in TS 24.501 [47]) for each individual QoS Flow. NOTE 1: The GFBR is recommended as the lowest acceptable service bitrate where the service will survive. NOTE 2: For each QoS Flow of Delay-critical GBR resource type, the SMF can ensure that the GFBR of the QoS Flow can be achieved with the MDBV of the QoS Flow using the QoS Flow binding functionality described in clause 6.1.3.2.4 of TS 23.503 [45]. NOTE 3: The network can set MFBR larger than GFBR for a particular QoS Flow based on operator policy and the knowledge of the end point capability, i.e. support of rate adaptation at application / service level.

Thus, the MAC layer XR rate control signalling should be per QoS flow.

Proposal 1: RAN2 to introduce a new MAC CE for rate control signalling per-QoS flow over downlink, for faster uplink source rate adaptation.

Recommended Bit Rate Values Enhancement for XR

Since we have not received the appropriate range and granularity of the recommended bit rate values for XR application from SA4, the relevant discussion can be postponed until receiving the reply LS.

Proposal 2: RAN2 to postpone the discussion on the recommended bit rate values enhancements for XR until receiving the SA4 reply LS.

Conclusion

Based on the discussion above, we have the following observations and proposals:

Observation 1: From the responsibility point of view, it is gNB who is supposed to be responsible for enforcing the QoS requirement for each QoS flow, based on the corresponding QoS profile provided by CN.

Observation 2: From the technical point of view, it is feasible for gNB to decide which QoS flow to be rate adapted, and the corresponding rate, by making use of the information in QoS profile, e.g., priority level, ARP, GFBR, and MFBR, per QoS flow.

Proposal 1: RAN2 to introduce a new MAC CE for rate control signalling per-QoS flow over downlink, for faster uplink source rate adaptation.

Proposal 2: RAN2 to postpone the discussion on the recommended bit rate values enhancements for XR until receiving the SA4 reply LS.

References

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3